AJNETSDK documentation

(AJNetSDK.dll)

V1.6

Tables of contents

Definition of API (AJNETSDK)	7

AJ_NETSDK_IPC_Init (initialize)	7

AJ_NETSDK_IPC_LoadVendorId()	7

AJ_NETSDK_IPC_Cleanup (release resources)	7

AJ_NETSDK_IPC_SetStatusEventCallBack (callback function)	7

AJ_NETSDK_IPC_GetSDKVersion (get AJNETSDK version)	10

AJ_NETSDK_IPC_GetSDKBuildVersion (get AJNETSDK compile version)	10

AJ_NETSDK_IPC_SetAutoReconnect (connect automatically)	10

AJ_NETSDK_IPC_Login (connect device)	11

AJ_NETSDK_IPC_Reconnect (connect again)	11

AJ_NETSDK_IPC_Logout (disconnect device)	12

AJ_NETSDK_IPC_LogoutAll (disconnect all devices)	12

AJ_NETSDK_IPC_SetAUXResponseCallBack (callback function for auxiliary channel)	12

AJ_NETSDK_IPC_SetLogToFile (log output)	13

AJ_NETSDK_IPC_RealPlay (receive stream)	13

AJ_NETSDK_IPC_RealPlayBySimple（Receive unicast media stream interface）	16

AJ_NETSDK_IPC_SetRealDataCallBack (callback function for receiveing stream)	18

AJ_NETSDK_IPC_StopAllRealPlay (stop all media stream)	19

AJ_NETSDK_IPC_StopRealPlay (stop receiving stream)	19

AJ_NETSDK_IPC_GetVideoParam (get video parameter)	19

AJ_NETSDK_IPC_GetAudioParam (get audio parameter)	20

AJ_NETSDK_IPC_GetFileByName (download file)	20

AJ_NETSDK_IPC_StopGetFile (stop downloading)	21

AJ_NETSDK_IPC_GetDownloadPos (get download progress)	21

AJ_NETSDK_IPC_PTZControl (basic PTZ)	21

AJ_NETSDK_IPC_PTZPreset (preset)	22

AJ_NETSDK_IPC_PTZControlEx (PTZ expansion interface, 485 transparent transmission)	23

AJ_NETSDK_IPC_StartVoiceCom (voice talk)	23

AJ_NETSDK_IPC_StopVoiceCom (stop voice talkback)	24

AJ_NETSDK_IPC_FormatDisk (format storage)	24

AJ_NETSDK_IPC_Upgrade (upgrade)	25

AJ_NETSDK_IPC_GetUpgradeProgress (obtain progress of upgrading)	25

AJ_NETSDK_IPC_GetUpgradeState (obtain upgrading status)	26

AJ_NETSDK_IPC_CloseUpgradeHandle (stop upgrading)	26

AJ_NETSDK_IPC_FindDVRLogFile (query log)	26

AJ_NETSDK_IPC_RestoreConfig (restore configuration)	27

AJ_NETSDK_IPC_GetConfigFile (dowload configuration file)	27

AJ_NETSDK_IPC_SetConfigFile (upload and replace configuration file)	27

AJ_NETSDK_IPC_RebootDVR (reboot)	28

AJ_NETSDK_IPC_GetDVRConfig (configuration)	28

AJ_NETSDK_IPC_SetDVRConfig (set configuration)	28

AJ_NETSDK_IPC_SystemControl (system control)	29

AJ_NETSDK_IPC_GetDeviceAbility (obtain device ability)	29

AJ_NETSDK_IPC_GetReplayAblity (front-end playback online)	30

AJ_NETSDK_IPC_ControlPlay (control front-end playback)	30

AJ_NETSDK_IPC_SetReplayDataCallBack (front-end playback callback function)	31

AJ_NETSDK_IPC_PlayDeviceFile (play front-end file)	32

AJ_NETSDK_IPC_SetPlayActionEventCallBack (front-end playback callback)	33

AJ_NETSDK_IPC_SetSearchStatusCallBack (search IPC callback function)	33

AJ_NETSDK_IPC_StartSearchIPC (start to search device)	34

AJ_NETSDK_IPC_StopSearchIPC (stop searching device)	34

AJ_NETSDK_IPC_GetSearchIPCCount (obtain IPC number)	34

AJ_NETSDK_IPC_GetIPCInfo (structure format of IPC information)	34

AJ_NETSDK_IPC_ModifyIPC (modify structure format of IPC information)	36

AJ_NETSDK_IPC_GetIPCInfoXML (obtain device information of XML format)	37

AJ_NETSDK_IPC_ModifyIPCXML (modify IP address XML format)	38

AJ_NETSDK_IPC_GetOneIPAddress (obatain a avaiable IP address)	39

AJ_NETSDK_IPC_GetNetworkParam (obtain network configuration)	39

AJ_NETSDK_IPC_StartTalk (initialize broadcast parameter)	40

AJ_NETSDK_IPC_StopTalk (release broadcast resource)	40

AJ_NETSDK_IPC_InputAudioData Enter the custom PCM intercom audio data.	41

AJ_NETSDK_IPC_AddTalk (add device for broadcast)	41

AJ_NETSDK_IPC_RemoveTalk (stop specific voice talkback)	42

AJ_NETSDK_IPC_CreateIFrame (create key frame)	42

AJ_NETSDK_IPC_SetUserData (save user data)	42

AJ_NETSDK_IPC_GetUserData (read saved data)	43

AJ_NETSDK_IPC_StartRecord(Start Record)	43

AJ_NETSDK_IPC_StopRecord(stop Record)	44

AJ_NETSDK_IPC_GetLastErrorCode(get and eliminate error code)	44

AJ_NETSDK_IPC_StartRecordStream(enable stream mode record)	45

AJ_NETSDK_IPC_InputRecordStream(record file by input media data)	46

AJ_NETSDK_IPC_StopRecordStream (stop outer media recording)	46

AJ_NETSDK_IPC_AddInviteAudioStream(request capture compressed audio)	47

AJ_NETSDK_IPC_SetEncodeAudioCallBack(set capture compressed audio callback)	47

AJ_NETSDK_IPC_RemoveInviteAudioStream(cancle capture compressed audio)	47

AJ_NETSDK_IPC_SerialStart (open serial port)	48

AJ_NETSDK_IPC_SerialSend (send serial data)	48

AJ_NETSDK_IPC_SerialStop (close serial port)	49

Note about AJNETSDK	50

Steps of realizing video stream play	50

Realize auxiliary channel connection for PTZ control, system control, device configuration, etc	51

Steps of realizing audio broadcast	51

Steps of controlling PTZ	52

Steps of receiving stream record	52

Steps of outer input stream record	52

Steps of playing front-end record file	52

Steps of searching online device	53

Receive audio capture compressed stream	53

Schedule 1 PTZ XML command	54

1.	PTZ instruction	54

2.	XML command	54

2.1	Ordinary command	54

2.1.1	UP	54

2.1.2	down	54

2.1.3	left	55

2.1.4	right	55

2.1.5	Left-up	55

2.1.6	Left-down	55

2.1.7	Right-up	56

2.1.8	Right-down	56

2.1.9	stop	56

2.1.10	zoom out	56

2.1.11	zoom in	56

2.1.12	FocusAutoOn	57

2.1.13	FocusAutoOff	57

2.1.14	focus- (focusnear)	57

2.1.15	focus+ (focusfar)	57

2.1.16	IrisAutoOn	57

2.1.17	IrisAutoOff	57

2.1.18	Iris- (irisclose)	58

2.1.19	Iris+ (irisopen)	58

2.2	Orbit and scan	58

2.2.1	Orbit	58

2.2.2	Scan begin	58

2.2.3	Scan end	58

2.2.4	Scan on	58

2.2.5	Scan off	59

2.3	Preset command	59

2.3.1	Get current preset list	59

2.3.2	Add Preset	59

2.3.3	Delete preset	59

2.3.4	Call preset	60

2.3.5	Set for orbit	60

2.3.6	Set for scan	60

2.3.7	Delete orbit setting	61

2.3.8	Delete scan setting	61

2.4	Advanced command	61

2.4.1	Black/White	61

2.4.2	Color	61

2.4.3	Mirror off	61

2.4.4	Mirror on	62

2.4.5	Freeze off	62

2.4.6	Freeze on	62

2.4.7	Digital zoom off	62

2.4.8	Digital zoom on	62

2.4.9	OSD off	62

2.4.10	OSD on	63

2.4.11	Illumination off	63

2.4.12	Illumination on	63

2.4.13	Ill off	63

2.4.14	Ill on	63

2.4.15	Reset	63

2.4.16	WB auto	64

2.4.17	WB manual	64

2.4.18	WB_R	64

2.4.19	WB_B	64

2.4.20	WB_M	64

2.4.21	WB_G	64

2.4.22	Initialize	65

2.4.23 Menu	65

2.5 Transparent Transmission Control Command	65

Schedule2 Advanced function of auxiliary channel	66

1. Advanced function	66

2. Call auxiliary channel function	66

3. Read and set command	66

Obtain PTZ command (201 command)	66

Modify PTZ parameter (225 command)	70

Modify PTZ advanced parameter (226 command)	71

Read configuration about synchronize time (202 command)	74

Modify method about synchronize time (222 command)	75

Obtain user list (203 command)	75

Modify user list (223 command)	76

Output log (204 command)	76

Set Output log (224 command)	76

Read default language (205 command)	77

Set default language (225 command)	77

Read system configuration XML (command 200)	77

Modify system configuration (command 220)	81

4. Read and set network configuration information	85

Read network configuration (command 200)	85

Modify network configuration (command 220)	87

Read Ethernet configuration (command 305)	89

Modify Ethernet configuration (command 325)	89

Read the WIFI configuration (command 306)	90

Modify the WIFI configuration (command 326)	91

Read the ADSL configuration (command 310)	92

Modify the ADSL configuration (command 330)	92

Read the DDNS configuration (command 311)	92

Modify the DDNS configuration (command 331)	93

Read the UPNP configuration (command 312)	93

Modify UPNP configuration (command 332)	93

Read the 3G configuration (command 313)	94

Modify the 3G configuration (command 333)	94

5. FTP & SMTP related configuration	95

Read FTP & SMTP related configuration (command 400)	95

Modify FTP & SMTP related configuration (command 420)	96

Read the FTP configuration (command 401)	98

Modify the FTP configuration (command 421)	99

Read the SMTP configuration (command 402)	100

Modify the SMTP configuration (command 422)	100

6. Media parameter setup	101

Read media configuration parameter (command 500)	101

Read video parameter configuration (command 501)	104

Read audio parameter configuration (command 502)	106

Modify media parameter configuration (command 520)	106

Modify video parameter configuration (command 523)	109

Modify video parameter configuration (command 524)	110

Modify time title parameter configuration (command 525)	110

Modify picture capture configuration (command 526)	111

Modify audio capture configuration (command 527)	111

Modify audio encode configuration (command 528)	112

Modify privacy mask (command 529)	112

Read streaming media parameter (command 600)	113

Modify streaming media parameter (command 620)	113

Read storage setup parameter (command 700)	114

Modify storage setup parameter (command 720)	115

7. Alarm setup command	117

Read alarm setting parameter (command 800)	117

Modify alarm setting parameter (command 820)	120

Read IO input alarm parameter (command 801)	123

Modify IO input alarm parameter (command 821)	124

Read motion detect alarm parameter (command 802)	125

Modify motion detect alarm parameter (command 822)	126

Read video lost alarm parameter (command 803)	128

Modify video lost alarm parameter (command 823)	128

Read video lost alarm parameter (command 804)	129

Modify video lost alarm parameter (command 824)	129

Read storage alarm patameter (command 805)	130

Modify storage alarm patameter (command 825)	130

8. System control command	131

Read system information (command 1012)	131

Restore to default configuration (command 1002)	131

Modify system time (command 1005)	132

Restart system (command 1007)	132

Receive storage information (command 1014)	132

Format front-end storage device (command 1017)	133

Unmount front-end storage device (command 1018)	133

Receive device serial number information (command 1019)	133

Read system capacity set (command 1020)	134

Device video file search (picture or video file) (command 1021)	134

File upload (configuration file and update file) (command 1022)	135

File download (command 1023)	136

Delete file (command 1024)	136

Search log file (command 1025)	137

Read device type (command 1030)	137

Enforce device generate key frame (only V2.0 firmware has) (command 1032)	137

Read user storage data (only V2.0 firmware has) (command 1034)	138

Set user storage data (only V2.0 firmware has) (command 1033)	138

9. Related micro definition table	138

# Definition of API (AJNETSDK)

## AJ_NETSDK_IPC_Init (initialize)

Function:

LONG	__stdcall 	AJ_NETSDK_IPC_Init();

:

Parameterize DLL; call the function before call all functions

Parameter:

0: success

ERR_INIT_SOCKET_ERROR: failed to initialize socket

Note:

The “API” can not be called again after call AJ_NETSDK_IPC_Cleanup

## AJ_NETSDK_IPC_LoadVendorId()

Function:

LONG	__stdcall		AJ_NETSDK_IPC_LoadVendorId();

Feature:

Read vendor id, call the function before call all functions

Parameter:

0: success

## AJ_NETSDK_IPC_Cleanup (release resources)

Function:

LONG	__stdcall 	AJ_NETSDK_IPC_Cleanup();

Feature:

Clean up the used recourses in DLL

Return:

0: success

Note:

Please call the function for exiting all threads when exit application

## AJ_NETSDK_IPC_SetStatusEventCallBack (callback function)

Function:

LONG	__stdcall	 AJ_NETSDK_IPC_SetStatusEventCallBack(StatusEventCallBack fStatusEventCallBack,void * pUser);

Feature:

Call the function when device status has been changed or receive event or alarm message

Feature:

fStatusEventCallBack: callback function

Prototype: typedef LONG(CALLBACK *StatusEventCallBack)(LONG lUser,LONG nStateCode,char *pResponse,void *pUser);

Meaning of parameters:

lUser: device mark

nStateCode: event type

enum enumNetSatateEvent

{

EVENT_CONNECTING,//connecting

EVENT_CONNECTOK,//connect successfully

EVENT_CONNECTFAILED,//connect failed

EVENT_SOCKETERROR,//sock failed

EVENT_LOGINOK,// login sucessfully

EVENT_LOGINFAILED,// login failed

EVENT_STARTAUDIOOK,// voice talk sucess

EVENT_STARTAUDIOFAILED,// voice talk failed

EVENT_STOPAUDIOOK,// stop voice talking sucessfully

EVENT_STOPAUDIOFAILED,// stop voice talking failed

EVENT_SENDPTZOK,// send PTZ command successfully

EVENT_SENDPTZFAILED,// send PTZ command failed

EVENT_SENDAUXOK,// send auxiliary channel sucessfully

EVENT_SENDAUXFAILED,// send auxiliary channel failed

EVENT_UPLOADOK,// upload file successfully

EVENT_UPLOADFAILED,// upload file failed

EVENT_DOWNLOADOK,// download successfully

EVENT_DOWNLOADFAILED,// download failed

EVENT_REMOVEOK,// delete file successfully

EVENT_REMOVEFAILED,// delete file failed

EVENT_SENDPTZERROR,// failed to control PTZ

EVENT_PTZPRESETINFO,// presets

EVNET_PTZNOPRESETINFO,// no preset

EVENT_PTZALARM,// alarm message

EVENT_RECVVIDEOPARAM,// return video parameter

EVENT_RECVAUDIOPARAM,// return audio parameter

EVENT_CONNECTRTSPERROR,// request video failed

EVENT_CONNECTRTSPOK,// obtain media stream successfully

EVENT_RTSPTHREADEXIT,// exit media stream thread

EVENT_URLERROR,// failed to connect media stream URL

EVENT_RECVVIDEOAUDIOPARAM,// obtain audio and video parameter

EVENT_LOGIN_USERERROR,// username or password error

EVENT_LOGOUT_FINISH,	// the login thread has already exit main loop

EVENT_LOGIN_RECONNECT,	// login again

EVENT_LOGIN_HEARTBEAT_LOST,// hearheat is lost

EVENT_STARTAUDIO_ISBUSY,// voice talk is occupied

EVENT_STARTAUDIO_PARAMERROR,// voice talk parameter error

EVENT_STARTAUDIO_AUDIODDISABLED,// voice talk of the device is unavailable

EVENT_CONNECT_RTSPSERVER_ERROR=38,	// connect IP or port error

EVENT_CREATE_RTSPCLIENT_ERROR,		// create rtsp client error

EVENT_GET_RTSP_CMDOPTION_ERROR,		// return error when connecting rtsp server

EVENT_RTSP_AUTHERROR,				// username or password failed

};

While nStateCode= EVENT_PTZALARM, then the meaning of code in pResponse as following:

pResponse is structure: ALARM_ITEM

typedef struct

{

ALARM_TIME time;//alarm happen time(device time)

int code;//alarm type

int flag;

int level;

char data[128];//description

}ALARM_ITEM;

ALARM_TIME structure :

typedef struct

{

int year;//year

int month;//month

int day;//day

int wday;//week

int hour;//hour

int minute;//minute

int second;//second

}ALARM_TIME;

1 : device is offline

2 : device is online

3 : USB ready

4 : USB not ready

5 : SD1 ready

6 : SD1 not ready

7 : SD2 ready

8 : SD2 not ready

9 : the free space of USB is lacking

10: the free space of SD1 is lacking

11: the free space of SD2 is lacking

12: video lost

13: video block

14: motion detect

15: IO high to low

16: IO low to high

17: free space lacking

18: start record success

19: record finish

20: start record failed

While nStateCode= EVENT_RECVVIDEOPARAM, pResponse is structure:

typedef struct

{

CHAR codec[256]; //H264 or MPEG4, means current video decoding type

INT width; //video width

INT height; //video height

INT colorbits; // colorbits, invalid here, only use in internal

INT framerate; //reference framerate

INT bitrate; //reference bitrate

BYTE vol_data[256]; //extension paratemer,necessary data when decoding

INT vol_length; //extension parameter length

}VIDEO_PARAM;

While nStateCode= EVENT_RECVAUDIOPARAM, pResponse is structure:

typedef struct

{

CHAR codec[256]; //audio type, AAC or G711

INT samplerate; // reference samplerate

INT bitspersample; // reference bitrate

INT channels; //channel number

INT framerate;

INT bitrate;

}AUDIO_PARAM;

While nStateCode= EVENT_RECVVIDEOAUDIOPARAM, pResponse is structure:

typedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	// ==AV_FALG

short  bHaveVideo;//0 means no video parameter

short  bHaveAudio;//0 means no audio parameter

VIDEO_PARAM videoParam;//video parameter

AUDIO_PARAM audioParam;//audio parameter

}STREAM_AV_PARAM;

Some message about pResponse:

pUser: the parameter is void *pUser when call the callback function

Return:

0: success

Note:

The callback function is not tread-safe, be careful the thread-safe (the callback function can be called by many threads)

As a general rule, PostMessage to main thread and let main thread or special thread to process this message after receive callback status, otherwise, it maybe deadlock when process callback status on callback login

## AJ_NETSDK_IPC_GetSDKVersion (get AJNETSDK version)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetSDKVersion();

Feature:

Get AJNETSDK version

Return:

It will return a 32-bits integer such as 1110050101, 111005 means release date, 0101 means version 1.01, for example, 1110050101 means it is 1.01 version release in October 5, 2011

## AJ_NETSDK_IPC_GetSDKBuildVersion (get AJNETSDK compile version)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetSDKBuildVersion();

Feature:

Return AJNETSDK complie date

Return:

Return compile date such as 2012011411 means January 14, 2012

## AJ_NETSDK_IPC_SetAutoReconnect (connect automatically)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_SetAutoReconnect(LONG lUserID,int bAutoReconnect);

Feature:

Whether connect automatically while login error or video stream error. Note: Library defaults connect automatically.

Parameter:

lUserID: when it is 0 means it is new connection and will use the pre-set bAutoReconnect

bAutoReconnect: 0 means it can not connect automatically, otherwise connect automatically.

Return:

0: success

ERR_NOT_FIND_DEVICE: can not find the corresponding device of lUserID

## AJ_NETSDK_IPC_Login (connect device)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_Login(char * szDVRIP,WORD wDVRPort,char *sUserName , char  *sPassword , LPAJ_NETSDK_IPC_DEVICEINFO lpDeviceInfo);

Feature:

The unique mark for communication channel, used for PTZ and configuration

Parameter:

szDVRIP: the IP address for connecting (such as 192.168.0.10)

wDVRPort: communication port (such as 8091)

sUserName: user name

sPassword: password

lpDeviceInfo: pass in null, for expansion and compatibility

Return:

Return 0 means failed (all the parameter named lUserID use the value)

Reasons for failure:

- szDVRIP is empty

- wDVRPort is not between 1 and 65535(invalid port)

- the IP and port has already logged

Note:

1. The same devie can login many times

2. It will start a thread for connection and login after call login and it will be re-connection, if want to set up whether automatically connect, you can call AJ_NETSDK_IPC_SetAutoReconnectv (0, 0) first.

3. The login status will be notified by (AJ_NETSDK_IPC_SetStatusEventCallBack).

Some possible status

nStateCode: //event type

EVENT_CONNECTING,// connecting

EVENT_CONNECTOK,// connect successfully, but not be certified

EVENT_CONNECTFAILED,// connect failed

EVENT_SOCKETERROR,//sock failed

EVENT_LOGINOK,// login successfully, certified successfully

EVENT_LOGINFAILED=5,// login failed

EVENT_LOGIN_USERERROR=31,// username or password error

EVENT_LOGOUT_FINISH,	// exit main loop

EVENT_LOGIN_RECONNECT,	// login again

EVENT_LOGIN_HEARTBEAT_LOST,// heartbeat is lost

## AJ_NETSDK_IPC_Reconnect (connect again)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_Reconnect(LONG lUserID);

Feature:

Reconnect specified device or media stream

Parameter:

lUserID: handle for login device

Return:

0: re-connect successfully

ERR_NOT_FIND_DEVICE: handle error

ERR_START_THREADERROR: start thread failed

ERR_DEV_NOT_LOGIN: not call login or video streaming handle

Note:

- It does not take effect until the logged device exit connection. And it maybe deadlock when process callback status on callback login.

- It will re-connect automatically in default.

## AJ_NETSDK_IPC_Logout (disconnect device)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_Logout(LONG 	lUserID);

Feature:

Cancellation of login devices

Parameter:

lUserID: a return value of AJ_NETSDK_IPC_Login

Return

0: logout successfully

ERR_NOT_FIND_DEVICE: handle error

Note:

1. call AJ_NETSDK_IPC_Logout after call AJ_NETSDK_IPC_Login

2. It will release relevant resources after call the function and wait for all thread logout.

## AJ_NETSDK_IPC_LogoutAll (disconnect all devices)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_LogoutAll();

Feature:

Logout all login devices

Parameter:

none

Return:

0: sucess

Note:

1. Call the function for logout all logged devices(keep media connection).

## AJ_NETSDK_IPC_SetAUXResponseCallBack (callback function for auxiliary channel)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_SetAUXResponseCallBack(AUXResponseCallBack fAUXCallBack,void * pUser);

Feature:

Set callback function for an impact auxiliary channel

Parameter:

fAUXCallBack: it will call the function when receive message from auxiliary channel.

Prototype: typedef LONG(CALLBACK *AUXResponseCallBack)(LONG lUser,LONG nType,char *pResponse,void *pUser);

Meaning:

lUser: identify the logged devices

nType: message type, the most significant byte of nType is not 0 (nType & 0xff000000!=0) means the corresponding message was carried out failed, the low 24 bit(Type=nType & 0xffffff) is the real message type, please refer to schedule 2 for detail meaning.

pUser: user pointer, is the same with the parameter of callback.

pUser: for call calback function.

Return:

0: success

Note:

- The callback function is not tread-safe, be careful the thread-safe (the callback function can be called by many threads)

- It maybe deadlock when process callback status directly

## AJ_NETSDK_IPC_SetLogToFile (log output)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_SetLogToFile(DWORD bLogEnable,char *strLogDir,BOOL bAutoDel);

Feature:

Log output

Parameter:

bLogEnable: 0 means it will not ouput log; 1 means it will output local log

strLogDir: it is a log directory when log output enable

bAutoDel: non-zero, it will delete log automatically when the logs are more than 10 (it does not work now)

Return:

0: success

## AJ_NETSDK_IPC_RealPlay (receive stream)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_RealPlay(LONG lUserID,LPAJ_NETSDK_IPC_CLIENTINFO lpClientInfo,fRealDataCallBack cbRealDataCallBack,LPUSRE_VIDEOINFO pUser,BOOL bBlocked);

Feature:

Start for receive stream

Parameter:

lUserID: a logged device IP address. Used for get IP address

lpClientInfo: compatible with orther SDK of different company (pass NULL)

cbRealDataCallBack:(a callback function for receive stream)

prototype: typedef LONG(CALLBACK *fRealDataCallBack)(LONG lRealHandle,DWORD dwDataType,BYTE *pBuffer,DWORD dwBufSize,LPFRAME_EXTDATA  pExtData);

meaning:

lRealHandle used for mark up video stream, a return value of this function

dwDataType=0 means video, 1 means audio,2 means decoding parameter

pBuffer: media data pointer

when dwDataType is 2, pBuffer structure and its significance for the body :

typedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	//==AV_FALG

short  bHaveVideo;//0 means no video parameter

short  bHaveAudio;//0 means no audio parameter

VIDEO_PARAM videoParam;//video parameter

AUDIO_PARAM audioParam;//audio parameter

}STREAM_AV_PARAM;

inside:

typedef struct

{

CHAR codec[256];//H264 or MPEG4,etc, indicate current video encoding type

INT width;      //video width

INT height;	  //video height

INT colorbits;  //color bit, invalid here,only use for internal

INT framerate;  //referance framerate

INT bitrate;    //referance code rate

BYTE vol_data[256];//extension parameter,necessary data for decoding

INT vol_length;    //extension parameter length

}VIDEO_PARAM;

typedef struct

{

CHAR codec[256];	//audio type, AAC or G711

INT samplerate;	//referance bitrate

INT bitspersample; //referance bitrate

INT channels;		//sound channel quantity

INT framerate;

INT bitrate;

}AUDIO_PARAM;

dwBufSize: media data length

pExtData //expansion parameter, when dwDataType=2, this pointer is null, when dwDataType=0,1,struct body as follow:

typedef struct

{

int		bIsKey;//wheter it is a key frame

double	timestamp;// timestamp

void    *pUserData;// user data pointer

}FRAME_EXTDATA,* LPFRAME_EXTDATA;

pUser: // connection parameter,struct body as follow:

typedef struct

{

int  nVideoPort;// video port

int  bIsTcp;// whether enable TCP

int  nVideoChannle; // video channel, 32-bits integer, the previous 16-bit is type, it can be 0 or1, 0 is IPC, 1 is DVS; the subsequent 16-bit is video channel number, when the type is 0, it can be 0 or 1,0 means main stream, 1 means sub stream, when the type is 1, it can be 0,1,2,3,4,5 represent channel 1,2,3,4 and CIF for output

void *pUserData;//user pointer

}USRE_VIDEOINFO,* LPUSRE_VIDEOINFO;

bBlocked: a save parameter for compatibility

Return:

0 means fail, otherwise return unqiue indentifier

Note:

1. It can not take a long of time for calling callback function

2. cbRealDataCallBack is not thread-safe

3. In order to prevent mosaics, then you need to change the subsequent 16-bit, such as flag=0x1a2b3c4d which will be removed

typedef struct

{

unsigned long flag;			//flag, fixed as 0x1a2b3c4d

unsigned long data;			//last byte, data & 0xff, if the value is not 0 then it means it is the atual frame

unsigned long frame_index;		//the current frame ID

unsigned long keyframe_index;	    //the key frame ID which the current frame depend on

}VIDEO_FRAME_HEADER;

In order to prevent mosaics, you can drop frame as following:

Char * inbuf_ptr;// the memory pointer of the current frame

int size;// the current frame length

VIDEO_FRAME_HEADER *pHeader = (VIDEO_FRAME_HEADER *)(inbuf_ptr + size - sizeof(VIDEO_FRAME_HEADER));

if(pHeader->flag == 0x1a2b3c4d)

{

size = size - sizeof(VIDEO_FRAME_HEADER);

if(pHeader->frame_index == pHeader->keyframe_index)  //key frame

{

m_dwLastFrameIndex = m_dwLastKeyFrameIndex = pHeader->keyframe_index;

}

else

{

if(pHeader->frame_index !=m_dwLastFrameIndex + 1)

{//skip, does not decode the frame

return -1;

}

else if(pHeader->keyframe_index != m_dwLastKeyFrameIndex)

{//skip, does not decode the frame

return -1;

}

else

{// record the last decode frame number

m_dwLastFrameIndex++;

}

}

}

4. It will receive the following status from status callback function

EVENT_RECVVIDEOPARAM=24,// receive video encode parameter

EVENT_RECVAUDIOPARAM,// receive audio encode parameter

EVENT_CONNECTRTSPERROR,// connect failed

EVENT_CONNECTRTSPOK,// connect successfully

EVENT_RTSPTHREADEXIT,// exit connection thread

EVENT_URLERROR,// URL error

EVENT_RECVVIDEOAUDIOPARAM,// receive audio and video parameter at the same time

EVENT_CONNECT_RTSPSERVER_ERROR=38,	// connect failed, because of wrong IP or port

EVENT_CREATE_RTSPCLIENT_ERROR,		// create a wrong rtsp client

EVENT_GET_RTSP_CMDOPTION_ERROR,		// connect rtsp, return error

EVENT_RTSP_AUTHERROR,			// authentication failure, because of wrong username or password

When nStateCode= EVENT_RECVVIDEOPARAM then pResponse is structure:

typedef struct

{

CHAR codec[256]; //H264 or MPEG4, etc, means current vido encoding type

INT width; //video width

INT height; //video height

INT colorbits; //color bit, invalid here, only use for internal

INT framerate; //referance frame rate

INT bitrate; //referance bit rate

BYTE vol_data[256]; //extension parameter,necessary data for decoding

INT vol_length; //extension parameter length

}VIDEO_PARAM;

When nStateCode= EVENT_RECVAUDIOPARAM, pResponse is structure

typedef struct

{

CHAR codec[256]; //audio type,AAC or G711

INT samplerate; //referance bit rate

INT bitspersample; //referance bit rate

INT channels; //audio channel quantity

INT framerate;

INT bitrate;

}AUDIO_PARAM;

When nStateCode= EVENT_RECVVIDEOAUDIOPARAM, pResponse is structure

typedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	//==AV_FALG

short  bHaveVideo;//0 means no video parameter

short  bHaveAudio;//0 means no audio parameter

VIDEO_PARAM videoParam;// video parameter

AUDIO_PARAM audioParam;// audio parameter

}STREAM_AV_PARAM;

## AJ_NETSDK_IPC_RealPlayBySimple（Receive unicast media stream interface）

Function LONG    __stdcall  AJ_NETSDK_IPC_RealPlayBySimple(LONG lUserID, char * pserverip, char *user, char *pass, fRealDataCallBack cbRealDataCallBack, int iStreamType, int  nVideoPort, int  bIsTcp, void *pUserData, BOOL bBlocked);

Feature：

Initiate a received audio and video stream for receiving only a video stream or an audio stream.

Parameter：

lUserID：The value returned when logging in to the device, used to determine whether the current device is successfully connected.

serverip：Device IP address, when directly connected to the device, compared with IP address,

user：The device is logged on to the user

pass：Device login password

cbRealDataCallBack：Callback function when a stream is received

prototype：typedef LONG(CALLBACK *fRealDataCallBack)(LONG lRealHandle,DWORD dwDataType,BYTE *pBuffer,DWORD dwBufSize,LPFRAME_EXTDATA  pExtData);

The meaning of the parameters is:

lRealHandle：Which is used to mark the video stream, which is the return value of this function,

dwDataType：0 for video, 1 for audio, 2 for decoding parameters

pBuffer：Media data pointer

When dwDataType = 2, pBuffer structure and its meaningtypedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	//==AV_FALG

short  bHaveVideo;//0 Indicates that there are no video parameters

short  bHaveAudio;//0 Indicates that there are no audio parameters

VIDEO_PARAM videoParam;// Video parameters

AUDIO_PARAM audioParam;// Audio parameters

}STREAM_AV_PARAM;

them:

typedef struct

{

CHAR codec[256];// H264 or H265, and so on, that the current video encoding type

INT width;      // Video width

INT height;	  // Video is high

INT colorbits;  // The number of color bits is invalid here, only for internal use

INT framerate;  // Reference frame rate

INT bitrate;    // Reference bitrate

BYTE vol_data[256];// Extended parameters, the necessary data decoding

INT vol_length;    // Extended parameter length

}VIDEO_PARAM;

typedef struct

{

CHAR codec[256];	// Audio type, AAC or G711

INT samplerate;	// Reference sample rate

INT bitspersample; // Reference bit rate

INT channels;		// Number of channels

INT framerate;

INT bitrate;

}AUDIO_PARAM;

dwBufSize：Media Data Length

pExtData：Extended parameters，When dwDataType = 0,1, the structure and its meaning is,

typedef struct

{

int		bIsKey;// Whether the key frame

double	timestamp;// timestamp

void    *pUserData;// User data pointer, this is the function pUser set pointer

}FRAME_EXTDATA,* LPFRAME_EXTDATA;

When dwDataType = 2, this pointer is empty

pUser：Connection-related parameters, the structure is defined as:

typedef struct

{

int  nVideoPort;// Connect the video port

int  bIsTcp;// Whether to use a TCP connection

int  nVideoChannle;// The video channel is a 32-bit integer, and a high 16 indicates the device type: 0 for IPC, 1 for DVS, and 2 for NVR.

// IPC：The lower 16 bits represent. 0 indicates the main stream, and 1 indicates the sub-stream

//DVS：The lower 16 bits represent DVS device channels, which can be 0, 1, 2, 3, 4, representing channel 1, 2, 3, 4, and CIF outputs, respectively.

//NVR ：The upper 8 bits of the lower 16 bits indicate the stream type: 0 indicates the main stream, 1 indicates the sub-stream, and the lower 8 bits of the lower 16 bits indicate the channel number of the device, which can be 0-32.

void *pUserData;// The user pointer, used as the last parameter for the callback

}USRE_VIDEOINFO,* LPUSRE_VIDEOINFO;

iStreamType：Request stream type, 0: Primary stream, 1: Sub-stream.

int  nVideoPort;// Connect the video port

int  bIsTcp;// Whether to use a TCP connection

void *pUserData;// The user pointer, used as the last parameter for the callback

bBlocked：Leave the parameters for compatibility

Return：

0 means fail, otherwise return unqiue indentifier

## AJ_NETSDK_IPC_SetRealDataCallBack (callback function for receiveing stream)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_SetRealDataCallBack(fRealDataCallBack cbRealDataCallBack, void *pUser);

Feature:

Set callback function for receiving media stream,take effect when callback function pass in Null.

Parameter:

cbRealDataCallBack: //callback function for receiving stream

Prototype: typedef LONG(CALLBACK *fRealDataCallBack)(LONG lRealHandle,DWORD dwDataType,BYTE *pBuffer,DWORD dwBufSize,LPFRAME_EXTDATA  pExtData);

Meaning:

lRealHandle // mark up video, the return value of this function

dwDataType=0 means video, 1 means audio, 2 means decoding parameter

pBuffer //media data pointer

when dwDataType= 2, pBuffer structure and its significance for the body

typedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	//==AV_FALG

short  bHaveVideo;//0 means no video parameter

short  bHaveAudio;//0 means no audio parameter

VIDEO_PARAM videoParam;//video parameter

AUDIO_PARAM audioParam;//audio parameter

}STREAM_AV_PARAM;

inside:

typedef struct

{

CHAR codec[256];//H264 or MPEG4, etc, means current vido encoding type

INT width;      //video width

INT height;	  //video height

INT colorbits;  //color bit, invalid here, only use for internal

INT framerate;  //referance frame rate

INT bitrate;    //referance bit rate

BYTE vol_data[256];//extension parameter,necessary data for decoding

INT vol_length;    //extension parameter length

}VIDEO_PARAM;

typedef struct

{

CHAR codec[256];	//audio type,AAC or G711

INT samplerate;	//referance bit rate

INT bitspersample; //referance bit rate

INT channels;		//audio channel quantity

INT framerate;

INT bitrate;

}AUDIO_PARAM;

dwBufSize //media data length

pExtData //extended parameter, when dwDataType=2, this pointer is null, when dwDataType=0,1,struct body as follow:

typedef struct

{

int		bIsKey;// whether it is key frame

double	timestamp;// timestamp

void    *pUserData;//user data pointer

}FRAME_EXTDATA,* LPFRAME_EXTDATA;

pUser: only for user, used as parameter when received media data

Return:

0 means success

Note:

The function set up callback will take effect when incoming callback function of request flow return NULL,regard as all the channels' default callback function. If each incoming callback function of request flow return non-empty,this function can be not called.

## AJ_NETSDK_IPC_StopAllRealPlay (stop all media stream)

Function

LONG		__stdcall		AJ_NETSDK_IPC_StopAllRealPlay();

Feature

Stop all stream

Parameter:

none

Return:

0 means success

Note:

Call the function for stopping all media stream

## AJ_NETSDK_IPC_StopRealPlay (stop receiving stream)

Function

LONG		__stdcall		AJ_NETSDK_IPC_StopRealPlay(LONG lRealHandle);

Feature:

Stop receiving stream

Parameter:

lRealHandle: return non-zero after call AJ_NETSDK_IPC_RealPlayEx or AJ_NETSDK_IPC_RealPlay

Return:

0 means success

ERR_NOT_FIND_STREAM: handle error

Note:

It does not return until the thread of receiving stream end, it will take a lot of time less than 100 millisecond.

## AJ_NETSDK_IPC_GetVideoParam (get video parameter)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetVideoParam(LONG  lRealHandle,VIDEO_PARAM *pParam);

Feature:

Get video parameter structure of video stream

Parameter:

lRealHandle: is non-zero return from AJ_NETSDK_IPC_RealPlayEx or AJ_NETSDK_IPC_RealPlay

pParam: video parameter output

Return:

0 means success

ERR_NOT_FIND_STREAM: can not find the stream handle

Note:

It only take effect when login successfully and EVENT_RECVAUDIOPARAM is called.

VIDEO_PARAM

typedef struct

{

CHAR codec[256]; //H264 or MPEG4, etc, means current vido encoding type

INT width; //video width

INT height; //video height

INT colorbits; //color bit, invalid here, only use for internal

INT framerate; //referance frame rate

INT bitrate; //referance bit rate

BYTE vol_data[256]; //extension parameter,necessary data for decoding

INT vol_length; //extension parameter length

}VIDEO_PARAM;

## AJ_NETSDK_IPC_GetAudioParam (get audio parameter)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetAudioParam(LONG lRealHandle,AUDIO_PARAM *pParam);

Feature:

Get audio parameter of video stream

Parameter:

lRealHandle: is non-zero return from AJ_NETSDK_IPC_RealPlayEx or AJ_NETSDK_IPC_RealPlay

pParam: audio parameter output

Return:

0 means sucess

ERR_NOT_FIND_STREAM: can not find stream handle

Note:

It only take effect when login successfully and EVENT_RECVVIDEOPARAM is called

AUDIO_PARAM

typedef struct

{

CHAR codec[256]; //audio type,AAC or G711

INT samplerate; //referance bit rate

INT bitspersample; //referance bit rate

INT channels; //audio channel quantity

INT framerate;

INT bitrate;

}AUDIO_PARAM;

## AJ_NETSDK_IPC_GetFileByName (download file)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetFileByName(LONG lUserID,LONG nFileType,char *sDVRFileName,char *sSavedFileName);

Feature:

Download specified file

Parameter:

lUserID: the return value for login device

nFileType: downloaded file type, 0 means log file, 1 means record file

sDVRFileName: downloaded file name

sSavedFileName: file name saved in local

Return:

0: means it has already submit download task, and call AJ_NETSDK_IPC_GetDownloadPos for query, status callbcak function will return message for downloading success or not

ERR_NOT_FIND_DEVICE: handle error

ERR_PARAM_ERROR: means sDVRFileName or sSavedFileName is null

ERR_DEV_NOT_CONNECTED: device is disconnect

ERR_DEV_NOT_LOGIN: device login failed

ERR_ISUPLOADING_ERROR: uploading

ERR_ISDOWNLOADING_ERROR: downloading

ERR_IS_STARTAUDIO_ERROR: can not download while voice talk

Note:

1. only can upload or download a file of the same device

2. return 0, then it will receive status from callback function as following:

EVENT_DOWNLOADOK=16,	//download compeletely

EVENT_DOWNLOADFAILED,	//download failed

## AJ_NETSDK_IPC_StopGetFile (stop downloading)

Function

LONG		__stdcall		AJ_NETSDK_IPC_StopGetFile(LONG lUserID);

Feature:

Stop downloading file

Parameter:

lUserID: a return value

Return:

0: means success

ERR_NOT_FIND_DEVICE: handl error

ERR_ISFINISH_ERROR: means download competely or does not download

## AJ_NETSDK_IPC_GetDownloadPos (get download progress)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetDownloadPos(LONG lUserID);

Feature:

Get download progress

Parameter

lUserID: a return value

Return:

101: download compeletely

ERR_NOT_DOWNLOAD_MODE_ERROR: does not download

Greater than or equal 0: the current download progress

## AJ_NETSDK_IPC_PTZControl (basic PTZ)

Function

LONG		__stdcall		AJ_NETSDK_IPC_PTZControl(LONG lUser,DWORD dwPTZCommand,DWORD nTspeed,DWORD nPpeed);

Feature:

Basic PTZ control

parameter

lUserID: a return value

dwPTZCommand: //PTZ type

enum PTZ_CMD_TYPE

{

LIGHT_PWRON=2,// 2 power on for light

WIPER_PWRON,// 3 siwtch on for wipers

FAN_PWRON,// 4 siwtch on for fan

HEATER_PWRON,// 5 siwtch on for heater

AUX_PWRON1,// 6 siwtch on for auxiliary device

AUX_PWRON2,// 7 siwtch on for auxiliary device

ZOOM_IN= 11,// zoom in

ZOOM_OUT, //12 zoom out

FOCUS_NEAR, //13 focus -

FOCUS_FAR, //14 focus +

IRIS_OPEN, //15 iris +

IRIS_CLOSE,// 16 iris -

TILT_UP,// 21 up

TILT_DOWN, //22  down

PAN_LEFT,// 23 left

PAN_RIGHT,// 24 right

UP_LEFT,// 25 up and left

UP_RIGHT,// 26 up and right

DOWN_LEFT, //27 down and left

DOWN_RIGHT,// 28 down and right

PAN_AUTO,// 29 scan

STOPACTION// 30 stop

};

nTspeed: horizontal speed,value is from 1 to 10

nPpeed: vertical speed,value is from 1 to 10

Return:

0 means success

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_PTZCMD_ACTION_ERROR: PTZ command error

ERR_OUTOFF_MEMORY: out of memory

Note:

0 means the command has already added in sending queue, then EVENT_SENDPTZOK and EVENT_SENDPTZFAILED will return message for send successfully or not.

## AJ_NETSDK_IPC_PTZPreset (preset)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_PTZPreset(LONG lUser,DWORD dwPTZPresetCmd,DWORD dwPresetIndex);

Feature:

Basic operation of presets

Parameter

lUserID: a return value

dwPTZPresetCmd: operation type

enum PTZ_PRESET_TYPE

{

SET_PRESET= 8 ,//set

CLE_PRESET= 9,//delete

GOTO_PRESET= 39//query

};

dwPresetIndex:preset number from 0 to 255

Return:

0 means success

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

ERR_PTZCMD_ACTION_ERROR: dwPTZPresetCmd error

ERR_PARAM_ERROR: dwPresetIndex error

Note:

0 means the command has already added in sending queue, then EVENT_SENDPTZOK and EVENT_SENDPTZFAILED will return message for send successfully or not.

## AJ_NETSDK_IPC_PTZControlEx (PTZ expansion interface, 485 transparent transmission)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_PTZControlEx(LONG lUser,char *pXml);

Feature:

Parameter:

lUserID: a return value

pXml: a XML format for PTZ, please refer to schedule 1 for more information, if it is 485 transparent transmission,turn the byte to hexadecimal string to call

Return:

0: success

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

ERR_PARAM_ERROR: pXml is null

Note:

0 means the command has already added in sending queue, then EVENT_SENDPTZOK and EVENT_SENDPTZFAILED will return message for you to judge whether send successfully.

## AJ_NETSDK_IPC_StartVoiceCom (voice talk)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StartVoiceCom(LONG lUserID,DWORD dwVoiceChan,BOOL bNeedCBNoEncData,fVoiceDataCallBack cbVoiceDataCallBack,void *pUser);

Feature:

Start voice talking

Parameter:

lUserID: a return value

dwVoiceChan: save for compatibility, pass in 0

bNeedCBNoEncData: save for compatibility, pass in 1

cbVoiceDataCallBack: save for compatibility, pass in null

pUser: save for compatibility, pass in null

Return:

0: success

ERR_NOT_FIND_DEVICE: handle error

ERR_IS_STARTAUDIO_ERROR: the device is voice talking

ERR_DEV_NOT_CONNECTED: disconnect devcie

ERR_DEV_NOT_LOGIN: logged failed

Note:

Call the function and return success only tell you it has already requested to device and also you need to wait for response form callback function

EVENT_STARTAUDIOOK=6,// start voice talkback successfully

EVENT_STARTAUDIOFAILED=7,// start voice talkback failed

EVENT_STARTAUDIO_ISBUSY // voice talkback is occuiped

EVENT_STARTAUDIO_PARAMERROR // voice talkback is unavaiable

EVENT_STARTAUDIO_AUDIODDISABLED // not support voice talkback

Note:

It only start voice talkback after call the function, you also need to use AJ_NETSDK_IPC_StartTalk for initializing audio capture and AJ_NETSDK_IPC_AddTalk to add the devie to broadcast list, then voice talk is available

## AJ_NETSDK_IPC_StopVoiceCom (stop voice talkback)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StopVoiceCom(LONG lVoiceComHandle);

Feature:

Stop voice talkback

Parameter:

lUserID: a return value

Return:

0: success

ERR_NOT_FIND_DEVICE: handle error

ERR_AUDIO_NOT_START: does not in voice talkback state

Note:

Return 0 and also need to wait two status from callback function as following to judge whether stop successfully.

EVENT_STOPAUDIOOK,

EVENT_STOPAUDIOFAILED,

## AJ_NETSDK_IPC_FormatDisk (format storage)

Function

LONG		__stdcall		AJ_NETSDK_IPC_FormatDisk(LONG lUserID,LONG lDiskNumber);

Feature:

Format storage device

Parameter:

lUserID: return value

lDiskNumber: 0 is sd1,1 is sd2,2 is USB

Return:

0 means success

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

There is a callback message whose type is 1017 from auxiliary channel callback function, return 0 means the command has already added to the queue. If flag=0 return to XML means success.

## AJ_NETSDK_IPC_Upgrade (upgrade)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_Upgrade(LONG lUserID,char *sFileName);

Feature:

Parameter:

lUserID: return value

sFileName: an absolute path for saving firmware

Return:

0 means request successfully

ERR_NOT_FIND_DEVICE: handle error

ERR_LOC_FILE_ERROR: sFileName error

ERR_ISDOWNLOADING_ERROR: can not upgrade when downloading

ERR_ISUPLOADING_ERROR: can not upgrade when uploading

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

1. Return 0 means the command has already added to the queue

2. If receive ID=EVENT_UPLOADFAILED from StateEventCallBack function means upload file failed

3. If receive ID=EVENT_UPLOADOK from StateEventCallBack function means upload file successfully.

4. Upload compeletely then it will reboot. If upgrade successfully then the version is not same with the previous version

## AJ_NETSDK_IPC_GetUpgradeProgress (obtain progress of upgrading)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetUpgradeProgress(LONG lUserID);

Feature:

Obtain progress of upgrading

Parameter:

lUserID: return value

Return:

ERR_NOT_FIND_DEVICE: handle error

ERR_LOC_FILE_ERROR: sFileName error

Return 101 means does not upgrade or upgrade finish

Return number less than 0 means upgrade failed

Note:

Positive number is effect; negative number is not effect.

## AJ_NETSDK_IPC_GetUpgradeState (obtain upgrading status)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetUpgradeState(LONG lUserID);

Feature:

Obtain upgrading status

Parameter:

lUserID: return value

Return:

0: upgrading

ERR_NOT_FIND_DEVICE: can not find device

101: upgrade compeletely

Negative number: no in upgrade mode

## AJ_NETSDK_IPC_CloseUpgradeHandle (stop upgrading)

Function

LONG		__stdcall		AJ_NETSDK_IPC_CloseUpgradeHandle(LONG lUserID);

Feature:

Stop upgrading

Parameter:

lUserID: return value

Return:

0: means success

ERR_NOT_FIND_DEVICE: handle error

ERR_ISFINISH_ERROR: upload compeletely but does not start to upgrade

## AJ_NETSDK_IPC_FindDVRLogFile (query log)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_FindDVRLogFile(LONG lUserID);

Feature:

Query log

Parameter:

lUserID: return value

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

It will return a number of CMD_GET_LOGFILE_LIST(1025) from AJ_NETSDK_IPC_SetAUXResponseCallBack

## AJ_NETSDK_IPC_RestoreConfig (restore configuration)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_RestoreConfig(LONG lUserID);

Feature:

Restore configuration

Parameter:

lUserID: return value

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

## AJ_NETSDK_IPC_GetConfigFile (dowload configuration file)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetConfigFile(LONG lUserID,char *sFileName);

Feature

Download configuration file

Parameter:

lUserID: return value

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

## AJ_NETSDK_IPC_SetConfigFile (upload and replace configuration file)

Function

LONG		__stdcall		AJ_NETSDK_IPC_SetConfigFile(LONG lUserID,char *sFileName);

Feature

Upload and replace configuration file

Parameter:

lUserID: return value

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

## AJ_NETSDK_IPC_RebootDVR (reboot)

Function

LONG		__stdcall		AJ_NETSDK_IPC_RebootDVR(LONG lUserID);

Feature:

Reboot

Parameter:

lUserID: return value

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

## AJ_NETSDK_IPC_GetDVRConfig (configuration)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetDVRConfig(LONG lUserID,DWORD dwCommand,LONG lChannel,LPVOID lpOutBuffer,DWORD dwOutBufferSize,LPDWORD lpBytesReturned);

Feature

Obtain device configuration

Parameter:

lUserID: return value

dwCommand: configuration information, please refer to schedule2

lChannel: save for compatibility, pass in 0

lpOutBuffer: save for compatibility, pass in null

dwOutBufferSize: save for compatibility, pass in 0

lpBytesReturned: save for compatibility, pass in 0

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

If error flag bit is 0 means success, the message type is the same with dwCommand.

## AJ_NETSDK_IPC_SetDVRConfig (set configuration)

Function

LONG		__stdcall		AJ_NETSDK_IPC_SetDVRConfig(LONG lUserID,DWORD dwCommand,LONG lChannel,LPVOID pXml,DWORD dwInBufferSize);

Feature:

Set configuration

Parameter:

lUserID: return value

dwCommand: configuration information, please refer to schedule2

lChannel: save for compatibility, pass 0

pXml: configure XML content

dwInBufferSize: length of content, strlen(pXml)

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

If error flag bit is 0 means success, the message type is the same with dwCommand.

## AJ_NETSDK_IPC_SystemControl (system control)

Function

LONG		__stdcall		AJ_NETSDK_IPC_SystemControl(LONG lUserIDDWORD dwCommand ,LONG flag,char * pXml);

Feature

Advanced control

Parameter:

lUserID: return value

dwCommand: configuration relevant information, please refer to schedule2

flag: pass in 0

pXml: configure relevant XML content

Return:

0: means success

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

If error flag bit is 0 means success, the message type is the same with dwCommand.

## AJ_NETSDK_IPC_GetDeviceAbility (obtain device ability)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetDeviceAbility(LONG lUserID);

Feature:

Obtain configuration parameter, it will return from auxiliary channel function after reading successfully

Parameter:

lUserID: return value

Return:

0: means success

RR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

Note:

If error flag bit is 0 means success, and type of message is 1020, content of XML:

<RESPONSE_PARAM SystemConfigString=" + audio_support + schedule_record + ftpemail_storage + picture_capture + ptz_control + gpio_input + gpio_output + zh_cn + en_us + zh_tw + front_replay + media_capabiltiy" />

audio_support 		// support audio

schedule_record 		// schedule record

ftpemail_storage 		// support ftp and email

picture_capture 		// support capture

ptz_control 			// support PTZ

gpio_input 			// support IO input

gpio_output 			// support IO output

zh_cn 				// support simplified Chinese

en_us 				// support English

zh_tw 				// support traditional Chinese

front_replay 			// support front-end playback online

media_capabiltiy		// resolution

wireless_accesspoint	// support WIFI as client

wireless_station		// support WIFI as server

network_storage		// support network storage

ptz_action			// support PTZ linkage

ircut_setting			// support IRCUT

evdo_support			// support EVDO 3G

wcdma_support			// support WCDMA 3G

tdscdma_support		// support TDSCDMA 3G

## AJ_NETSDK_IPC_GetReplayAblity (front-end playback online)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetReplayAblity(LONG lUserID);

Parameter:

lUserID: return value

Return:

ERR_NOT_FIND_DEVICE: handle error

Return -1: unknown status, call AJ_NETSDK_IPC_GetDeviceAbility for viewing

0: front-end playback online is unavailable

1: front-end playback online is available

## AJ_NETSDK_IPC_ControlPlay (control front-end playback)

Function

LONG		__stdcall		AJ_NETSDK_IPC_ControlPlay(LONG lUserID,LONG Action,LONG param);

Feature:

Control front-end playback

Parameter

lUserID: the return handle

Action:

enum REPLAY_IPC_ACTION

{

ACTION_PLAY=0,// play

ACTION_PAUSE,// pause

ACTION_RESUME,// re-play

ACTION_FAST,// fast forward

ACTION_SLOW,// slow

ACTION_SEEK,// seek

ACTION_FRAMESKIP,// single frame

ACTION_STOP,// stop

};

Param: (control parameter)

ACTION_FAST, ACTION_SLOW means playback speed

ACTION_SEEK means playback time

Return:

0 means success

ERR_NOT_REPLAY_MODE_ERROR: not in playback mode

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect devie

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

ERR_PLAY_ACTION_ERROR: Action error

## AJ_NETSDK_IPC_SetReplayDataCallBack (front-end playback callback function)

Function

LONG		__stdcall		AJ_NETSDK_IPC_SetReplayDataCallBack(LONG lUserID,fRealDataCallBack cbRealDataCallBack,DWORD dwUser);

Feature:

Set front-end playback callback function

Parameter

lUserID: return handle

cbRealDataCallBack: //callback function for received stream

function: typedef LONG(CALLBACK *fRealDataCallBack)(LONG lRealHandle,DWORD dwDataType,BYTE *pBuffer,DWORD dwBufSize,LPFRAME_EXTDATA  pExtData);

Meaning:

lRealHandle //a return value of the function

dwDataType=0 is video, 1 is audio, 2 is decoding parameter

pBuffer  //media data pointer

when dwDataType= 2, pBuffer structure and its significance for the body

typedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	//==AV_FALG

short  bHaveVideo;//0 means no video parameter

short  bHaveAudio;//0 means no audio parameter

VIDEO_PARAM videoParam;//video parameter

AUDIO_PARAM audioParam;//audio parameter

}STREAM_AV_PARAM;

inside:

typedef struct

{

CHAR codec[256];//H264 or MPEG4, etc, means current vido encoding type

INT width;      //video width

INT height;	  //video height

INT colorbits;  //color bit, invalid here, only use for internal

INT framerate;  //referance frame rate

INT bitrate;    //referance bit rate

BYTE vol_data[256];//extension parameter,necessary data for decoding

INT vol_length;    //extension parameter length

}VIDEO_PARAM;

typedef struct

{

CHAR codec[256];	//audio type,AAC or G711

INT samplerate;	//referance bit rate

INT bitspersample; //referance bit rate

INT channels;		//audio channel quantity

INT framerate;

INT bitrate;

}AUDIO_PARAM;

dwBufSize  //media data length

pExtData  //expansion parameter, when dwDataType=2,this pointer is null, when dwDataType=0,1, structure body and its significance as follow:

typedef struct

{

int		bIsKey; // whether it is key frame

double	timestamp;// time stamp

void    *pUserData;// data pointer

}FRAME_EXTDATA,* LPFRAME_EXTDATA;

Return:

0 means success

## AJ_NETSDK_IPC_PlayDeviceFile (play front-end file)

Function

LONG		__stdcall		AJ_NETSDK_IPC_PlayDeviceFile(LONG lUserID,char * filenme);

Feature

Play specific fron-end file

Parameter

lUserID: return handle

filenme: the file name need to be played

Return

ERR_NOT_FIND_DEVICE: handle error

ERR_NOT_ALLOW_REPLAY_ERROR: not support playback

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

Note:

Before call the function, you need to know about whether front-end play is available and set playback stream callback function by AJ_NETSDK_IPC_SetReplayDataCallBack

## AJ_NETSDK_IPC_SetPlayActionEventCallBack (front-end playback callback)

Funtion:

LONG		__stdcall		AJ_NETSDK_IPC_SetPlayActionEventCallBack(fPlayActionEventCallBack cbAcCallback,void * pUser);

Feature:

Set up front-end playback file relevant event callback

Parameter:

cbAcCallback: received stream callback function,archetype definiton:

typedef LONG(CALLBACK *fPlayActionEventCallBack)(LONG lUser,LONG nType,LONG nFlag,char * pData,void * pUser);

Parameter description:

lUser:control handle return for login

nType:callback event type,definiton as follow

enum REPLAY_IPC_ACTION

{

ACTION_PLAY=0,//play

ACTION_PAUSE,//pause

ACTION_RESUME,//resume

ACTION_FAST,//fast-forward

ACTION_SLOW,//slow

ACTION_SEEK,//skip

ACTION_FRAMESKIP,//single frame

ACTION_STOP,//stop

ACTION_CAPIMG=10,//capture

ACTION_CHANGE_SOUND,//change sound

ACTION_RECV_DECODEPARAM,//receive decoding parameter

};

nFlag:0 means operate normal,others means error code.

pData:relevant data.

pUser:user pointer, called by AJ_NETSDK_IPC_SetPlayActionEventCallBack funtion for specify pUser.

pUser:user pointer,return pUser to user when callback.

Return value:

0 means success

not likely to return others.

## AJ_NETSDK_IPC_SetSearchStatusCallBack (search IPC callback function)

Funtion Archetype:

LONG		__stdcall		AJ_NETSDK_IPC_SetSearchStatusCallBack(SearchIPCCallBack fcallBack,void * pUser);

Feature:

Set searching device state change callback function

Parameter:

fcallBack:callback function,definition archetype:

typedef LONG(CALLBACK *SearchIPCCallBack)(LONG nEventCode,LONG index,IPC_ENTRY *pResponse,void *pUser);

callback funciton description:

nEventCode:update type,definition:

enum SEARCH_EVENT

{

EVENT_SEARCH_RECV_NEWIPCINFO=1,// search IPC,find the new IPC

EVENT_SEARCH_UPDATEINFO,//modify IPC address,address has been changed

};

Index:means device order

pResponse:updated status content

pUser: incoming user pointer wher call AJ_NETSDK_IPC_SetSearchStatusCallBack

pUser:user pointer,the last one parameter to user use for status callback

## AJ_NETSDK_IPC_StartSearchIPC (start to search device)

Function

LONG		__stdcall		AJ_NETSDK_IPC_StartSearchIPC();

Feature

Search devices in a same LAN

Parameter: null

Return:

0 means success

ERR_START_THREADERROR: start thread failed

Note:

- Call AJ_NETSDK_IPC_GetSerchIPCCount for add up the device number and call AJ_NETSDK_IPC_GetIPCInfoXML or AJ_NETSDK_IPC_GetIPCInfo for obtaining IPC information

2.  The search time is more than 5 minutes because of using broadcast package.

## AJ_NETSDK_IPC_StopSearchIPC (stop searching device)

Function

LONG		__stdcall		AJ_NETSDK_IPC_StopSearchIPC();

Parameter

null

Return:

0 means success

## AJ_NETSDK_IPC_GetSearchIPCCount (obtain IPC number)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetSearchIPCCount ();

Parameter:

null

Return:

Return the count of IPC

## AJ_NETSDK_IPC_GetIPCInfo (structure format of IPC information)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetIPCInfo(LONG index, IPC_ENTRY * pIPCInfo);

Feature:

Obtain the index+1 IPC information

Parameter

Index: the IPC number must be less than the number from AJ_NETSDK_IPC_GetSearchIPCCount(),device number begin from 0

pIPCInfo:  // information saved pointer

#define MAX_IP_NAME_LEN 256

#define MAC_ADDRESS_LEN 256

typedef struct

{

char  MACAddress[MAC_ADDRESS_LEN];

int	  dhcpEnable;

char  IPAddress[MAX_IP_NAME_LEN];

char  netMask[MAX_IP_NAME_LEN];

char  gateWay[MAX_IP_NAME_LEN];

char  DNS1[MAX_IP_NAME_LEN];

char  DNS2[MAX_IP_NAME_LEN];

}LANConfig;

typedef struct

{

int auth;

int videoPort;

int rtpoverrtsp;//added by johnnyling 20090323

int ptzPort;

int webPort;

}StreamAccessConfig;

#define GROUP_NAME_MAX_LEN 32

#define ACCOUNT_STATUS_MAX_LEN 8

#define ACCOUNT_NAME_MAX_LEN 40

#define ACCOUNT_PASSWORD_MAX_LEN  40

#define MAX_ACCOUNT_COUNT 20

typedef struct

{

char 	userName[ACCOUNT_NAME_MAX_LEN];

char 	password[ACCOUNT_PASSWORD_MAX_LEN];

char 	group[GROUP_NAME_MAX_LEN];

char    status[ACCOUNT_STATUS_MAX_LEN];

}UserAccount;

typedef struct

{

int count;

UserAccount accounts[MAX_ACCOUNT_COUNT];

}UserConfig;

#define MAX_IPC_SERIALNUMBER	32

#define MAX_DEVICETYPE_LEN		32

typedef struct

{

char				ipc_sn[MAX_IPC_SERIALNUMBER];

char				deviceType[MAX_DEVICETYPE_LEN];

UserConfig			userCfg;

StreamAccessConfig	streamCfg;

LANConfig			lanCfg;

}IPC_ENTRY;

Return

0 means success

ERR_PARAM_ERROR: parameter error, may be pIPCInfo is empty or index is not a valid value

## AJ_NETSDK_IPC_ModifyIPC (modify structure format of IPC information)

Function

LONG		__stdcall		AJ_NETSDK_IPC_ModifyIPC(LONG index, IPC_ENTRY * pIPCInfo);

Feature:

Index: the IPC number must be less than the number from AJ_NETSDK_IPC_GetSearchIPCCount(), device number begin from 0

pIPCInfo: //save pointer information

#define MAX_IP_NAME_LEN 256

#define MAC_ADDRESS_LEN 256

typedef struct

{

char  MACAddress[MAC_ADDRESS_LEN];

int	  dhcpEnable;

char  IPAddress[MAX_IP_NAME_LEN];

char  netMask[MAX_IP_NAME_LEN];

char  gateWay[MAX_IP_NAME_LEN];

char  DNS1[MAX_IP_NAME_LEN];

char  DNS2[MAX_IP_NAME_LEN];

}LANConfig;

typedef struct

{

int auth;

int videoPort;

int rtpoverrtsp;//added by johnnyling 20090323

int ptzPort;

int webPort;

}StreamAccessConfig;

#define GROUP_NAME_MAX_LEN 32

#define ACCOUNT_STATUS_MAX_LEN 8

#define ACCOUNT_NAME_MAX_LEN 40

#define ACCOUNT_PASSWORD_MAX_LEN  40

#define MAX_ACCOUNT_COUNT 20

typedef struct

{

char 	userName[ACCOUNT_NAME_MAX_LEN];

char 	password[ACCOUNT_PASSWORD_MAX_LEN];

char 	group[GROUP_NAME_MAX_LEN];

char    status[ACCOUNT_STATUS_MAX_LEN];

}UserAccount;

typedef struct

{

int count;

UserAccount accounts[MAX_ACCOUNT_COUNT];

}UserConfig;

#define MAX_IPC_SERIALNUMBER	32

#define MAX_DEVICETYPE_LEN		32

typedef struct

{

char				ipc_sn[MAX_IPC_SERIALNUMBER];

char				deviceType[MAX_DEVICETYPE_LEN];

UserConfig			userCfg;

StreamAccessConfig	streamCfg;

LANConfig			lanCfg;

}IPC_ENTRY;

Return

0 means success

ERR_PARAM_ERROR: parameter error, may be pIPCInfo is empty or index is not a valid value

Note:

1.Because of using broadcast packets to modify IPC information, we can only through keep searching to find devices.When find a new device, through the same serial number to judge relevant value, if relevant value consistent with the value we modified, it means modify IPC information successfully.

2.Permit to modify network parameter and server port,only LANConfig and StreamAccessConfig structure content can be modified,other modification is invalid.

Function return success only means modified broadcast packet has been sent, modified operation successfully or not can be judged through IP is modified successfully or not after the EVENT_SEARCH_UPDATEINFO event which search callback function receive. User should set a certain time for timeout to estimate modify successfully or not,if callback exceed fixed time regard as IPC modification failure.

## AJ_NETSDK_IPC_GetIPCInfoXML (obtain device information of XML format)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetIPCInfoXML(LONG index, char * pXMLInfo,int maxLen);

Parameter:

Index: the IPC number must be less than the number from AJ_NETSDK_IPC_GetSearchIPCCount() and number count begin from 0

pXMLInfo: information saved pointer

maxLen: If pXMLInfo is not NULL, then it means the maximun available memory in byte of pXMLInfo

Return:

If pXMLInfo is NULL, then it will return the required length of XML

ERR_PARAM_ERROR: parameter error, maybe pIPCInfo is empty or index is not a valid value

ERR_MEMORY_SIZE_ERROR: the required memory is more than maxLen

ERR_OUTOFF_MEMORY: out of memory

Returned XML content example as follow:

<IPC_INFO>

<IPC_SERIALNUMBER

SerialNumber="0000000000005971"    //device serial number

/>

<LANConfig

MacAddress="00:3e:71:22:ee:19"		//MAC address,ordinary don’t modify

DHCP="0"						//whether enable DHCP

IPAddress="192.168.88.151"		//IP address

Netmask="255.255.255.0"			//subnet mask

Gateway="192.168.88.1"			//gateway address

DNS1="202.96.128.86"				//host DNS

DNS2="202.96.134.133"			//reserved DNS

/>

<MediaStreamConfig>

<StreamAccess

Auth="1"					//whether enable authentication

VideoPort="554"				//video stream port

RTPOverRTSP="1"			//whether TCP is used when IPC access video

PTZPort="8091"				//auxiliary channel port

WEBPort="8080"				//access port of WEB configuration interface

/>

</MediaStreamConfig>

<UserConfig>

<Account

Username="admin"			//login username

Password="123456"		//login password

Group="Administrator"	//user classification: Administrator means administrator, Operator means operator, Viewer means viewer

Status="Enable"		//account status, when status is Enable account can be used, Disable means user account is locked

/>

<Account

Username="test"			//login username

Password="123456"		//login password

Group="Operator"			// user classification: Administrator means administrator, Operator means operator, Viewer means viewer

Status="Enable"		// account status, when status is Enable means user account can be used, Disable means user account is locked

/>

</UserConfig>

</IPC_INFO>

## AJ_NETSDK_IPC_ModifyIPCXML (modify IP address XML format)

Function

LONG		__stdcall		AJ_NETSDK_IPC_ModifyIPCXML(LONG index, const char * strXML);

Parameter

Index: the IPC number must be less than the number from AJ_NETSDK_IPC_GetSearchIPCCount() and number count begin from 0

strXML: configuration inforamtion,corresponding format:

<IPC_INFO>

<LANConfig

MacAddress="00:3e:71:22:ee:19"		//MAC address,ordinary don’t modify

DHCP="0"						//whether enable DHCP

IPAddress="192.168.88.151"		//IP address

Netmask="255.255.255.0"			//subnet mask

Gateway="192.168.88.1"			//gateway address

DNS1="202.96.128.86"				//host DNS

DNS2="202.96.134.133"			//reserved DNS

/>

<MediaStreamConfig>

<StreamAccess

Auth="1"					//whether enable authentication

VideoPort="554"				//video stream port

RTPOverRTSP="1"			//choose TCP or not when IPC access video

PTZPort="8091"				//auxiliary channel port

WEBPort="8080"				//access port of WEB configuration interface

/>

</MediaStreamConfig>

</IPC_INFO>

Include at least one of LANConfig or StreamAccess inside.

Return”

0 means success

ERR_PARAM_ERROR: parameter error, maybe pIPCInfo is empty or index is not a valid value

ERR_MEMORY_SIZE_ERROR: the required memory is more than maxLen

ERR_OUTOFF_MEMORY: out of memory

ERR_XML_FORMAT_ERROR: strXML format error

ERR_CREATE_SOCKET_ERROR: create Socket failed

ERR_SEND_MODIFYCMD_ERROR: send failed

Note:

Function return success only means modified broadcast packet has been sent, modified operation successfully or not can be judged through IP is modified successfully or not after the EVENT_SEARCH_UPDATEINFO event which search callback function receive. User should set a certain time for timeout to estimate modify success or not,if callback exceed fixed time regard as IPC modification failure.

## AJ_NETSDK_IPC_GetOneIPAddress (obatain a avaiable IP address)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetOneIPAddress(char * strResult,int nSize);

Parameter

strResult: location to save the IP address

nSize: memory address, 33-bit

Return:

ERR_MEMORY_SIZE_ERROR: the required memory is more than nSize

If strResult=NULL, then return 33, otherwise return the actual bit number

## AJ_NETSDK_IPC_GetNetworkParam (obtain network configuration)

Function

LONG		__stdcall		AJ_NETSDK_IPC_GetNetworkParam(ULONG nParamIndex, char * strResult,int nSize);

Parameter

nParamIndex: the type of parameter to be read

meaning:

enum

{

NETWORK_PARAM_IPADDRESS=0,// obtain IP address

NETWORK_PARAM_NETMASK,// obtain netmask

NETWORK_PARAM_GATEWAY,// obtain gateway

NETWORK_PARAM_DNS1,// obtain main DNS

NETWORK_PARAM_DNS2,// obtain auxiliary DNS

NETWORK_PARAM_MAX// end flag(no use)

};

strResult: save poiter

nSize: avaiable memory size, generally more than 32(make sure success, don’t be less than 16)

Return

0 means success

ERR_PARAM_ERROR: nParamIndex parameter error

ERR_MEMORY_SIZE_ERROR: the actual memory more than nSize

## AJ_NETSDK_IPC_StartTalk (initialize broadcast parameter)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StartTalk(int talkMode, int audiotype, int samplerate, int bitspersample, int channels);

Parameter:

talkMode: Indicates intercom mode, 0 means local acquisition of audio, 1 means that the input of custom audio data.

audiotype:means audio type,1 means AAC,0 means G711

samplerate:sampling rate,AAC generally as 16000,G711 generally as 8000(parameter can look in samplerate of AUDIO_PARM parameter)

bitspersample:generally as 16(parameter can look in bitspersample of AUDIO_PARM parameter)

channels:voice channel,AAC generally as 2,G711 generally 1(parameter can look in channels of AUDIO_PARM parameter)

typedef struct

{

char codec[256];

int samplerate;

int bitspersample;

int channels;

int framerate;

int bitrate;

}AUDIO_PARAM;

Return:

0 means success

ERR_OPEN_AUDIOCAPTURE_FAIL: device opens audio capture failure,may be sound card capture is occupied.

ERR_START_AUDIOCAPTURE_FAIL:starts audio capture failure ,may be parameter is wrong.

Note:

If current talkback has been started, first to close current talkback and reuse the specific parameter to start talkback.

Example:

G711 talkbalk call AJ_NETSDK_IPC_StartTalk(0,8000,16,1)

AAC talkback call AJ_NETSDK_IPC_StartTalk(1,16000,16,2);( front-end IPC set up 16K)

## AJ_NETSDK_IPC_StopTalk (release broadcast resource)

Function

LONG		__stdcall		AJ_NETSDK_IPC_StopTalk();

Parameter

NUll

Return:

0 means success

Note:

If start broadcast then all voice talk would be stopped

## AJ_NETSDK_IPC_InputAudioData Enter the custom PCM intercom audio data.

Function：

LONG __stdcall AJ_NETSDK_IPC_InputAudioData(BYTE* pBuff, int len);

Parameter：

pBuff：AudioData。

Len：Data length。

Return：

0：means success。

Note：

First callAJ_NETSDK_IPC_StartTalk et the relevant parameters talkMode parameter data is 1..

And then calls AJ_NETSDK_IPC_AddTalk to add the device to the broadcast list。

## AJ_NETSDK_IPC_AddTalk (add device for broadcast)

Function

LONG		__stdcall		AJ_NETSDK_IPC_AddTalk(LONG lUserID);

Parameter

lUserID: the return handle

Return:

0 means success

ERR_NOT_FIND_DEVICE: handle error

ERR_NOT_STARTTALK_MODE_ERROR: uninitialize broadcast parameter, please call AJ_NETSDK_IPC_StartTalk to set up corresponding parameter

ERR_AUDIO_PARAM_ERROR: the device does not start voice talk, please call AJ_NETSDK_IPC_StartVoiceCom to add the deivce for broadcast

ERR_NOT_STARTTALK_MODE_ERROR: uninitialize broadcast, please call AJ_NETSDK_IPC_StartTalk for initialzation.

Note:

Call AJ_NETSDK_IPC_StartVoiceCom to start voice talkback, if the parameter is the same with call AJ_NETSDK_IPC_StartTalk then add device for broadcast

If call this function error, please call AJ_NETSDK_IPC_StopVoiceCom to stop voice talk, otherwise after a while will automatically reconnect talkback and receive EVENT_STARTAUDIOOK event

## AJ_NETSDK_IPC_RemoveTalk (stop specific voice talkback)

Function

LONG		__stdcall		AJ_NETSDK_IPC_RemoveTalk(LONG lUserID);

Parameter

lUserID: the return flag

Return

0 means success

ERR_NOT_FIND_DEVICE: hanlde error

ERR_AUDIO_NOT_START: the device does not start voice talkback

## AJ_NETSDK_IPC_CreateIFrame (create key frame)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_CreateIFrame(ULONG lUserId,int bIsSubStream);

Parameter:

lUserID: the return flag

bIsSubStream: 0 means main stream, 1 means sub stream

Return:

0 means success

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

Note:

The function does not judge whether request media stream

## AJ_NETSDK_IPC_SetUserData (save user data)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_SetUserData(ULONG lUserId,char * pBuffer,int len);

Parameter

lUserID: the return flag

pBuffer: the data to be saved, must be string,for binary data or can’t be printed character must be %02d to change to string

len: data length

Return:

0: success to join the queue, auxiliay channel callback function will return the result

-1: pBuffer is NULL

ERR_NOT_FIND_DEVICE: handle error

ERR_DEV_NOT_CONNECTED: disconnect device

ERR_DEV_NOT_LOGIN: logged failed

ERR_OUTOFF_MEMORY: out of memory

It will receive nType=1034 from AJ_NETSDK_IPC_SetAUXResponseCallBack callback function when success

## AJ_NETSDK_IPC_GetUserData (read saved data)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetUserData(ULONG lUserId,void * pOutBuffer,int* nInOutLen);

Parameter:

lUserID: the return flag

void * pOutBuffer: data saved location, if pOutBuffer=NULL then return immediately

int* nInOutLen: it is available size of pOutBuffer before called, it is actual size after called. If * nInOutLen<=0 then return immediately

Return:

0 means success to add to queue

Note:

After data reading successfully, it will receive a nType= CMD_GET_USER_DEV_DATA (1033) event from AJ_NETSDK_IPC_SetAUXResponseCallBack callback function,XML format :

<RESPONSE_PARAM UserDevData="616263646566"/>,UserDevData is the result of user’s setting data %02x.in pratice,string per 2 bits converts by hexadecimal to turn into a character,for example: The transformed result of 616263646566 is abcdef; abcdef is the actual setting data.

## AJ_NETSDK_IPC_StartRecord(Start Record)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StartRecord(LONG lRealHandle,const char * filePath,int nFileMaxSeconds,int nAllRecordMaxSeconds);

Parameter:

lRealHandle:request stream return.

filePath:saved directory or filename,directory must be exist otherwise file can not be created,incoming absolutely path must be not exceed 512 bytes,if path is suffix as .avi or .AVI regard as create single file and nAllRecordMaxSeconds is invalid.

nFileMaxSeconds:means maximum video duration of single file,support to record 10 to 3600 seconds, video high definiton 1080P above parameter value should not bigger than 1800.

nAllRecordMaxSeconds: video record duration

Return:

0:means success

ERR_RECORD_FILEMAXSECONDS_ERROR:means parameter nFileMaxSeconds error,available value should be 10 to 1800

ERR_RECORD_ALLRECORDSECONDS_ERROR:means parameter nAllRecordMaxSeconds error,must be bigger than 0

ERR_NOT_FIND_STREAM:means corresponding video stream of lRealHandle has not been found

ERR_RECORD_ISRECORDING:means recoding, stop recoding first when you want to record a new video

ERR_RECORD_MEDIA_PARAM_ERROR:means video parameter error

ERR_RECORD_FILEPATH_ERROR:means filePath error,filepath is null or less than 2 characters

Note:

1.Incoming file must be exist,otherwise cannot record.

2.When recording it will create current date for directory name on the directory you set up,fromat as 2012-03-11.

3.video file name will automatically generated by record time,format as 155301-av-2.avi,155302-vv-1.avi,front of six bits for time,middle position av means it has audio,vv means no audio,1 means mainstream,2 means substream.

4.Start recording when hard disk space available bigger than 2G,otherwise it will receive a EVENT_RECORD_DISKFREESPACE_TOOLOW event.

5.When video high definition 1080P,nFileMaxSeconds suggest to set up less than 1800.

6.If name suffix as .avi or .AVI,regard as create single file,nAllRecordMaxSeconds invalid.Internal will force to set it as nFileMaxSeconds to video single file.

Event may be received:

EVNET_RECORD_FILEBEGIN,//begin to record a file

EVENT_RECORD_FILEEND,//finish recording a file

EVENT_RECORD_TASKEND,//finish all video,video will stop

EVENT_RECORD_DISKFREESPACE_TOOLOW,//hard disk space avaiable too small,will not begin to video and wait for enough space

EVENT_RECORD_MEDIA_PARAM_ERROR,//video parameter error,request video stream may be failure EVENT_RECORD_INITAVIHEAD_ERROR,//write file head error

EVNET_RECORD_FILEBEGIN_ERROR,//create file error

EVNET_RECORD_WRITE_FILE_ERROR,	//write file error

## AJ_NETSDK_IPC_StopRecord(stop Record)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StopRecord(LONG lRealHandle);

Parameter:

lRealHandle:request stream return

Return:

0: means success

ERR_NOT_FIND_STREAM:lRealHandle handle not found

ERR_RECORD_NOTRUNNING:this stream not in the video or has been stopped

## AJ_NETSDK_IPC_GetLastErrorCode(get and eliminate error code)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_GetLastErrorCode(int nType);

Parameter:

nType:invalid at present,fixed set it’s value 1,use for extension

Return:

Non-zero means error code

0:means no error

Note:

1.Incoming file must be exist,otherwise cannot record.

2.When recording it will create current date for directory name on the directory you setup,fromat as 2012-03-11.

3.video file name will automatically generated by record time,format as 155301-av-2.avi,155302-vv-1.avi,front of six bits for time,middle position av means it has audio,vv means it has not audio,1 means mainstream,2 means substream.

4.Start recording when hard disk space available bigger than 2G,otherwise it will receive a EVENT_RECORD_DISKFREESPACE_TOOLOW event.

5.For high definition 1080P video,nFileMaxSeconds value suggest to set less than 1800

6.If name suffix as .avi or .AVI,regard as create single file,nAllRecordMaxSeconds invalid. Internal will force to set it as nFileMaxSeconds to video single file.

## AJ_NETSDK_IPC_StartRecordStream(enable stream mode record)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StartRecordStream(STREAM_AV_PARAM * pAvParam,const char * filePath,int nFileMaxSeconds,int nAllRecordMaxSeconds);

Parameter:

pAvParam:video and audio parameter,this parameter can get in EVENT_RECVVIDEOAUDIOPARAM event callback,specific structure:

typedef struct

{

char codec[256];

int width;

int height;

int colorbits;

int framerate;

int bitrate;

BYTE vol_data[256];

int vol_length;

}VIDEO_PARAM;

typedef struct

{

char codec[256];

int samplerate;

int bitspersample;

int channels;

int framerate;

int bitrate;

}AUDIO_PARAM;

typedef struct  __STREAM_AV_PARAM

{

unsigned char	ProtocolName[32];	//set at H264

short  bHaveVideo;//0 means no video parameter

short  bHaveAudio;//0 means no audio parameter

VIDEO_PARAM videoParam;//video parameter

AUDIO_PARAM audioParam;//audio parameter

}STREAM_AV_PARAM;

filePath:saved directory or filename,directory must be exist otherwise file cannot be created,should not exceed 512 characters, if directory name is suffix as .avi or .AVI regard as create single file.

nFileMaxSeconds:maximum record duration for single file, record time from 10 to 1800 seconds.

nAllRecordMaxSeconds:total sum of record duration

Return:

0:means failure, call AJ_NETSDK_IPC_GetLastErrorCode(1) to get error code after return failure

Return stream control handle when success

Note:

If name suffix as .avi or .AVI, regard as create single file,nAllRecordMaxSeconds invalid here.Internal will force to set it as nFileMaxSeconds to video single file.

Event may be received in status callback function:

EVNET_RECORD_FILEBEGIN,//begin to record a file

EVENT_RECORD_FILEEND,//finish recording a file

EVENT_RECORD_TASKEND,//finish all record, stop recording

EVENT_RECORD_DISKFREESPACE_TOOLOW,// hard disk space avaiable too small,will not begin to record and wait for enough space

EVENT_RECORD_MEDIA_PARAM_ERROR,// video parameter error, request video stream may be failure

EVENT_RECORD_INITAVIHEAD_ERROR,//write file head error

EVNET_RECORD_FILEBEGIN_ERROR,//create file error

EVNET_RECORD_WRITE_FILE_ERROR,	//write file error

## AJ_NETSDK_IPC_InputRecordStream(record file by input media data)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_InputRecordStream(LONG lRealHandle,const void * pBuffer,int nSize,int isVideo,int iskey,double timestamp);

Parameter:

lRealHandle: return by call AJ_NETSDK_IPC_StartRecordStream

pBuffer:media data

nSize:size of media data characters

isVideo:whether it is video media data

iskey:whether it is keyframe

timestamp:timestamp

Return:

0:means success

ERR_RECORD_ISNOTRECORDSTREAM_MODE:current handle is realtime stream mode,not allow to call this function

ERR_RECORD_WRITETEMPBUFFER_ERROR:means write cache failure

ERR_RECORD_NOTRUNNING: means no enable video or video has stopped

## AJ_NETSDK_IPC_StopRecordStream (stop outer media recording)

Function:

LONG		__stdcall		AJ_NETSDK_IPC_StopRecordStream(LONG lRealHandle);

Parameter:

lRealHandle:it is stream control handle return by call AJ_NETSDK_IPC_StartRecordStream function

Return:

0:means success

ERR_NOT_FIND_STREAM:means handle error

ERR_RECORD_NOTRUNNING:means no in video mode

ERR_RECORD_NOTINPUTSTREAM_MODE:means not in the input stream record mode

## AJ_NETSDK_IPC_AddInviteAudioStream(request capture compressed audio)

Prototype:

LONG	__stdcall	AJ_NETSDK_IPC_AddInviteAudioStream(LONG * lPort);

Parameter:

lPort: If success, this parameter is control handle

Return:

0 means success

ERR_DLL_NOINITORRELEASE_ERROR: The library has been released

ERR_NOT_STARTTALK_MODE_ERROR: Has not called AJ_NETSDK_IPC_StartTalk to start capture.

ERR_OUTOFF_MEMORY: out of memory

Note:

Call AJ_NETSDK_IPC_StartTalk to start capturing audio before calling this function.

## AJ_NETSDK_IPC_SetEncodeAudioCallBack(set capture compressed audio callback)

Prototype:

LONG	__stdcall	AJ_NETSDK_IPC_SetEncodeAudioCallBack(LONG lPort,fEncodeAudioCallBack fnCallBack);

Parameter:

lPort: control handle, return by AJ_NETSDK_IPC_AddInviteAudioStream

fnCallBack: callback function, function prototype:

typedef	long (CALLBACK* fEncodeAudioCallBack)(long lType, long lPara1, long lPara2);

when callback,

lType:0x10000002(fixed value)

lPara1: compressed audio data pointer

lPara2: compressed audio data length

Return:

0: means success

ERR_DLL_NOINITORRELEASE_ERROR: the library has been released

ERR_NOT_FIND_DEVICE: parameter handle lPort error

## AJ_NETSDK_IPC_RemoveInviteAudioStream(cancle capture compressed audio)

Prototype:

LONG	__stdcall	AJ_NETSDK_IPC_RemoveInviteAudioStream(LONG lPort);

Parameter:

lPort: control handle, return by AJ_NETSDK_IPC_AddInviteAudioStream

Return:

0: means success

ERR_NOT_FIND_DEVICE: parameter handle lPort error

## AJ_NETSDK_IPC_SerialStart (open serial port)

Prototype:

AJ_NETSDK_IPC_SerialStart(LONG lUserId, SerialDataCallBack cbSDCallBack, void* pUser);

Parameter:

lUserId: return by login device, mark up from which device.

pUser: user pointer, return as callback function.

cbSDCallBack: serial data receive callback function.

Calback function architype definition:

LONG (CALLBACK* SerialDataCallBack) (LONG lUser, char *pRecvDataBuffer, DWORD dwBufSize, void *pUser);

Callback function parameters description:

lUser: return by login device, mark up from which device.

pRecvDataBuffer: received serial data, return as alarm structure mode, read mode: ALARM_ITEM *pAlarmItem = (ALARM_ITEM*)pRecvDataBuffer;

dwBufSize: data length equal with sizeof（ALARM_ITEM）。

pUser: user pointer

Return:

0: success

Non-zero: means failure, ERR_NOT_FIND_DEVICE means lUserId which user imports is error.

## AJ_NETSDK_IPC_SerialSend (send serial data)

Prototype:

AJ_NETSDK_IPC_SerialSend(LONG lUserId, LONG lChannel, char *pSendBuf);

Parameter:

lUserId: return by login device, mark up from which device.

lChannel: menas the channel communication port which wants to communicate, cause IPC camera only has one channel, invalid temporary.

pSendBuf: the data “buf” you want to send. Data format as follow:

<xml>

<cmd>trans</cmd>

<data>control command</data>

</xml>

Return:

0: success

Non-zero: means failure, ERR_NOT_FIND_DEVICE means lUserId which user imports is error.

## AJ_NETSDK_IPC_SerialStop (close serial port)

Prototype:

AJ_NETSDK_IPC_SerialStop(LONG lUserId);

Parameter:

lUserId: return by login device, mark up from which device.

Return:

0: success

Non-zero: means failure, ERR_NOT_FIND_DEVICE means lUserId which user imports is error.

Steps of using AJNETSDK

## Note about AJNETSDK

Note:

- Should not deal with the callback function directly, analyze and submit it to other thread is a good method.

- Return success about auxiliary channel only means the task has been applied and status callback function or auxiliary channel function will tell you the result.

- Before use SDK, call AJ_NETSDK_IPC_Init for exit SDK rapidly and release resources. When not use it any more then call AJ_NETSDK_IPC_Cleanup but not AJ_NETSDK_IPC_StopRealPlay or AJ_NETSDK_IPC_Logout. If you want to stop all stream, call AJ_NETSDK_IPC_StopAllRealPlay. Call AJ_NETSDK_IPC_LogoutAll for logout device.

- About C# language (not C++), you should change char * into pointer (C# is IntPtr) but not string.

- About media stream decode. If you want to decode youself then you should remove the subsequent 16-bit, please refer to AJ_NETSDK_IPC_SetRealDataCallBack for more information

## Steps of realizing video stream play

1. Call AJ_NETSDK_IPC_Init to initialize AJNETSDK

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up callback function

3. Call AJ_NETSDK_IPC_SetRealDataCallBack to set up the received media data callback function (if the return value of callback function in step 4 is not null then this step can be omitted, generally condition, the format to call this function can be: AJ_NETSDK_IPC_SetRealDataCallBack(NULL,NULL);

4. Call AJ_NETSDK_IPC_RealPlayEx to request video stream

5. when receive EVENT_RECVVIDEOPARAM event and EVENT_RECVVIDEOAUDIOPARAM event from status callback function or receive EVENT_RECVVIDEOAUDIOPARAM event or receive media data type is 2 from media stream callback function (0 means video media,1 means audio media,2 means decoding parameter media), call IP_TPS_OpenStream to initialize decoder (if device has audio, call IP_TPS_OpenStream twice and nPort must be same as the first time), call IP_TPS_Play to enter the status of play.

6. When it receives media data, call IP_TPS_InputVideoData to trasfer video stream to player control, call IP_TPS_InputAudioData to transfer video stream to player control.

7. Call AJ_NETSDK_IPC_StopRealPlay to stop receving stream (call this function if want to stop one of the streams)

8. Call IP_TPS_Stop to stop video.

9. Call IP_TPS_ReleaseAll to release play control resource.

10. Call AJ_NETSDK_IPC_Cleanup to release AJNETSDK control resource.

Step 1 to 3 to initialize SDK, step 4 to 8 to play, step 9 to 10 to release SDK resource.

Note:

IP_TPS_Play, IP_TPS_OpenStream, IP_TPS_Stop, IP_TPS_ReleaseAll, IP_TPS_InputVideoData and IP_TPS_InputAudioData function are provided in DllPlayer.dll.

Call AJ_NETSDK_IPC_Cleanup to release resource when you want to exit application.

## Realize auxiliary channel connection for PTZ control, system control, device configuration, etc

1. Call AJ_NETSDK_IPC_Init to initialize AJNETSDK

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up callback function.

3. Call AJ_NETSDK_IPC_SetAUXResponseCallBack to set up auxiliary communication information callback function.

4. Call AJ_NETSDK_IPC_Login to set up login.

5. It means login successfully when you receive EVENT_LOGINOK event from status callback function.

6. Call relevant function after login successfully,such as the basic ptz control function:AJ_NETSDK_IPC_PTZControl to control ptz.

7. Call AJ_NETSDK_IPC_Logout to logout.

8. Call AJ_NETSDK_IPC_Cleanup to release AJNETSDK control resource, then it will exit all login and stop all request stream.

Note:

PTZ control will receive an EVENT_PTZPRESETINFO event from status callback function; it will return operation result from auxiliary channel message callback function when you read configuration.modify configuration.system control and etc.

LONG  OnAUXResponseCallBack(LONG lUser,LONG nType,char *pResponse,void *pUser);

Parameter:

lUser:login device return this parameter,to identify from which device

nType:means message type,the top bit of nType is flag bit(nType & 0xff000000!=0),if the flag bit is non-zero,it means operation failure.

Low 24 bits(Type=nType & 0xffffff)is actual type,different values indicate different operations,details of Schedule 2

pUser:user pointer,the same as pUser parameter of the function which it is to set up OnAUXResponseCallBack

## Steps of realizing audio broadcast

1. Call AJ_NETSDK_IPC_Init to initilize library.

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up status callback function.

3. Call AJ_NETSDK_IPC_StartTalk function

4. Call login device function:m_nLastCmdId=AJ_NETSDK_IPC_Login().

5. Call AJ_NETSDK_IPC_StartVoiceCom(m_nLastCmdId,0,1,NULL,NULL) to start device talkback funciton;

6. Wait for below events in status callback function:

EVENT_STARTAUDIOOK,//talkback starts successfully

EVENT_STARTAUDIOFAILED,//talkback starts failed

if it is EVENT_STARTAUDIOOK call AJ_NETSDK_IPC_AddTalk(m_nLastCmdId)

7. Repeat 4 to 7 to add more devices.

8. If you want to quit broadcast of one device,first to call AJ_NETSDK_IPC_RemoveTalk function then call  AJ_NETSDK_IPC_StopVoiceCom function.

9. If you want to quit broadcast,call AJ_NETSDK_IPC_StopTalk to stop and release resource.

## Steps of controlling PTZ

1. Call AJ_NETSDK_IPC_Init to initialize library.

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up status callback function.

3. Call AJ_NETSDK_IPC_SetAUXResponseCallBack to set up auxiliary channel information callback function.

4. Call login device function:m_nLastCmdId=AJ_NETSDK_IPC_Login().

5. It means login successfully when callback function receive EVENT_LOGINOK event.

6. Call PTZ control command(AJ_NETSDK_IPC_PTZControl or AJ_NETSDK_IPC_PTZControlEx)

## Steps of receiving stream record

1. Call AJ_NETSDK_IPC_Init to initialize library.

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up OnStatusEventCallBack status callback function.

3. Call AJ_NETSDK_IPC_RealPlayEx to request video stream.

4. After OnStatusEventCallBack status callback function has been received EVENT_RECVVIDEOAUDIOPARAM then execute step 5(note:step5 execute after step 4 executing successfully)

5. Call AJ_NETSDK_IPC_StartRecord,you may call AJ_NETSDK_IPC_StartRecordFile if it is recording a file.

6. Call AJ_NETSDK_IPC_StopRecord function.

7. Call AJ_NETSDK_IPC_StopRealPlay to stop receiving stream and releasing relevant resource.

## Steps of outer input stream record

1. Call AJ_NETSDK_IPC_Init to initialize library.

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up status callback function.

3. Call AJ_NETSDK_IPC_StartRecordStream to play outer media stream.

4. Repeatedly call AJ_NETSDK_IPC_InputRecordStream to input data.

5. Call AJ_NETSDK_IPC_StopRecordStream to stop and release relevant resource.

## Steps of playing front-end record file

1. Call AJ_NETSDK_IPC_Init to initialize library.

2. Call AJ_NETSDK_IPC_SetStatusEventCallBack to set up status callback function.

3. Call AJ_NETSDK_IPC_SetReplayDataCallBack to set up callback function of receiving front-end playback media stream.

4. Call AJ_NETSDK_IPC_SetPlayActionEventCallBack to set up and control callback.

5. Call AJ_NETSDK_IPC_Login to login device.

6. When login signal from status callback function return success, call AJ_NETSDK_IPC_GetReplayAblity to search device whether support front-end online playback,continue when support.

7. Call AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_RECORD_FILE_LIST,0,searchxml) to search video file.

8. Call AJ_NETSDK_IPC_PlayDeviceFile to play the file which has been searched.

9. Receive decoding parameter from callback function that AJ_NETSDK_IPC_SetReplayDataCallBack set up, then initialize decoder and at the same time input media stream into decoder.

10. Call AJ_NETSDK_IPC_ControlPlay to control front-end video play.

## Steps of searching online device

1. Call AJ_NETSDK_IPC_Init to initialize library.

2. Call AJ_NETSDK_IPC_SetSearchStatusCallBack to search status callback function.

3. Call AJ_NETSDK_IPC_StartSearchIPC to begin to search device.

4. Call AJ_NETSDK_IPC_GetSearchIPCCount to get the sum of searching IPC

5. If device has been searched,call AJ_NETSDK_IPC_GetIPCInfo or AJ_NETSDK_IPC_GetIPCInfoXML function to get the corresponding device’s relevant information

6. If want to modify IP relevant information of IPC,call AJ_NETSDK_IPC_ModifyIPC or AJ_NETSDK_IPC_ModifyIPCXML

7. Call AJ_NETSDK_IPC_StopSearchIPC to stop searching IPC

Call callback function to notice when new device is searched.

## Receive audio capture compressed stream

//define callback

long __stdcall  fAudioRecv(long lType, long lPara1, long lPara2 )

{

if(lType== 0x10000002)

{

TRACE1("recv encode audio buffer len=%d\r\n",lPara2);

}

return 0;

}

int m_nAudioStreamHandle=0;

int nCode=::AJ_NETSDK_IPC_StartTalk(1,16000,16,2);//aac

if(nCode!=0)

{//start failed

return;

}

int nErrorCode=::AJ_NETSDK_IPC_AddInviteAudioStream(&m_nAudioStreamHandle);

if(nErrorCode==0)

{

::AJ_NETSDK_IPC_SetEncodeAudioCallBack(m_nAudioStreamHandle,fAudioRecv);

}

# Schedule 1 PTZ XML command

# PTZ instruction

Mainly to explain XML command between IPC and auxiliary channel PTZ control. There are many protocols which are abstracted into XML format in camera underlying, call XML command for connecting PTZ

Call API AJ_NETSDK_IPC_PTZControlEx(LONG lUser,char *pXml) of NETSDK for PTZ command

# XML command

The XML command mainly reference PELCO-P, extend to a lot of PTZ, but there is difference among different compan, so can not sure to support all PTZ

Command format: <xml></xml> as nodes and there are <cmd></cmd> means different command,specific information please refer section 2.1.

## Ordinary command

### UP

<xml>

<cmd>up</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed, this value is no use for up

tiltspeed: value from 1 to 10, the higher value the faster speed

### down

<xml>

<cmd>down</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed: value from 1 to 10, the higher value the faster speed

### left

<xml>

<cmd>left</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed: value from 1 to 10, the higher value the faster speed

### right

<xml>

<cmd>right</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed: value from 1 to 10, the higher value the faster speed

### Left-up

<xml>

<cmd>left_up</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed:  value from 1 to 10, the higher value the faster speed

### Left-down

<xml>

<cmd>left_down</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed: value from 1 to 10, the higher value the faster speed

### Right-up

<xml>

<cmd>right_up</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed: value from 1 to 10, the higher value the faster speed

### Right-down

<xml>

<cmd>right_down</cmd>

<panspeed>5</panspeed>

<tiltspeed>5</tiltspeed>

</xml>

panspeed: value from 1 to 10, the higher value the faster speed

tiltspeed: value from 1 to 10, the higher value the faster speed

### stop

<xml>

<cmd>stop/cmd>

</xml>

### zoom out

<xml>

<cmd>zoomwide</cmd>

</xml>

### zoom in

<xml>

<cmd>zoomtele</cmd>

</xml>

### FocusAutoOn

<xml>

<cmd>FocusAutoOn</cmd>

</xml>

### FocusAutoOff

<xml>

<cmd> FocusAutoOff</cmd>

</xml>

### focus- (focusnear)

<xml>

<cmd>FocusNearAutoOff</cmd>

</xml>

Mode: FocusAutoOff

### focus+ (focusfar)

<xml>

<cmd>FocusFarAutoOff</cmd>

</xml>

Mode: FocusAutoOff

### IrisAutoOn

<xml>

<cmd>IrisAutoOn</cmd>

</xml>

### IrisAutoOff

<xml>

<cmd>IrisAutoOff</cmd>

</xml>

### Iris- (irisclose)

<xml>

<cmd>IrisCloseAutoOff</cmd>

</xml>

Mode: IrisAutoOff

### Iris+ (irisopen)

<xml>

<cmd>IrisOpenAutoOff</cmd>

</xml>

Mode: IrisAutoOff

## Orbit and scan

### Orbit

<xml>

<cmd>Orbit</cmd>

</xml>

### Scan begin

<xml>

<cmd>ScanBegin</cmd>

</xml>

### Scan end

<xml>

<cmd>ScanEnd</cmd>

</xml>

### Scan on

<xml>

<cmd>ScanOn</cmd>

</xml>

### Scan off

<xml>

<cmd>ScanOff</cmd>

</xml>

## Preset command

### Get current preset list

<xml>

<cmd>GetPresetList</cmd>

</xml>

After calling, you can receive EVENT_PTZPRESETINFO event from status callback function, preset number.

### Add Preset

<xml>

<cmd>setpreset</cmd>

<preset>presetnum</preset>

<r>

<b>bhour:bmin</b>

<e>ehour:emin</e>

</r>

</xml>

Note:

- Presetnum from 1 to 66 integer

- <r></r> is a specific time, does not operate PTZ 5 mintes then it will come back the preset, remove it if it not supports this function. <b></b> is start time, <e></e> is end time

- Preset 1 to 8 can be regular homing

### Delete preset

<xml>

<cmd>clearpreset</cmd>

<preset>presetnum</preset>

</xml>

Presetnum from 1 to 66 integer

### Call preset

<xml>

<cmd>callpreset</cmd>

<preset>presetnum</preset>

</xml>

Presetnum from 1 to 66 integer (some ball can call 67 to 255)

### Set for orbit

<xml>

<cmd>SetOrbit</cmd>

<preset>presetnum</preset>

<r>

<b>bhour:bmin</b>

<e>ehour:emin</e>

</r>

</xml>

Note:

- If not operate PTZ 5 mintes then it will orbit automatically

- If there is preset as section 2.1 then it will come back the preset first and then orbit

- Presetnum can be integer from 1 to 8

- <b>00:00</b>is start time <e>23:59</e>is end time

- The setting time can not conflict

Note: time and minute must be expressed by two-digit.

### Set for scan

<xml>

<cmd>SetScan</cmd>

<preset>presetnum</preset>

<r>

<b>bhour:bmin</b>

<e>ehour:emin</e>

</r>

</xml>

Note:

- If not operate PTZ 5 mintes it will scan automatically

- If there is preset as section 2.1 then it will come back the preset first and then scan

- Presetnum can be integer from 1 to 8

- <b>00:00</b>is start time, <e>23:59</e>is end time

- The setting time can not conflict

Note: time and minute must be expressed by 2-digit.

### Delete orbit setting

<xml>

<cmd>DelOrbit</cmd>

<preset>presetnum</preset>

</xml>

Presetnum can be integer from 1 to 8

### Delete scan setting

<xml>

<cmd>DelScan</cmd>

<preset>presetnum</preset>

</xml>

Presetnum can be integer from 1 to 8

## Advanced command

### Black/White

<xml>

<cmd>BLCOff</cmd>

</xml>

### Color

<xml>

<cmd>BLCOn</cmd>

</xml>

### Mirror off

<xml>

<cmd>MirrorOff </cmd>

</xml>

### Mirror on

<xml>

<cmd>MirrorOn</cmd>

</xml>

### Freeze off

<xml>

<cmd>FreezeOff</cmd>

</xml>

### Freeze on

<xml>

<cmd>FreezeOn</cmd>

</xml>

### Digital zoom off

<xml>

<cmd>StepsZOff</cmd>

</xml>

### Digital zoom on

<xml>

<cmd>StepsZOn</cmd>

</xml>

### OSD off

<xml>

<cmd>ScreenOff</cmd>

</xml>

### OSD on

<xml>

<cmd>ScreenOn</cmd>

</xml>

### Illumination off

<xml>

<cmd>Min.SenOff</cmd>

</xml>

### Illumination on

<xml>

<cmd>Min.SenOn</cmd>

</xml>

### Ill off

<xml>

<cmd>ILLOff</cmd>

</xml>

### Ill on

<xml>

<cmd>ILLOn</cmd>

</xml>

### Reset

<xml>

<cmd>CamReset</cmd>

</xml>

### WB auto

<xml>

<cmd>AWBOn</cmd>

</xml>

### WB manual

<xml>

<cmd>AWBOff</cmd>

</xml>

### WB_R

<xml>

<cmd>RWB</cmd>

</xml>

Mode: WB manual

### WB_B

<xml>

<cmd>BWB</cmd>

</xml>

Mode: WB manual

### WB_M

<xml>

<cmd>MWB</cmd>

</xml>

Mode: WB manual

### WB_G

<xml>

<cmd>GWB</cmd>

</xml>

Mode: WB manual

### Initialize

<xml>

<cmd>InitSet</cmd>

</xml>

### 2.4.23 Menu

<xml>

<cmd>Menu</cmd>

</xml>

# 2.5 Transparent Transmission Control Command

<xml>

<cmd>trans</cmd>

<data>control command</data>

</xml>

Control command description:

Take YAAN protocol for example:

A5 01 08 7B 7B 00 A4 up

A5 01 04 7B 7B 00 A0 down

A5 01 02 7B 7B 00 9E left

A5 01 01 7B 7B 00 9D right

A5 01 0A 7B 7B 00 A6 left up

A5 01 09 7B 7B 00 A5 right up

A5 01 06 7B 7B 00 A2 left down

A5 01 05 7B 7B 00 A1 right down

Different protocols have different control command data, you just need to pass in control command data into <data></data> segment in xml.

# Schedule2 Advanced function of auxiliary channel

## 1. Advanced function

There are many other functions of auxiliary channel such as query camera configuration, record, log, upload configuration and firmware, download configuration and log and record, delete, reboot, system parameter, format storage and so on.

## 2. Call auxiliary channel function

Configure IPC, first define a callback function LONG CALLBACK OnAUXResponse(LONG lUser,LONG nType,char *pResponse,void *pUser); then call AJ_NETSDK_IPC_SetAUXResponseCallBack(OnAUXResponse,NULL); it will not return at the same time when query and configure device. lUser is device handle, if nType&0xff000000 is not 0 then means operate success, ntype&0x00ffffff is the command number, different configuration is corresponding to different command

Call AJ_NETSDK_IPC_GetDVRConfig API for reading, dwCommand is macro ( the macro definition characteristic is macro name GET), call AJ_NETSDK_IPC_SetDVRConfig API for configuring devices, dwCommand is macro ( the macro name is SET), call AJ_NETSDK_IPC_SystemControl for operate system.

#define 			CMD_SYSTEM_CONFIG_BASE		200

The following content suppose m_nLastCmdId is the control handle returned from login

## 3. Read and set command

After call AJ_NETSDK_IPC_GetDVRConfig or AJ_NETSDK_IPC_SetDVRConfig  then it will receive a message from OnAuxResponse,if the previous byte of cmdId is cmdId&0xff000000 is 0 means success,others means failed. cmdId&0xffffff is corresponding command.

### Obtain PTZ command (201 command)

#define CMD_GET_SYSTEM_PTZ_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 1) //201

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId,201,0,"",0,0);

Return:

If failed, then it will receive nType&0xff000000!=0 && nType==201 on OnAUXResponse

If success, then it will receive  nType=201 on OnAUXResponse, then content of pResponse as following:

<PTZConfig	// PTZ basic paramter

Protocol="PELCO_D"	// PTZ protocol type, possible value PELCO_P, PELCO_D, MOLYN_P, SANTA_P

ComPort="511"		// address code (1-512)

BaudRate="2400"		// bandrate

DataBits="8"		 	// databits, possible value 5,6,7,8

StopBits="10"			// stopbits, 10 replace 1, 15 replace 1.5, 20 replace 2

Verify="NONE"		 //verify ODD, EVEN, NONE

FlowControl="NONE”   //Data control, NONE, HARDWARE, SOFTWARE

BootAction="0"		//boot action, 0 means no action, 1 means call preset 1

>

<AdvanceConfig>  		// PTZ advanced parameter

<FunctionConfig

FunctionName="ScanBegin" //type

PresetNumber="51"        //preset, from1 to 255 integer

Function="Set"			//operate type, Set or Call

/>

<FunctionConfig

FunctionName="ScanEnd"

PresetNumber="52"

Function="Set"

/>

<FunctionConfig

FunctionName="ScanOn"

PresetNumber="51"

Function="Call"

/>

<FunctionConfig

FunctionName="ScanOff"

PresetNumber="52"

Function="Call"

/>

<FunctionConfig

FunctionName="Orbit"

PresetNumber="53"

Function="Call"

/>

<FunctionConfig

FunctionName="BLCOff"

PresetNumber="64"

Function="Set"

/>

<FunctionConfig

FunctionName="BLCOn"

PresetNumber="64"

Function="Call"

/>

<FunctionConfig

FunctionName="MirrorOff"

PresetNumber="63"

Function="Set"

/>

<FunctionConfig

FunctionName="MirrorOn"

PresetNumber="63"

Function="Call"

/>

<FunctionConfig

FunctionName="StepsZOff"

PresetNumber="58"

Function="Set"

/>

<FunctionConfig

FunctionName="StepsZOn"

PresetNumber="58"

Function="Call"

/>

<FunctionConfig

FunctionName="ScreenOff"

PresetNumber="57"

Function="Set"

/>

<FunctionConfig

FunctionName="ScreenOn"

PresetNumber="57"

Function="Call"

/>

<FunctionConfig

FunctionName="MinSenOff"

PresetNumber="56"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOff"

PresetNumber="62"

Function="Set"

/>

<FunctionConfig

FunctionName="MinSenOn"

PresetNumber="56"

Function="Call"

/>

<FunctionConfig

FunctionName="ILLOff"

PresetNumber="55"

Function="Set"

/>

<FunctionConfig

FunctionName="ILLOn"

PresetNumber="55"

Function="Call"

/>

<FunctionConfig

FunctionName="AWBOff"

PresetNumber="61"

Function="Set"

/>

<FunctionConfig

FunctionName="AWBOn"

PresetNumber="61"

Function="Call"

/>

<FunctionConfig

FunctionName="CamReset"

PresetNumber="54"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOn"

PresetNumber="62"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOn"

PresetNumber="59"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOff"

PresetNumber="59"

Function="Set"

/>

<FunctionConfig

FunctionName="IrisAutoOn"

PresetNumber="60"

Function="Call"

/>

<FunctionConfig

FunctionName="IrisAutoOff"

PresetNumber="60"

Function="Set"

/>

<FunctionConfig

FunctionName="Menu"

PresetNumber="65"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOn"

PresetNumber="89"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOff"

PresetNumber="89"

Function="Set"

/>

<FunctionConfig

FunctionName="PathOn"

PresetNumber="97"

Function="Call"

/>

<FunctionConfig

FunctionName="PathOff"

PresetNumber="96"

Function="Call"

/>

</AdvanceConfig>

</PTZConfig>

PTZ advanced parameter

| 1 | ScanBegin |
| --- | --- |
| 2 | ScanEnd |
| 3 | ScanOn |
| 4 | ScanOff |
| 5 | Orbit |
| 6 | Black/White |
| 7 | Color |
| 8 | MirrorOff |
| 9 | MirrorOn |
| 10 | FreezeOff |
| 11 | FreezeOn |
| 12 | StepsOff |
| 13 | StepsOn |
| 14 | ScreenOff |
| 15 | ScreenOn |
| 16 | LumenOff |
| 17 | LumenOn |
| 18 | IlluminationOff |
| 19 | IlluminationOn |
| 20 | WhiteBalanceManual |
| 21 | WhiteBalanceAuto |
| 22 | CamerReset |
| 23 | AutoFocusAuto |
| 24 | AutoFocusManual |
| 25 | IrisAuto |
| 26 | IrisAuto |
| 27 | Menu |
| 28 | TrackOn |
| 29 | TrackOff |
| 30 | PathOn |
| 31 | PathOff |

### Modify PTZ parameter (225 command)

#define CMD_SET_SYSTEM_PTZ_COMMON	(CMD_SYSTEM_CONFIG_BASE + 25) //225

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId,CMD_SET_SYSTEM_PTZ_COMMON,0,pXml,strlen(pXml));

The content of pXml:

<PTZConfig			 //PTZ basic parameter

Protocol="PELCO_D"	 //PTZ protocol type, possible value PELCO_P, PELCO_D, MOLYN_P, SANTA_P

ComPort="511"		 //address code (1-512)

BaudRate="2400"		 //baudrate

StopBits="10"			// stopbits, 10 replace 1, 15 replace 1.5, 20 replace 2

Verify="NONE"		 //verify

BootAction="0"		//boot action, 0 means no action, 1 means call preset 1> ODD, EVEN, NONE

FlowControl="NONE” //Data control, NONE, HARDWARE, SOFTWARE

<AdvanceConfig />   //can not remove, keep no parameter

</PTZConfig>

### Modify PTZ advanced parameter (226 command)

#define CMD_SET_SYSTEM_PTZ_ADVANCE	(CMD_SYSTEM_CONFIG_BASE + 26) //226

Call method:

AJ_NETSDK_IPC_SetDVRConfig (m_nLastCmdId, CMD_SET_SYSTEM_PTZ_ADVANCE,0,pXml,strlen (pXml));

The content of pXml:

<PTZConfig>

<AdvanceConfig>  		//PTZ advanced parameter

<FunctionConfig

FunctionName="ScanBegin" //type

PresetNumber="51"        //preset from 1 to 255 integer

Function="Set"			//operation type only can be Set orCall

/>

<FunctionConfig

FunctionName="ScanEnd"

PresetNumber="52"

Function="Set"

/>

<FunctionConfig

FunctionName="ScanOn"

PresetNumber="51"

Function="Call"

/>

<FunctionConfig

FunctionName="ScanOff"

PresetNumber="52"

Function="Call"

/>

<FunctionConfig

FunctionName="Orbit"

PresetNumber="53"

Function="Call"

/>

<FunctionConfig

FunctionName="BLCOff"

PresetNumber="64"

Function="Set"

/>

<FunctionConfig

FunctionName="BLCOn"

PresetNumber="64"

Function="Call"

/>

<FunctionConfig

FunctionName="MirrorOff"

PresetNumber="63"

Function="Set"

/>

<FunctionConfig

FunctionName="MirrorOn"

PresetNumber="63"

Function="Call"

/>

<FunctionConfig

FunctionName="StepsZOff"

PresetNumber="58"

Function="Set"

/>

<FunctionConfig

FunctionName="StepsZOn"

PresetNumber="58"

Function="Call"

/>

<FunctionConfig

FunctionName="ScreenOff"

PresetNumber="57"

Function="Set"

/>

<FunctionConfig

FunctionName="ScreenOn"

PresetNumber="57"

Function="Call"

/>

<FunctionConfig

FunctionName="MinSenOff"

PresetNumber="56"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOff"

PresetNumber="62"

Function="Set"

/>

<FunctionConfig

FunctionName="MinSenOn"

PresetNumber="56"

Function="Call"

/>

<FunctionConfig

FunctionName="ILLOff"

PresetNumber="55"

Function="Set"

/>

<FunctionConfig

FunctionName="ILLOn"

PresetNumber="55"

Function="Call"

/>

<FunctionConfig

FunctionName="AWBOff"

PresetNumber="61"

Function="Set"

/>

<FunctionConfig

FunctionName="AWBOn"

PresetNumber="61"

Function="Call"

/>

<FunctionConfig

FunctionName="CamReset"

PresetNumber="54"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOn"

PresetNumber="62"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOn"

PresetNumber="59"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOff"

PresetNumber="59"

Function="Set"

/>

<FunctionConfig

FunctionName="IrisAutoOn"

PresetNumber="60"

Function="Call"

/>

<FunctionConfig

FunctionName="IrisAutoOff"

PresetNumber="60"

Function="Set"

/>

<FunctionConfig

FunctionName="Menu"

PresetNumber="65"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOn"

PresetNumber="89"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOff"

PresetNumber="89"

Function="Set"

/>

<FunctionConfig

FunctionName="PathOn"

PresetNumber="97"

Function="Call"

/>

<FunctionConfig

FunctionName="PathOff"

PresetNumber="96"

Function="Call"

/>

</AdvanceConfig>

</PTZConfig>

PTZ advanced parameter

| 1 | ScanBegin |
| --- | --- |
| 2 | ScanEnd |
| 3 | ScanOn |
| 4 | ScanOff |
| 5 | Orbit |
| 6 | Black/White |
| 7 | Color |
| 8 | MirrorOff |
| 9 | MirrorOn |
| 10 | FreezeOff |
| 11 | FreezeOn |
| 12 | StepsOff |
| 13 | StepsOn |
| 14 | ScreenOff |
| 15 | ScreenOn |
| 16 | LumenOff |
| 17 | LumenOn |
| 18 | IlluminationOff |
| 19 | IlluminationOn |
| 20 | WhiteBalanceManual |
| 21 | WhiteBalanceAuto |
| 22 | CamerReset |
| 23 | AutoFocusAuto |
| 24 | AutoFocusManual |
| 25 | IrisAuto |
| 26 | IrisAuto |
| 27 | Menu |
| 28 | TrackOn |
| 29 | TrackOff |
| 30 | PathOn |
| 31 | PathOff |

### Read configuration about synchronize time (202 command)

#define CMD_GET_SYSTEM_TIME_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 2)

Call method:

AJ_NETSDK_IPC_GetDVRConfig (m_nLastCmdId, CMD_GET_SYSTEM_TIME_CONFIG,0,"",0,0);

Return XML:

<TimeConfig

TimeMode="NTP"			//update mode, NTP is NTP sever synchronize, MANUAL is synchronize manually

TimeZone="1200"         //time zone, formula (12+a)*60, a ranges from -12 to +12

>

<NTPConfig				// work when TimeMode="NTP"

ServerIP="192.168.1.116"	//NTP address

ServerPort="123"			//NTP port

RefreshInterval="60"       //refresh time (mimute)

/>

</TimeConfig>

### Modify method about synchronize time (222 command)

#define CMD_SET_SYSTEM_TIME_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 22)

Call methos:

AJ_NETSDK_IPC_SetDVRConfig (m_nLastCmdId, CMD_SET_SYSTEM_TIME_CONFIG,0,pXml,strlen (pXml));

Return XML:

<TimeConfig

TimeMode="NTP"			//update mode, NTP is NTP sever synchronize, MANUAL is synchronize manually

TimeZone="1200"         //time zone, formula (12+a)*60, a ranges from -12 to +12

>

<NTPConfig				// work when TimeMode="NTP"

ServerIP="192.168.1.116"	//NTP address

ServerPort="123"			//NTP port

RefreshInterval="60"       //refresh time (mimute)

/>

</TimeConfig>

Note: If it is synchronize manually, use command 1005 to modify time.

### Obtain user list (203 command)

#define CMD_GET_SYSTEM_USER_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 3)

Call method:

AJ_NETSDK_IPC_GetDVRConfig (m_nLastCmdId, CMD_GET_SYSTEM_TIME_CONFIG,0,"",0,0);

Return XML:

<UserConfig>

<Account

Username="admin"			//user name

Password="123456"			//password

Group="Administrator"			//group:Administrator, Operator, Viewer

Status="Enable"				//enable user, disable user

/>

<Account

Username="test"

Password="123456"

Group="Administrator"

Status="Enable"

/>

</UserConfig>

### Modify user list (223 command)

#define CMD_SET_SYSTEM_USER_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 23)

Call method:

AJ_NETSDK_IPC_SetDVRConfig (m_nLastCmdId, CMD_SET_SYSTEM_LOG_CONFIG,0,pXml,strlen (pXml));

The content of pXml:

<UserConfig>

<Account

Username="admin"			//user name

Password="123456"			//password

Group="Administrator"			//group:Administrator, Operator, Viewer

Status="Enable"				//enable user, disable user

/>

<Account

Username="test"

Password="123456"

Group="Administrator"

Status="Enable"

/>

</UserConfig>

### Output log (204 command)

#define CMD_GET_SYSTEM_LOG_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 4)

Call method:

AJ_NETSDK_IPC_GetDVRConfig (m_nLastCmdId, CMD_GET_SYSTEM_LOG_CONFIG,0,"",0,0);

Return XML:

<SystemLogConfig

LogLevel="DEBUG+RUN+ERROR+OPERATION+ALARM+STAT+COMMON"

//output log type:debug, run, error, operation, alarm, statistic, common,connect by “+”

MaxDay="30"				// Saving Days from 7 to 30

StoreMedia="FLASH"			// FLASH means store in flash storage

StorePolicy="OVERWRITE"	// FULLSTOP means stop when full;OVERWRITE means overwrite when full

AutoBackup="0"			//0 means do not backup;1 means backup

BackupWay="EMAIL"			//backup type when AutoBackup=1

/>

### Set Output log (224 command)

#define 		CMD_SET_SYSTEM_LOG_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 24)

Call method:

AJ_NETSDK_IPC_SetDVRConfig (m_nLastCmdId, CMD_SET_SYSTEM_LOG_CONFIG,0,pXml,strlen (pXml));

Return XML:

<SystemLogConfig

LogLevel="DEBUG+RUN+ERROR+OPERATION+ALARM+STAT+COMMON"

//output log type:debug, run, error, operation, alarm, statistic, common,connect by “+”

MaxDay="30"				// Saving Days from 7 to 30

StoreMedia="FLASH"			// FLASH means store in flash storage

StorePolicy="OVERWRITE"		// FULLSTOP means stop when full;OVERWRITE means overwrite when full

AutoBackup="0"			//0 means do not backup;1 means backup

BackupWay="EMAIL"			//backup type when AutoBackup=1

/>

### Read default language (205 command)

#define CMD_GET_SYSTEM_MISC_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 5)

Call method:

AJ_NETSDK_IPC_GetDVRConfig (m_nLastCmdId, CMD_GET_SYSTEM_MISC_CONFIG,0,"",0,0);

Return XML:

<MiscConfig

Language="en_us"  //the defualt language are zh_cn,zh_tw,en_us: zh_cn is Chinese; zh_tw is tranditional chinese; en_us is English

/>

### Set default language (225 command)

#define CMD_SET_SYSTEM_MISC_CONFIG	(CMD_SYSTEM_CONFIG_BASE + 25)

Call method:

AJ_NETSDK_IPC_SetDVRConfig (m_nLastCmdId, CMD_SET_SYSTEM_MISC_CONFIG,0,pXml,strlen(pXml));

Return XML:

<MiscConfig

Language="en_us"  //the defualt language are zh_cn,zh_tw,en_us: zh_cn is Chinese; zh_tw is tranditional chinese; en_us is English

/>

### Read system configuration XML (command 200)

#define 		CMD_SYSTEM_CONFIG_BASE	(CMD_SYSTEM_CONFIG_BASE + 0)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SYSTEM_CONFIG_BASE,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<SystemConfig>

<PTZConfig

Protocol="PELCO_P"

ComPort="1"

BaudRate="9600"

DataBits="8"

StopBits="10"

Verify="NONE"

FlowControl="NONE"

BootAction="0"

>

<AdvanceConfig>

<FunctionConfig

FunctionName="ScanBegin"

PresetNumber="51"

Function="Set"

/>

<FunctionConfig

FunctionName="ScanEnd"

PresetNumber="52"

Function="Set"

/>

<FunctionConfig

FunctionName="ScanOn"

PresetNumber="51"

Function="Call"

/>

<FunctionConfig

FunctionName="ScanOff"

PresetNumber="52"

Function="Call"

/>

<FunctionConfig

FunctionName="Orbit"

PresetNumber="53"

Function="Call"

/>

<FunctionConfig

FunctionName="BLCOff"

PresetNumber="64"

Function="Set"

/>

<FunctionConfig

FunctionName="BLCOn"

PresetNumber="64"

Function="Call"

/>

<FunctionConfig

FunctionName="MirrorOff"

PresetNumber="63"

Function="Set"

/>

<FunctionConfig

FunctionName="MirrorOn"

PresetNumber="63"

Function="Call"

/>

<FunctionConfig

FunctionName="StepsZOff"

PresetNumber="58"

Function="Set"

/>

<FunctionConfig

FunctionName="StepsZOn"

PresetNumber="58"

Function="Call"

/>

<FunctionConfig

FunctionName="ScreenOff"

PresetNumber="57"

Function="Set"

/>

<FunctionConfig

FunctionName="ScreenOn"

PresetNumber="57"

Function="Call"

/>

<FunctionConfig

FunctionName="MinSenOff"

PresetNumber="56"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOff"

PresetNumber="62"

Function="Set"

/>

<FunctionConfig

FunctionName="MinSenOn"

PresetNumber="56"

Function="Call"

/>

<FunctionConfig

FunctionName="ILLOff"

PresetNumber="55"

Function="Set"

/>

<FunctionConfig

FunctionName="ILLOn"

PresetNumber="55"

Function="Call"

/>

<FunctionConfig

FunctionName="AWBOff"

PresetNumber="61"

Function="Set"

/>

<FunctionConfig

FunctionName="AWBOn"

PresetNumber="61"

Function="Call"

/>

<FunctionConfig

FunctionName="CamReset"

PresetNumber="54"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOn"

PresetNumber="62"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOn"

PresetNumber="59"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOff"

PresetNumber="59"

Function="Set"

/>

<FunctionConfig

FunctionName="IrisAutoOn"

PresetNumber="60"

Function="Call"

/>

<FunctionConfig

FunctionName="IrisAutoOff"

PresetNumber="60"

Function="Set"

/>

<FunctionConfig

FunctionName="Menu"

PresetNumber="65"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOn"

PresetNumber="89"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOff"

PresetNumber="89"

Function="Set"

/>

<FunctionConfig

FunctionName="PathOn"

PresetNumber="97"

Function="Call"

/>

<FunctionConfig

FunctionName="PathOff"

PresetNumber="96"

Function="Call"

/>

</AdvanceConfig>

</PTZConfig>

<TimeConfig

TimeMode="MANUAL"

TimeZone="1200"

>

<NTPConfig

ServerIP="time.windows.com"

ServerPort="123"

RefreshInterval="60"

/>

</TimeConfig>

<UserConfig>

<Account

Username="admin"

Password="123456"

Group="Administrator"

Status="Enable"

/>

<Account

Username="opr"

Password="123456"

Group="Operator"

Status="Enable"

/>

<Account

Username="view"

Password="123456"

Group="Viewer"

Status="Disable"

/>

</UserConfig>

<SystemLogConfig

LogLevel="DEBUG+RUN+ERROR+OPERATION+ALARM+STAT+COMMON"

MaxDay="30"

StoreMedia="FLASH"

StorePolicy="OVERWRITE"

AutoBackup="0"

BackupWay="EMAIL"

/>

<MiscConfig

Language="en_us"

/>

</SystemConfig>

Among them, the information for each section references the above separating command corresponding to.

### Modify system configuration (command 220)

#define CMD_SET_SYSTEM_CONFIG		(CMD_SYSTEM_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_SYSTEM_LOG_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<SystemConfig>

<PTZConfig

Protocol="PELCO_P"

ComPort="1"

BaudRate="9600"

DataBits="8"

StopBits="10"

Verify="NONE"

FlowControl="NONE"

BootAction="0"

>

<AdvanceConfig>

<FunctionConfig

FunctionName="ScanBegin"

PresetNumber="51"

Function="Set"

/>

<FunctionConfig

FunctionName="ScanEnd"

PresetNumber="52"

Function="Set"

/>

<FunctionConfig

FunctionName="ScanOn"

PresetNumber="51"

Function="Call"

/>

<FunctionConfig

FunctionName="ScanOff"

PresetNumber="52"

Function="Call"

/>

<FunctionConfig

FunctionName="Orbit"

PresetNumber="53"

Function="Call"

/>

<FunctionConfig

FunctionName="BLCOff"

PresetNumber="64"

Function="Set"

/>

<FunctionConfig

FunctionName="BLCOn"

PresetNumber="64"

Function="Call"

/>

<FunctionConfig

FunctionName="MirrorOff"

PresetNumber="63"

Function="Set"

/>

<FunctionConfig

FunctionName="MirrorOn"

PresetNumber="63"

Function="Call"

/>

<FunctionConfig

FunctionName="StepsZOff"

PresetNumber="58"

Function="Set"

/>

<FunctionConfig

FunctionName="StepsZOn"

PresetNumber="58"

Function="Call"

/>

<FunctionConfig

FunctionName="ScreenOff"

PresetNumber="57"

Function="Set"

/>

<FunctionConfig

FunctionName="ScreenOn"

PresetNumber="57"

Function="Call"

/>

<FunctionConfig

FunctionName="MinSenOff"

PresetNumber="56"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOff"

PresetNumber="62"

Function="Set"

/>

<FunctionConfig

FunctionName="MinSenOn"

PresetNumber="56"

Function="Call"

/>

<FunctionConfig

FunctionName="ILLOff"

PresetNumber="55"

Function="Set"

/>

<FunctionConfig

FunctionName="ILLOn"

PresetNumber="55"

Function="Call"

/>

<FunctionConfig

FunctionName="AWBOff"

PresetNumber="61"

Function="Set"

/>

<FunctionConfig

FunctionName="AWBOn"

PresetNumber="61"

Function="Call"

/>

<FunctionConfig

FunctionName="CamReset"

PresetNumber="54"

Function="Set"

/>

<FunctionConfig

FunctionName="FreezeOn"

PresetNumber="62"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOn"

PresetNumber="59"

Function="Call"

/>

<FunctionConfig

FunctionName="FocusAutoOff"

PresetNumber="59"

Function="Set"

/>

<FunctionConfig

FunctionName="IrisAutoOn"

PresetNumber="60"

Function="Call"

/>

<FunctionConfig

FunctionName="IrisAutoOff"

PresetNumber="60"

Function="Set"

/>

<FunctionConfig

FunctionName="Menu"

PresetNumber="65"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOn"

PresetNumber="89"

Function="Call"

/>

<FunctionConfig

FunctionName="TrackOff"

PresetNumber="89"

Function="Set"

/>

<FunctionConfig

FunctionName="PathOn"

PresetNumber="97"

Function="Call"

/>

<FunctionConfig

FunctionName="PathOff"

PresetNumber="96"

Function="Call"

/>

</AdvanceConfig>

</PTZConfig>

<TimeConfig

TimeMode="MANUAL"

TimeZone="1200"

>

<NTPConfig

ServerIP="time.windows.com"

ServerPort="123"

RefreshInterval="60"

/>

</TimeConfig>

<UserConfig>

<Account

Username="admin"

Password="123456"

Group="Administrator"

Status="Enable"

/>

<Account

Username="opr"

Password="123456"

Group="Operator"

Status="Enable"

/>

<Account

Username="view"

Password="123456"

Group="Viewer"

Status="Disable"

/>

</UserConfig>

<SystemLogConfig

LogLevel="DEBUG+RUN+ERROR+OPERATION+ALARM+STAT+COMMON"

MaxDay="30"

StoreMedia="FLASH"

StorePolicy="OVERWRITE"

AutoBackup="0"

BackupWay="EMAIL"

/>

<MiscConfig

Language="en_us"

/>

</SystemConfig>

Among them, the information for each section references the above separating command corresponding to.

## 4. Read and set network configuration information

### Read network configuration (command 200)

#define CMD_NETWORK_CONFIG_BASE			    300

#define 		CMD_GET_NETWORK_CONFIG			(CMD_NETWORK_CONFIG_BASE + 0)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_GET_NETWORK_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<NetworkConfig>

<LANConfig

MacAddress="00:61:98:4c:0f:57"    //MAC address

DHCP="0"					//enable DHCP or not, 0 means no, 1 means yes

IPAddress="192.168.88.153"    //IP address

Netmask="255.255.255.0"      //Netmask

Gateway="192.168.88.1"       //Gateway

DNS1="202.96.128.86"        //DNS1

DNS2="202.96.134.133"       //DNS2

/>

<WIFIConfig

Enable="0"				   //enable WIFI or not, 0 means no, 1 means yes

OperationMode="managed"    //operation mode, current only supports managed

MacAddress=""			  //the MAC address of WIF

ChannelNumber="0"		  //Channel number

Region="CN"               //Region

ESSID=""                  //Ess id

BitRate="33"               //Bit rate

MacMode="9"

// 0 means 802.11b/g mixed

// 1 means 802.11B

// 2 means 802.11A

// 3 means 802.11a/b/g mixed

// 4 means 802.11G,

// 5 means 802.11a/b/g/n mixed

// 6 means 802.11N

// 7 means 802.11g/n mixed

// 8 means 802.11a/n mixed

// 9 means 802.11b/g/n mixed

// 10 means 802.11a/g/n mixed

>

<WirelessEncrypt

Enable="0"			//enable encrypt or not, 0 means no, 1 means yes

EncryptType=""		//Encrypt Type, none means no, wep means WEP encryption, wap means WAP encryption

>

<WEPEncrypt

AuthMode=""			//Auth Mode: OPEN, SHARE

EncryptType=""		//EncryptType:WEP64, WEP128

KeyMode=""			//Key Mode: HEX, ASCII

KeyValue=""			//Key Value

/>

<WPAEncrypt

EncryptType=""		//Encrypt Type: TKIP, AES

AuthMode=""			//Auth Mode: WPA2PSK, WPAPSK

KeyValue=""			//Key Value

/>

</WirelessEncrypt>

<PingWatchConfig

Enable="0"			//enable ping watchdog or not, 0 means no, 1 means yes

PingAddress=""		//PING IP address

Interval="0"			//PING time interval: 3, 5, 10, 20, 30, 40, 50 and 60

MaxFail="0"			//Max timeout number: 10, 20 and 30

/>

</WIFIConfig>

<ADSLConfig

Enable="0"					//enable ADSL or not

Username="szerytrt@163.gd"	//ADSL username

Password="88888888"          //ADSL password

/>

<DDNSConfig

Enable="0"                   //enable DDNS or not

Server="3322.org"             //DDNS server address

Domain="test"                //DDNS domain name

Username="test"              //DDNS admin username

Password="test"              //DDNS admin password

FreshInterval="600"           //fresh interval, the unit is second

DdnsTypeList="3322, 0, http://www.3322.org, 3322.org; DYNDNS, 0, http://www.dyndns.org, dyndns.org;"      // the list of current support servers, the property of this node is read-only. Among them, separated using”;”

/>

<UPNPConfig

Enable="0"		 //enable UPNP or not

/>

<G3Config

Enable="0"	     //enable 3G or not

Type="0"         //3G card type, 0 means can not find, 10 means EVDO, 20 means WCDMA, 30 means TD-SCDMA

DialNumber="*99***1#"	//3G Dial number

APN="&quot;UNINET&quot;"  //3G APN

Username=""                //3G username

Password=""                //3G password

DialOption="0"              //Dial option, 0 means automatic dialing, 1 means SMS or phone activation dialing

CenterNumber=""            //when DialOption="1", means this list is activation telephone number, use “;” to separate and support more than one number

SMSC=""				  //SMS center number

/>

</NetworkConfig>

### Modify network configuration (command 220)

#define 		CMD_SET_NETWORK_CONFIG			(CMD_NETWORK_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<NetworkConfig>

<LANConfig

MacAddress="00:61:98:4c:0f:57"    //MAC address

DHCP="0"					// enable DHCP or not, 0 means no, 1 means yes

IPAddress="192.168.88.153"    //IP address

Netmask="255.255.255.0"      // Net mask

Gateway="192.168.88.1"       // Gateway

DNS1="202.96.128.86"        //DNS1

DNS2="202.96.134.133"       //DNS2

/>

<WIFIConfig

Enable="0"				   // enable WIFI or not, 0 means no, 1 means yes

OperationMode="managed"    // operation mode, current only supports managed

MacAddress=""			  // the MAC address of WIF

ChannelNumber="0"		  // Channel Number

Region="CN"               // Region

ESSID=""                  //

BitRate="33"               // Bit Rate

MacMode="9"

// 0 means 802.11b/g mixed

// 1 means 802.11B

// 2 means 802.11A

// 3 means 802.11a/b/g mixed

// 4 means 802.11G,

// 5 means 802.11a/b/g/n mixed

// 6 means 802.11N

// 7 means 802.11g/n mixed

// 8 means 802.11a/n mixed

// 9 means 802.11b/g/n mixed

// 10 means 802.11a/g/n mixed

>

<WirelessEncrypt

Enable="0"			// enable encrypt or not, 0 means no, 1 means yes

EncryptType=""		// Encrypt Type, NONE means no, wep means WEP encryption, wap means WAP encryption

>

<WEPEncrypt

AuthMode=""			// Auth Mode: OPEN, SHARE

EncryptType=""		// Encrypt Type: WEP64, WEP128

KeyMode=""			// Key Mode: HEX, ASCII

KeyValue=""			// Key Value

/>

<WPAEncrypt

EncryptType=""		// Encrypt Type: TKIP, AES

AuthMode=""			// Auth Mode: WPA2PSK, WPAPSK

KeyValue=""			// Key Value

/>

</WirelessEncrypt>

<PingWatchConfig

Enable="0"			// enable ping watchdog or not, 0 means no, 1 means yes

PingAddress=""		// PING IP address

Interval="0"			// PING time interval: 3, 5, 10, 20, 30, 40, 50 and 60

MaxFail="0"			// Max timeout number: 10, 20 and 30

/>

</WIFIConfig>

<ADSLConfig

Enable="0"					// enable ADSL or not

Username="szerytrt@163.gd"	//ADSL username

Password="88888888"          //ADSL password

/>

<DDNSConfig

Enable="0"                   //enable DDNS or not

Server="3322.org"             //DDNS server address

Domain="test"                //DDNS domain name

Username="test"              //DDNS admin username

Password="test"              //DDNS admin password

FreshInterval="600"           //fresh interval time, the unit is second

DdnsTypeList="3322, 0, http://www.3322.org, 3322.org; DYNDNS, 0, http://www.dyndns.org, dyndns.org;"       // the list of current support servers, the property of this node is read-only. Among them, separated using “;”

/>

<UPNPConfig

Enable="0"		 //enable UPNP or not

/>

<G3Config

Enable="0"	     //enable 3G or not

Type="0"         //3G card type, 0 means can not find, 10 means EVDO, 20 means WCDMA, 30 means TD-SCDMA

DialNumber="*99***1#"	//3G dialing number

APN="&quot;UNINET&quot;"  //3G dialing APN

Username=""                //3G dialing username

Password=""                //3G dialing password

DialOption="0"              // Dial option, 0 means automatic dialing, 1 means SMS or phone activation dialing

CenterNumber=""            // when DialOption="1", means this list is activation telephone number, use “;” separated and support more than one number

SMSC=""				  //SMS center number

/>

</NetworkConfig>

### Read Ethernet configuration (command 305)

#define CMD_GET_NETWORK_LAN_CONFIG   			(CMD_NETWORK_CONFIG_BASE + 5)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_SYSTEM_MISC_CONFIG,0,"",0,0);

Return XML contents as follows:

<LANConfig

MacAddress="00:61:98:4c:0f:57"  //MAC address

DHCP="0"					//enable DHCP or not, 0 means no, 1 means yes

IPAddress="192.168.88.153"    //IP address

Netmask="255.255.255.0"      //Netmask

Gateway="192.168.88.1"       //Geteway

DNS1="202.96.128.86"        //DNS1

DNS2="202.96.134.133"       //DNS2

/>

### Modify Ethernet configuration (command 325)

#define CMD_SET_NETWORK_LAN_CONFIG				(CMD_NETWORK_CONFIG_BASE + 25)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_LAN_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<LANConfig

MacAddress="00:61:98:4c:0f:57"  //MAC address

DHCP="0"					//enable DHCP or not, 0 means no, 1 means yes

IPAddress="192.168.88.153"    //IP address

Netmask="255.255.255.0"      //Netmask

Gateway="192.168.88.1"       //Gateway

DNS1="202.96.128.86"        //DNS1

DNS2="202.96.134.133"       //DNS2

/>

### Read the WIFI configuration (command 306)

#define CMD_GET_NETWORK_WIFI_CONFIG			(CMD_NETWORK_CONFIG_BASE + 6)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_NETWORK_WIFI_CONFIG,0,"",0,0);

Return XML contents as follows:

<WIFIConfig

Enable="0"				   //enable WIFI or not, 0 means no, 1 means yes

OperationMode="managed"    //operation mode, current can only support managed

MacAddress=""			  //WIF MAC address

ChannelNumber="0"		  //Channel number

Region="CN"               //Region

ESSID=""                  //

BitRate="33"               //Bit rate

MacMode="9"

// 0 means 802.11b/g mixed

// 1 means 802.11B

// 2 means 802.11A

// 3 means 802.11a/b/g mixed

// 4 means 802.11G,

// 5 means 802.11a/b/g/n mixed

// 6 means 802.11N

// 7 means 802.11g/n mixed

// 8 means 802.11a/n mixed

// 9 means 802.11b/g/n mixed

// 10 means 802.11a/g/n mixed

>

<WirelessEncrypt

Enable="0"			// enable encrypt or not, 0 means no, 1 means yes

EncryptType=""	// Encrypt Type, NONE means no, wep means WEP encryption, wap means WAP encryption

>

<WEPEncrypt

AuthMode=""			// Auth Mode: OPEN, SHARE

EncryptType=""		// Encrypt Type: WEP64, WEP128

KeyMode=""			// Key Mode: HEX, ASCII

KeyValue=""			// Key Value

/>

<WPAEncrypt

EncryptType=""		// Encrypt Type: TKIP, AES

AuthMode=""			// Auth Mode: WPA2PSK, WPAPSK

KeyValue=""			// Key Value

/>

</WirelessEncrypt>

<PingWatchConfig

Enable="0"			// enable ping watchdog or not, 0 means no, 1 means yes

PingAddress=""		// PING IP address

Interval="0"			// PING time interval: 3, 5, 10, 20, 30, 40, 50 and 60

MaxFail="0"			// Max timeout number: 10, 20 and 30

/>

</WIFIConfig>

### Modify the WIFI configuration (command 326)

#define CMD_SET_NETWORK_WIFI_CONFIG			(CMD_NETWORK_CONFIG_BASE + 26)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_WIFI_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<WIFIConfig

Enable="0"				   // enable WIFI or not, 0 means no, 1 means yes

OperationMode="managed"    // operation mode, current only supports managed

MacAddress=""			  // the MAC address of WIF

ChannelNumber="0"		  // Channel Number

Region="CN"               //Region

ESSID=""                  //

BitRate="33"               //Bit rate

MacMode="9"

// 0 means 802.11b/g mixed

// 1 means 802.11B

// 2 means 802.11A

// 3 means 802.11a/b/g mixed

// 4 means 802.11G,

// 5 means 802.11a/b/g/n mixed

// 6 means 802.11N

// 7 means 802.11g/n mixed

// 8 means 802.11a/n mixed

// 9 means 802.11b/g/n mixed

// 10 means 802.11a/g/n mixed

>

<WirelessEncrypt

Enable="0"			// enable encrypt or not, 0 means no, 1 means yes

EncryptType=""	/ Encrypt Type, NONE means no, wep means WEP encryption, wap means WAP encryption

>

<WEPEncrypt

AuthMode=""			// Auth Mode: OPEN, SHARE

EncryptType=""		// Encrypt Type: WEP64, WEP128

KeyMode=""			// Key Mode: HEX, ASCII

KeyValue=""			// Key Value

/>

<WPAEncrypt

EncryptType=""		// Encrypt Type: TKIP, AES

AuthMode=""			// Auth Mode: WPA2PSK, WPAPSK

KeyValue=""			// Key Value

/>

</WirelessEncrypt>

<PingWatchConfig

Enable="0"			// enable ping watchdog or not, 0 means no, 1 means yes

PingAddress=""		// PING IP address

Interval="0"			// PING time interval: 3, 5, 10, 20, 30, 40, 50 and 60

MaxFail="0"			// Max timeout number: 10, 20 and 30

/>

</WIFIConfig>

### Read the ADSL configuration (command 310)

#define CMD_GET_NETWORK_ADSL_CONFIG			(CMD_NETWORK_CONFIG_BASE + 10)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_NETWORK_ADSL_CONFIG,0,"",0,0);

Return XML contents as follows:

<ADSLConfig

Enable="0"					//enable ADSL or not

Username="szerytrt@163.gd"	//ADSL username

Password="88888888"          //ADSL password

/>

### Modify the ADSL configuration (command 330)

#define CMD_SET_NETWORK_ADSL_CONFIG			(CMD_NETWORK_CONFIG_BASE + 30)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_ADSL_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<ADSLConfig

Enable="0"					//enable ADSL or not

Username="szerytrt@163.gd"	//ADSL username

Password="88888888"          //ADSL password

### Read the DDNS configuration (command 311)

#define CMD_GET_NETWORK_DDNS_CONFIG			(CMD_NETWORK_CONFIG_BASE + 11)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_NETWORK_DDNS_CONFIG,0,"",0,0);

Return XML contents as follow:

<DDNSConfig

Enable="0"                   //enable DDNS or not

Server="3322.org"             //DDNS server address

Domain="test"                //DDNS domain name

Username="test"              //DDNS admin username

Password="test"              //DDNS admin password

FreshInterval="600"           //fresh interval time, the unit is second

DdnsTypeList="3322,0,http://www.3322.org,3322.org;DYNDNS,0,http://www.dyndns.org,dyndns.org;"     // the list of current support servers, the property of this node is read-only. Among them, separated using “;”

/>

### Modify the DDNS configuration (command 331)

#define CMD_SET_NETWORK_DDNS_CONFIG			(CMD_NETWORK_CONFIG_BASE + 31)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_DDNS_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<DDNSConfig

Enable="0"                   //enable DDNS or not

Server="3322.org"             //DDNS server address

Domain="test"                //DDNS domain name

Username="test"              //DDNS admin username

Password="test"              //DDNS admin password

FreshInterval="600"           // fresh interval time, the unit is second

DdnsTypeList="3322, 0, http://www.3322.org, 3322.org; DYNDNS, 0, http://www.dyndns.org, dyndns.org;"   // the list of current support servers, the property of this node is read-only. Among them, separated using”;”

/>

### Read the UPNP configuration (command 312)

#define CMD_GET_NETWORK_UPNP_CONFIG			(CMD_NETWORK_CONFIG_BASE + 12)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_NETWORK_UPNP_CONFIG,0,"",0,0);

Return XML contents as follow:

<UPNPConfig

Enable="0"		 //enable UPNP or not, 0 means no, 1 means yes

/>

### Modify UPNP configuration (command 332)

#define CMD_SET_NETWORK_UPNP_CONFIG			(CMD_NETWORK_CONFIG_BASE + 32)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_UPNP_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<UPNPConfig

Enable="0"		//enable UPNP or not, 0 means no, 1 means yes

/>

### Read the 3G configuration (command 313)

#define CMD_GET_NETWORK_G3_CONFIG			(CMD_NETWORK_CONFIG_BASE + 13)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_NETWORK_G3_CONFIG,0,"",0,0);

Return XML contents as follows:

<G3Config

Enable="0"	     // enable 3G or not

Type="0"         //3G card type, 0 means can not find, 10 means EVDO, 20 means WCDMA, 30 means TD-SCDMA

DialNumber="*99***1#"	//3G dialing number

APN="&quot;UNINET&quot;"  //3G dialing APN

Username=""                //3G dialing username

Password=""                //3G dialing password

DialOption="0"              // Dial option, 0 means automatic dialing, 1 means SMS or phone activation dialing

CenterNumber=""            // when DialOption="1", means this list is activation telephone number, use “;” to separate and support more than one number

SMSC=""				  // SMS center number

/>

### Modify the 3G configuration (command 333)

#define CMD_SET_NETWORK_DDNS_CONFIG			(CMD_NETWORK_CONFIG_BASE + 31)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_NETWORK_DDNS_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<G3Config

Enable="0"	     // enable 3G or not

Type="0"         //3G card type, 0 means can not find, 10 means EVDO, 20 means WCDMA, 30 means TD-SCDMA

DialNumber="*99***1#"	//3G dialing number

APN="&quot; UNINET&quot;” //3G dialing APN

Username=""                //3G dialing username

Password=""                //3G dialing password

DialOption="0"              // Dial option, 0 means automatic dialing, 1 means SMS or phone activation dialing

CenterNumber=""            // when DialOption="1", means this list is activation telephone number, use “;” to separate and support more than one number

SMSC=""				  // SMS center number

/>

## 5. FTP & SMTP related configuration

Mainly the configuration of FTP account, SMTP account and so on. Including micro definitions as follows:

#define CMD_SERVER_CONFIG_BASE			400

#define CMD_GET_SERVER_CONFIG			    (CMD_SERVER_CONFIG_BASE + 0)

#define CMD_GET_SERVER_FTP_CONFIG		    (CMD_SERVER_CONFIG_BASE + 1)

#define CMD_GET_SERVER_SMTP_CONFIG		(CMD_SERVER_CONFIG_BASE + 2)

#define CMD_SET_SERVER_CONFIG			    (CMD_SERVER_CONFIG_BASE + 20)

#define CMD_SET_SERVER_FTP_CONFIG		    (CMD_SERVER_CONFIG_BASE + 21)

#define CMD_SET_SERVER_SMTP_CONFIG		(CMD_SERVER_CONFIG_BASE + 22)

#define CMD_SERVER_CONFIG_LAST		    	(CMD_SERVER_CONFIG_BASE + 22)

### Read FTP & SMTP related configuration (command 400)

#define CMD_SERVER_CONFIG_BASE			400

#define CMD_GET_SERVER_CONFIG			    (CMD_SERVER_CONFIG_BASE + 0)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_SERVER_CONFIG,0,"",0,0);

Return XML contents as follows:

<ServerConfig>

<FTPList>

<FTPConfig

Index="0" 			 //0 means the FTP configuration for uploading alarm

ServerIP="192.168.88.172"  		//ftp server IP

ServerPort="21"            		//ftp server port

Username="vss_55008866"    		//ftp username

Password="755008866b0a62c24"  	//ftp password

FilePath="1"   				//ftp server path

FileSize="0"  	 			//this property is temporarity meaningless

/>

<FTPConfig

Index="1"				//1 means the FTP configuration for backuping log

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

<FTPConfig

Index="2"			//2 means the FTP configuration for backuping configure

ServerIP="192.168.1.1"

ServerPort="21"

Username="user1"

Password="pas1"

FilePath="filepath1"

FileSize="1000"

/>

<FTPConfig

Index="3"			//3 means the FTP configuration for updating

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

</FTPList>

<SMTPList

ServerIP="192.168.88.8"		//SMTP server address

ServerPort="25"			//SMTP server port

Auth="1"					//need authentication or not, 0 means no, 1 means yes

Username="root@192.168.88.8"	//authentication account

Password="123456"				//authentication password

FromEmail="root@192.168.88.8"	//show account

>

<SMTPConfig

Index="0"			//0 means alarm upload, 1 means log backup, 2 means configuration backup

ToEmail="root@192.168.88.8"		//receive mail address

CcEmail="root@192.168.88.8"      //CC mail address

Subject="hello"				//mail subject

/>

<SMTPConfig

Index="1"

ToEmail="root@192.168.88.8"

CcEmail=""

Subject="alarm report"

/>

<SMTPConfig

Index="2"

ToEmail="root@192.168.88.8"

CcEmail="root@192.168.88.8"

Subject="hello"

/>

</SMTPList>

</ServerConfig>

### Modify FTP & SMTP related configuration (command 420)

#define CMD_SET_SERVER_CONFIG			(CMD_SERVER_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_SERVER_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<ServerConfig>

<FTPList>

<FTPConfig

Index="0" 			 //0 means the FTP configuration for uploading alarm ServerIP="192.168.88.172"  		//ftp server address

ServerPort="21"            		//ftp server port

Username="vss_55008866"    		//ftp username

Password="755008866b0a62c24"  	//ftp password

FilePath="1"   				//ftp server path

FileSize="0"  	 			//this property is temporarily meaningless

/>

<FTPConfig

Index="1"				//1 means the FTP configuration for backuping log

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

<FTPConfig

Index="2"			//2 means the FTP configuration for backuping configure

ServerIP="192.168.1.1"

ServerPort="21"

Username="user1"

Password="pas1"

FilePath="filepath1"

FileSize="1000"

/>

<FTPConfig

Index="3"			//3 means the FTP configuration for updating

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

</FTPList>

<SMTPList

ServerIP="192.168.88.8"		//SMTP server IP

ServerPort="25"			//SMTP server port

Auth="1"					// need authentication or not, 0 means no, 1 means yes

Username="root@192.168.88.8"	//authentication user account

Password="123456"				//authentication password

FromEmail="root@192.168.88.8"	//show account

>

<SMTPConfig

Index="0"			//0 means alarm upload, 1 means log backup, 2 means configuration backup

ToEmail="root@192.168.88.8"		//receive mail address

CcEmail="root@192.168.88.8"      //CC mail address

Subject="hello"				//mail subject

/>

<SMTPConfig

Index="1"

ToEmail="root@192.168.88.8"

CcEmail=""

Subject="alarm report"

/>

<SMTPConfig

Index="2"

ToEmail="root@192.168.88.8"

CcEmail="root@192.168.88.8"

Subject="hello"

/>

</SMTPList>

</ServerConfig>

### Read the FTP configuration (command 401)

#define CMD_GET_SERVER_FTP_CONFIG		(CMD_SERVER_CONFIG_BASE + 1)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_SERVER_FTP_CONFIG,0,"",0,0);

Return XML content as follows:

<FTPList>

<FTPConfig

Index="0" 			 //0 means the FTP configuration for uploading alarm

ServerIP="192.168.88.172"  		// ftp server IP

ServerPort="21"            		// ftp server port

Username="vss_55008866"    		// ftp username

Password="755008866b0a62c24"  	//ftp password

FilePath="1"   				//ftp server path

FileSize="0"  	 			//this property is temporarity meaningless

/>

<FTPConfig

Index="1"				//1 means the FTP configuration for backuping log

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

<FTPConfig

Index="2"			//2 means the FTP configuration for backuping configure

ServerIP="192.168.1.1"

ServerPort="21"

Username="user1"

Password="pas1"

FilePath="filepath1"

FileSize="1000"

/>

<FTPConfig

Index="3"			//3 means the FTP configuration for updating

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

</FTPList>

### Modify the FTP configuration (command 421)

#define CMD_SET_SERVER_FTP_CONFIG		(CMD_SERVER_CONFIG_BASE + 21)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_SERVER_FTP_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding content are as follows:

<FTPList>

<FTPConfig

Index="0" 			 //0 means the FTP configuration for uploading alarm

ServerIP="192.168.88.172"  		//ftp server IP

ServerPort="21"            		//ftp servee port

Username="vss_55008866"    		//ftp username

Password="755008866b0a62c24"  	//ftp password

FilePath="1"   				//ftp server path

FileSize="0"  	 			//this property is temporarily meaningless

/>

<FTPConfig

Index="1"				//1 means the FTP configuration for backuping log

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

<FTPConfig

Index="2"			//2 means the FTP configuration for backuping configure

ServerIP="192.168.1.1"

ServerPort="21"

Username="user1"

Password="pas1"

FilePath="filepath1"

FileSize="1000"

/>

<FTPConfig

Index="3"			//3 means the FTP configuration for updating

ServerIP="192.168.88.138"

ServerPort="21"

Username="ling"

Password="123456"

FilePath="alarm"

FileSize="1000"

/>

</FTPList>

### Read the SMTP configuration (command 402)

#define CMD_GET_SERVER_SMTP_CONFIG		(CMD_SERVER_CONFIG_BASE + 2)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_SERVER_SMTP_CONFIG,0,"",0,0);

Return XML content as follows:

<SMTPList

ServerIP="192.168.88.8"		//SMTP server IP

ServerPort="25"			//SMTP server port

Auth="1"					// need authentication or not, 0 means no, 1 means yes

Username="root@192.168.88.8"	//authentication user account

Password="123456"				//authentication password

FromEmail="root@192.168.88.8"	//show account

>

<SMTPConfig

Index="0"			//0 means alarm upload, 1 means log backup, 2 means configuration backup

ToEmail="root@192.168.88.8"		//receive mail address

CcEmail="root@192.168.88.8"      //CC mail address

Subject="hello"				//mail subject

/>

<SMTPConfig

Index="1"

ToEmail="root@192.168.88.8"

CcEmail=""

Subject="alarm report"

/>

<SMTPConfig

Index="2"

ToEmail="root@192.168.88.8"

CcEmail="root@192.168.88.8"

Subject="hello"

/>

</SMTPList>

### Modify the SMTP configuration (command 422)

#define CMD_SET_SERVER_SMTP_CONFIG		(CMD_SERVER_CONFIG_BASE + 22)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_SERVER_SMTP_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding content are as follows:

<SMTPList

ServerIP="192.168.88.8"		//SMTP server IP

ServerPort="25"			//SMTP server port

Auth="1"					// need authentication or not, 0 means no, 1 means yes

Username="root@192.168.88.8"	//authentication user account

Password="123456"				//authentication password

FromEmail="root@192.168.88.8"	//show account

>

<SMTPConfig

Index="0"			//0 means alarm upload, 1 means log backup, 2 means configuration backup

ToEmail="root@192.168.88.8"		//receive mail address

CcEmail="root@192.168.88.8"      //CC mail address

Subject="hello"				//mail subject

/>

<SMTPConfig

Index="1"

ToEmail="root@192.168.88.8"

CcEmail=""

Subject="alarm report"

/>

<SMTPConfig

Index="2"

ToEmail="root@192.168.88.8"

CcEmail="root@192.168.88.8"

Subject="hello"

/>

</SMTPList>

## 6. Media parameter setup

Mainly include audio and video capture, audio and video encode, OSD setup and so on, use micro definition as follows:

//media config cmd

#define CMD_MEDIA_CONFIG_BASE			500

#define CMD_GET_MEDIA_CONFIG			    (CMD_MEDIA_CONFIG_BASE + 0)

#define CMD_GET_MEDIA_VIDEO_CONFIG		(CMD_MEDIA_CONFIG_BASE + 1)

#define CMD_GET_MEDIA_AUDIO_CONFIG		(CMD_MEDIA_CONFIG_BASE + 2)

#define CMD_SET_MEDIA_CONFIG			    (CMD_MEDIA_CONFIG_BASE + 20)

#define CMD_SET_MEDIA_VIDEO_ENCODE		(CMD_MEDIA_CONFIG_BASE + 23)

#define CMD_SET_MEDIA_VIDEO_CAPTURE	(CMD_MEDIA_CONFIG_BASE + 24)

#define CMD_SET_MEDIA_VIDEO_OSD			(CMD_MEDIA_CONFIG_BASE + 25)

#define CMD_SET_MEDIA_VIDEO_JPEG		    (CMD_MEDIA_CONFIG_BASE + 26)

#define CMD_SET_MEDIA_AUDIO_CAPTURE	(CMD_MEDIA_CONFIG_BASE + 27)

#define CMD_SET_MEDIA_AUDIO_ENCODE		(CMD_MEDIA_CONFIG_BASE + 28)

#define CMD_SET_PRIVACY_CONFIG           (CMD_MEDIA_CONFIG_BASE + 29)

#define CMD_MEDIA_CONFIG_LAST			(CMD_MEDIA_CONFIG_BASE + 29)

### Read media configuration parameter (command 500)

#define CMD_MEDIA_CONFIG_BASE			500

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_MEDIA_CONFIG_BASE,0,"",0,0);

Return XML content as follows:

<MediaConfig>

<Video>

<Capture			//video capture

Brightness="128"	//Brightness 0-255

Contrast="128"	//Contrast 0-255

Saturation="128"	//Saturation 0-255

Sharpness="128"   //Sharpness 0-255

DayNight="1"		//DayNight or not, 1 is day mode, 0 is night mode

WhiteBalance="0"	//WhiteBalance,0 means auto, 1 means in door, 2 means out door

BackLight="128"	//BackLight 0-255

Switch2A="1"		//enable Switch2A or not, 1 means yes, 0 means no

BinningSkip="0"	// BinningSkip, 0 means Binning, 1 means Skip

TVSystem="1"		//TVSystem, 0 means 60HZ, 1 means 50HZ

HFlip="0"			//enable HFlip or not, 1 means yes, 0 means no

VFlip="0"			//enable VFlip or not, 1 means yes, 0 means no

TNF="0"			    //reserved

SNF="0"            //reserved

VSTAB="0"         //reserved

LDC="0"            //reserved

FD="0"             //reserved

IrcutMode="0"		//IrcutMode, 0 means auto, 1 means day-night mode, 2 means passive, 3 means manual

IrcutSensitivity="50"	//when ircutmode is auto, this value indicates sensitivity

IrcutNightStartTime="18:00:00"	//night start-time, when the ending-time is smaller than starting-time, express acrossed one day, such as this setting means that 6:00 in the night to 8:00 in the morning the next day

IrcutNightEndTime="08:00:00"	//night end-time

/>

<Encode>

<EncodeConfig			//video encode

Stream="1"			//1 means main stream, 2 means sub stream

Resolution="960P"		//Resolution

EncodeFormat="H264"	//EncodeFormat

BitRateControl="CBR"	//BitRateContro, support CBR, VBR, CVBR

Initquant="100"		//I Frame Interval

BitRate="4000"		//BitRate, different device has different value

FrameRate="25"		//FrameRate, support 1-30

/>

<EncodeConfig

Stream="2"

Resolution="480X270"

EncodeFormat="H264"

BitRateControl="CBR"

Initquant="100"

BitRate="300"

FrameRate="25"

/>

<AdvanceEncodeConfig      //video encode advanced parameters setting

EncodeProfile="0"          //Profile setting: 0 means disable base profile encode, 1 means enable base profile encode

WDRMode="0"            //reserved

WDRStartTime="00:00:00"   // reserved

WDREndTime="00:00:00"   // reserved

/>

</Encode>

<JpegConfig

Enable="1"	//enable picture capture or not

Stream="0"	//the stream of capture, 0 means main stream, 1 means sub stream

Quality="80"	//picture quality 20-100

Framerate="2" //capture speed, how many pictured need to be captured every second, you can choose one or two

/>

<Overlay

Enable="1"

Transparency="0"

>

<TimeOverlay

PosX="0"	//0 means left, 1 means right

PosY="0" //0 means top, 1 means bottom

Format="yyyy-mm-dd hh:mm:ss" //time show format, the supportting format are yyyy-mm-dd hh:mm:ss, yyyy/mm/dd hh:mm:ss, yy-mm-dd hh:mm:ss, yy/mm/dd hh:mm:ss, hh:mm:ss dd/mm/yyyy, hh:mm:ss dd-mm-yyyy, hh:mm:ss mm/dd/yyyy, hh:mm:ss mm-dd-yyyy

/>

<TitleOverlay

PosX="0" //0 means left, 1 means right

PosY="1" //0 menas top, 1 means bottom

Title="5433384647"		//Characters are converted to ASCII code 16 hexadecimal form

/>

</Overlay>

<Mask>// privacy mask

<MainStream // main stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

<SubStream // sub stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

</Mask>

</Video>

<Audio>

<Capture

Volume="25"	//capture volume, 0-100

/>

<Encode

Enable="1"			//enable video encode or not

SampleRate="16000"		//SampleRate

EncodeType="AAC"		//EncodeType, support AAC, G711

BitRate="16000"		//BitRate

/>

</Audio>

</MediaConfig>

### Read video parameter configuration (command 501)

#define CMD_GET_MEDIA_VIDEO_CONFIG		(CMD_MEDIA_CONFIG_BASE + 1)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_MEDIA_VIDEO_CONFIG,0,"",0,0);

Reture XML contents as follows:

<Video>

<Capture			// video capture

Brightness="128"	// Brightness 0-255

Contrast="128"	// Contrast 0-255

Saturation="128"	// Saturation 0-255

Sharpness="128"   // Sharpness 0-255

DayNight="1"		// DayNight or not, 1 is day mode, 0 is night mode

WhiteBalance="0"	// WhiteBalance,0 means auto, 1 means in door, 2 means out door

BackLight="128"	// BackLight 0-255

Switch2A="1"		// enable Switch2A or not, 1 means yes, 0 means no

BinningSkip="0"	// BinningSkip, 0 means Binning, 1 means Skip

TVSystem="1"		// TVSystem, 0 means 60HZ, 1 means 50HZ

HFlip="0"			// enable HFlip or not, 1 means yes, 0 means no

VFlip="0"			// enable VFlip or not, 1 means yes, 0 means no

TNF="0"			    //reserved

SNF="0"            //reserved

VSTAB="0"         //reserved

LDC="0"           //reserved

FD="0"            //reserved

IrcutMode="0"		//IrcutMode, 0 means auto, 1 means day-night mode, 2 means passive, 3 means manual

IrcutSensitivity="50"	//when ircutmode is auto, this value indicates sensitivity

IrcutNightStartTime="18:00:00"	//night start-time, when the ending-time is smaller than starting-time, express acrossed one day, such as this setting means that 6:00 in the night to 8:00 in the morning the next day

IrcutNightEndTime="08:00:00"	// night end-time

/>

<Encode>

<EncodeConfig			// video encode

Stream="1"			// 1 means main stream, 2 means sub stream

Resolution="960P"		// Resolution

EncodeFormat="H264"	// EncodeFormat

BitRateControl="CBR"	// BitRateControl, support CBR, VBR, CVBR

Initquant="100"		// I Frame Interval

BitRate="4000"		// BitRate, different device has different value

FrameRate="25"		// FrameRate, support 1-30

/>

<EncodeConfig

Stream="2"

Resolution="480X270"

EncodeFormat="H264"

BitRateControl="CBR"

Initquant="100"

BitRate="300"

FrameRate="25"

/>

<AdvanceEncodeConfig      //video encode advanced parameters setting

EncodeProfile="0"          //Profile setting: 0 means disable base profile encode, 1 means enable base profile encode

WDRMode="0"            //reserved

WDRStartTime="00:00:00"   // reserved

WDREndTime="00:00:00"   // reserved

/>

</Encode>

<JpegConfig

Enable="1"	// enable picture capture or not

Stream="0"	// the stream of capture, 0 means main stream, 1 means sub stream

Quality="80"	// picture quality 20-100

Framerate="2" // capture speed, how many pictures need to be captured every second, you can choose one or two

/>

<Overlay

Enable="1"

Transparency="0"

>

<TimeOverlay

PosX="0"	//0 means left, 1 means right

PosY="0" //0 means top, 1 means bottom

Format="yyyy-mm-dd hh:mm:ss" // time show format, the supportting format are yyyy-mm-dd hh:mm:ss, yyyy/mm/dd hh:mm:ss, yy-mm-dd hh:mm:ss, yy/mm/dd hh:mm:ss, hh:mm:ss dd/mm/yyyy, hh:mm:ss dd-mm-yyyy, hh:mm:ss mm/dd/yyyy, hh:mm:ss mm-dd-yyyy

/>

<TitleOverlay

PosX="0" //0 means left, 1 means right

PosY="1" //0 menas top, 1 means bottom

Title="5433384647"		// Characters are converted to ASCII code 16 hexadecimal form

/>

</Overlay>

<Mask>// privacy mask

<MainStream // main stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

<SubStream // sub stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

</Mask>

</Video>

### Read audio parameter configuration (command 502)

#define CMD_GET_MEDIA_AUDIO_CONFIG		(CMD_MEDIA_CONFIG_BASE + 2)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_MEDIA_AUDIO_CONFIG,0,"",0,0);

Return XML contents as follows:

<Audio>

<Capture

Volume="25"	// capture volume, 0-100

/>

<Encode

Enable="1"			// enable video encode or not

SampleRate="16000"		// Samplerate

EncodeType="AAC"		// EncodeType, support AAC, G711

BitRate="16000"		// BitRate

/>

</Audio>

### Modify media parameter configuration (command 520)

#define CMD_SET_MEDIA_CONFIG			(CMD_MEDIA_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<MediaConfig>

<Video>

<Capture			//video capture

Brightness="128"	//Brightness 0-255

Contrast="128"	//Contrast 0-255

Saturation="128"	//Saturation 0-255

Sharpness="128"   //Sharpness 0-255

DayNight="1"		//DayNight or not, 1 is day mode, 0 is night mode

WhiteBalance="0"	//WhiteBalance,0 means auto, 1 means in door, 2 means out door

BackLight="128"	//BackLight 0-255

Switch2A="1"		//enable Switch2A or not, 1 means yes, 0 means no

BinningSkip="0"	//BinningSkip, 0 means Binning, 1 means Skip

TVSystem="1"		//TVSystem, 0 means 60HZ, 1 means 50HZ

HFlip="0"			//enable HFlip or not, 1 means yes, 0 means no

VFlip="0"			//enable VFlip or not, 1 means yes, 0 means no

TNF="0"			    //reserved

SNF="0"            //reserved

VSTAB="0"         //reserved

LDC="0"           //reserved

FD="0"             //reserved

IrcutMode="0"			//IrcutMode, 0 means auto, 1 means day-night mode, 2 means passive, 3 means manual

IrcutSensitivity="50"	//when ircutmode is auto, this value indicates sensitivity

IrcutNightStartTime="18:00:00"	//night start-time, when the ending-time is smaller than starting-time, express acrossed one day, such as this setting means that 6:00 in the night to 8:00 in the morning the next day

IrcutNightEndTime="08:00:00"	//night end-time

/>

<Encode>

<EncodeConfig			//video encode

Stream="1"			//1 means main stream, 2 means sub stream

Resolution="960P"		//Resolution

EncodeFormat="H264"	//EncodeFormat

BitRateControl="CBR"	//BitRateControl, support CBR, VBR, CVBR

Initquant="100"		//I Frame Interval

BitRate="4000"		//BitRate, different device has different value

FrameRate="25"		//FrameRate, support 1-30

/>

<EncodeConfig

Stream="2"

Resolution="480X270"

EncodeFormat="H264"

BitRateControl="CBR"

Initquant="100"

BitRate="300"

FrameRate="25"

/>

<AdvanceEncodeConfig      //video encode advanced parameters setting

EncodeProfile="0"          //Profile setting: 0 means disable base profile encode, 1 means enable base profile encode

WDRMode="0"            //reserved

WDRStartTime="00:00:00"   // reserved

WDREndTime="00:00:00"   // reserved

/>

</Encode>

<JpegConfig

Enable="1"	//enable picture capture or not

Stream="0"	//the stream of capture, 0 means main stream, 1 means sub stream

Quality="80"	//picture quality 20-100

Framerate="2" //capture speed, how many pictures need to be captured every second, you can choose one or two

/>

<Overlay

Enable="1"

Transparency="0"

>

<TimeOverlay

PosX="0"	//0 means left, 1 means right

PosY="0" //0 means top, 1 means bottom

Format="yyyy-mm-dd hh:mm:ss" // time show format, the supportting format are yyyy-mm-dd hh:mm:ss, yyyy/mm/dd hh:mm:ss, yy-mm-dd hh:mm:ss, yy/mm/dd hh:mm:ss, hh:mm:ss dd/mm/yyyy, hh:mm:ss dd-mm-yyyy, hh:mm:ss mm/dd/yyyy, hh:mm:ss mm-dd-yyyy

/>

<TitleOverlay

PosX="0" //0 means left, 1 means right

PosY="1" //0 menas top, 1 means bottom

Title="5433384647"		//Characters are converted to ASCII code 16 hexadecimal form

/>

</Overlay>

<Mask>// privacy mask

<MainStream // main stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

<SubStream // sub stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

</Mask>

</Video>

<Audio>

<Capture

Volume="25"	//capture volume, 0-100

/>

<Encode

Enable="1"			//enable video encode or not

SampleRate="16000"	//SampleRate

EncodeType="AAC"	//EncodeType, support AAC, G711

BitRate="16000"		//BitRate

/>

</Audio>

</MediaConfig>

Note: Use command 520 to modify media configuration at one time need to reboot device to take effect, if want to take effect immediately, please use the following separated command.

### Modify video parameter configuration (command 523)

#define CMD_SET_MEDIA_VIDEO_ENCODE		(CMD_MEDIA_CONFIG_BASE + 23)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_VIDEO_ENCODE,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<Encode>

<EncodeConfig			// video encode

Stream="1"			// 1 means main stream, 2 means sub stream

Resolution="960P"		// Resolution

EncodeFormat="H264"	// EncodeFormat

BitRateControl="CBR"	// BitRateControl, support CBR, VBR, CVBR

Initquant="100"		// I Frame Interval

BitRate="4000"		// Bitrate, different device has different value

FrameRate="25"		// Framerate, support 1-30

/>

<EncodeConfig

Stream="2"

Resolution="480X270"

EncodeFormat="H264"

BitRateControl="CBR"

Initquant="100"

BitRate="300"

FrameRate="25"

/>

<AdvanceEncodeConfig      //video encode advanced parameters setting

EncodeProfile="0"          //Profile setting: 0 means disable base profile encode, 1 means enable base profile encode

WDRMode="0"            //reserved

WDRStartTime="00:00:00"   // reserved

WDREndTime="00:00:00"   // reserved

/>

</Encode>

### Modify video parameter configuration (command 524)

#define CMD_SET_MEDIA_VIDEO_CAPTURE		(CMD_MEDIA_CONFIG_BASE + 24)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_VIDEO_CAPTURE,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<Capture			// video capture

Brightness="128"	// Brightness 0-255

Contrast="128"	// Contrast 0-255

Saturation="128"	// Saturation 0-255

Sharpness="128"   // Sharpness 0-255

DayNight="1"		// DayNight or not, 1 is day mode, 0 is night mode

WhiteBalance="0"	// WhiteBalance,0 means auto, 1 means in door, 2 means out door

BackLight="128"	// BackLight 0-255

Switch2A="1"		// enable Switch2A or not, 1 means yes, 0 means no

BinningSkip="0"	// BinningSkip, 0 means Binning, 1 means Skip

TVSystem="1"		// TVSystem, 0 means 60HZ, 1 means 50HZ

HFlip="0"			// enable HFlip or not, 1 means yes, 0 means no

VFlip="0"			// enable VFlip or not, 1 means yes, 0 means no

TNF="0"			    //reserved

SNF="0"              //reserved

VSTAB="0"           //reserved

LDC="0"              //reserved

FD="0"                //reserved

IrcutMode="0"		// IrcutMode, 0 means auto, 1 means day-night mode, 2 means passive, 3 means manual

IrcutSensitivity="50"	// when ircutmode is auto, this value indicates sensitivity

IrcutNightStartTime="18:00:00"	// night start-time, when the ending-time is smaller than starting-time, express acrossed one day, such as this setting means that 6:00 in the night to 8:00 in the morning the next day

IrcutNightEndTime="08:00:00"	// night end-time

/>

### Modify time title parameter configuration (command 525)

#define CMD_SET_MEDIA_VIDEO_OSD			(CMD_MEDIA_CONFIG_BASE + 25)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_VIDEO_OSD,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<Overlay

Enable="1"		//enable time title information show or not

Transparency="0"

>

<TimeOverlay

PosX="0"	//0 means left, 1 means right

PosY="0" //0 means top, 1 means bottom

Format="yyyy-mm-dd hh:mm:ss" // time show format, the supportting format are yyyy-mm-dd hh:mm:ss, yyyy/mm/dd hh:mm:ss, yy-mm-dd hh:mm:ss, yy/mm/dd hh:mm:ss, hh:mm:ss dd/mm/yyyy, hh:mm:ss dd-mm-yyyy, hh:mm:ss mm/dd/yyyy, hh:mm:ss mm-dd-yyyy

/>

<TitleOverlay

PosX="0" //0 means left, 1 means right

PosY="1" //0 menas top, 1 means bottom

Title="5433384647"		// Characters are converted to ASCII code 16 hexadecimal form

/>

</Overlay>

Notice: PosX and PosY control the display content together, time and title must one in the top and another in the bottom, that is to say, their PosY can not the same.

### Modify picture capture configuration (command 526)

#define CMD_SET_MEDIA_VIDEO_JPEG		    (CMD_MEDIA_CONFIG_BASE + 26)

Call Method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_VIDEO_JPEG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<JpegConfig

Enable="1"	// enable picture capture or not

Stream="0"	// the stream of capture, 0 means main stream, 1 means sub stream

Quality="80"	// picture quality 20-100

Framerate="2" // capture speed, how many pictures need to be captured every second, you can choose one or two

/>

### Modify audio capture configuration (command 527)

#define CMD_SET_MEDIA_AUDIO_CAPTURE	(CMD_MEDIA_CONFIG_BASE + 27)

Call Method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_AUDIO_CAPTURE,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<Capture

Volume="25"	//capture volume, 0-100

/>

### Modify audio encode configuration (command 528)

#define CMD_SET_MEDIA_AUDIO_ENCODE		(CMD_MEDIA_CONFIG_BASE + 28)

Call Method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIA_AUDIO_ENCODE,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<Encode

Enable="1"			//enable video encode or not

SampleRate="16000"	//SampleRate

EncodeType="AAC"	//EncodeType, support AAC, G711

BitRate="16000"		//BitRate

/>

### Modify privacy mask (command 529)

#define CMD_SET_PRIVACY_CONFIG           (CMD_MEDIA_CONFIG_BASE + 29)

Call Method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_PRIVACY_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<Mask>// privacy mask

<MainStream // main stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

<SubStream // sub stream privacy area

Area1PosX="0"

Area1PosY="0"

Area1Width="0"

Area1Height="0"

Area2PosX="0"

Area2PosY="0"

Area2Width="0"

Area2Height="0"

Area3PosX="0"

Area3PosY="0"

Area3Width="0"

Area3Height="0"

Area4PosX="0"

Area4PosY="0"

Area4Width="0"

Area4Height="0"

/>

</Mask>

Note: Width and height value of privacy mask area can not exceed 1/2 of resolution value, otherwise it won’t take effect.

### Read streaming media parameter (command 600)

#define CMD_MEDIASTREAM_CONFIG_BASE			600

#define CMD_GET_MEDIASTREAM_CONFIG			(CMD_MEDIASTREAM_CONFIG_BASE + 0)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_MEDIASTREAM_CONFIG,0,"",0,0);

Return XML:

<MediaStreamConfig>

<StreamAccess

Auth="1"				//authentication or not, 1 means yes, 0 means no

VideoPort="554"		//video port

RTPOverRTSP="0"		//media access protocol, 1 means TCP, 0 means UDP

PTZPort="8091"		//PTZ control port

WEBPort="80"		//the port to access device Web configuration interface

/>

</MediaStreamConfig>

### Modify streaming media parameter (command 620)

#define CMD_SET_MEDIASTREAM_CONFIG			(CMD_MEDIASTREAM_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIASTREAM_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<MediaStreamConfig>

<StreamAccess

Auth="1"				// authentication or not, 1 means yes, 0 means no

VideoPort="554"		// video port

RTPOverRTSP="0"		// media access protocol, 1 means TCP, 0 means UDP

PTZPort="8091"		// PTZ control port

WEBPort="80"		// the port to access device Web configuration interface

/>

</MediaStreamConfig>

### Read storage setup parameter (command 700)

#define CMD_RECORD_CONFIG_BASE				700

#define CMD_GET_RECORD_CONFIG				(CMD_RECORD_CONFIG_BASE + 0)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_RECORD_CONFIG,0,"",0,0);

Return XML:

<RecordConfig>

<Common

LocalEnable="1"		// support local record or not, 1 means yes, 0 means no

StorageSequence="0"	// Storage sequence, 0 means automatically select, the others expressed in three digits, 1: SD1, 2: SD2, 3: USB, for example, 213 express in the order of SD2, SD1, USB, 321 express in the order of USB, SD2, SD1

RemoteEnable="0"		//support remote directory record or not, 1 means yes, 0 means no

MountParam=""		//mount parameter, only when it supports remote directory record this value has effective

StoragePolicy="FIFO"	//storage strategy, FIFO means delete the oldest files, STOP means stop when full

RecordFileSize="20"	//the record time for every file, the unit is minute, can choose 10, 20, 30

/>

<ScheduleRecord		//schedule record parameter

Stream="1"			//record stream, 1 means main stream, 2 means sub stream

FileFormat="AVI"		//record format, current only support AVI

MediaType="AV"	//media type, AV means video and audio, only the device with audio can support this option, VIDEO means only video

LocalStore="0"	//support local record or not, 1 means yes, 0 means no. only when support local record has effective, otherwise, must be 0

RemoteStore="0"	//use remote store or not, 1 means yes, 0 means no. only when support remote store has effective, otherwise, must be 0

>

<EnableTimeList>	//schedule record time setup, notice that when save it, must ensure that the time can not repeat

<Workday			//support more than one options

Day="7"				//0, 1, 2, 3, 4, 5, 6, 7, represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday

>

<TimeSpan			//in a Workday can have many Time Span

StartTime=" 0: 0: 0"	//start time

EndTime=" 7:59: 0"	//end time

/>

<TimeSpan

StartTime="18: 1: 0"	//start time

EndTime="24: 0: 0"	//end time

/>

</Workday>

</EnableTimeList>

</ScheduleRecord>

<MotionDetectRecord		//motion detect alarm record

Stream="1"				//record stream, 1 means main stream, 2 means sub stream

FileFormat="AVI"			// record format, current only support AVI

MediaType="AV"			// media type, AV means video and audio, only the device with audio can support this option, VIDEO means only video

PreRecordTime="5"		//prerecord time

RecordTime="100"			//record time

LocalStore="1"			// support local record or not, 1 means yes, 0 means no. only when support local record has effective, otherwise, must be 0

RemoteStore="0"			// use remote store or not, 1 means yes, 0 means no. only when support remote store has effective, otherwise, must be 0

FtpUpload="0"			//record file upload or not, 1 means yes, 0 means no. only when set FTP correct can upload

EmailUpload="0"			//after recording set mail or not, 1 means yes, 0 means no. only when set SMTP correct can set

/>

<MotionDetectCapture		//motion detect alarm capture

PreTakeTime="5"			//pre-capture time

TotalTakeTime="10"		//capture time, only when enable capture, this function has effective

LocalStore="0"			//enable local store or not

RemoteStore="0"			//enable remote store or not

FtpUpload="0"			//upload the picture to FTP or not

EmailUpload="0"			//set E-mail or not for picture

/>

<InputAlarmRecord		//IO alarm record

Stream="2"				//record stream, 1 means main stream, 2 means sub stream

FileFormat="AVI"			// record format, current only support AVI

MediaType="AV"			// media type, AV means video and audio, only the device with audio can support this option, VIDEO means only video

PreRecordTime="5"		// prerecord time

RecordTime="100"			// record time

LocalStore="1"			// support local record or not, 1 means yes, 0 means no. only when support local record has effective, otherwise, must be 0

RemoteStore="0"			// use remote store or not, 1 means yes, 0 means no. only when support remote store has effective, otherwise, must be 0

FtpUpload="0"			// record file upload or not, 1 means yes, 0 means no. only when set FTP correct can upload

EmailUpload="0"			// after recording set mail or not, 1 means yes, 0 means no. only when set SMTP correct can set

/>

<InputAlarmCapture		//IO alarm capture

PreTakeTime="5"			// pre-capture time

TotalTakeTime="10"		// capture time, only when enable capture, this function has effective

LocalStore="0"			// enable local store or not

RemoteStore="0"			// enable remote store or not

FtpUpload="0"			// upload the picture to FTP or not

EmailUpload="0"			// set E-mail or not for picture

/>

</RecordConfig>

### Modify storage setup parameter (command 720)

#define CMD_SET_RECORD_CONFIG				(CMD_RECORD_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_MEDIASTREAM_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<RecordConfig>

<Common

LocalEnable="1"		// support local record or not, 1 means yes, 0 means no

StorageSequence="0"	// Storage sequence, 0 means automatically select, the others expressed in three digits, 1: SD1, 2: SD2, 3: USB, for example, 213 express in the order of SD2, SD1, USB, 321 express in the order of USB, SD2, SD1

RemoteEnable="0"		// support remote directory record or not, 1 means yes, 0 means no

MountParam=""		// mount parameter, only when it supports remote directory record this value has effective

StoragePolicy="FIFO"	// storage strategy, FIFO means delete the oldest files, STOP means stop when full

RecordFileSize="20"	// the record time for every file, the unit is minute, can choose 10, 20, 30

/>

<ScheduleRecord		// schedule record parameter

Stream="1"			// record stream, 1 means main stream, 2 means sub stream

FileFormat="AVI"		// record format, current only support AVI

MediaType="AV"		// media type, AV means video and audio, only the device with audio can support this option, VIDEO means only video

LocalStore="0"		// support local record or not, 1 means yes, 0 means no. only when support local record has effective, otherwise, must be 0

RemoteStore="0"		// use remote store or not, 1 means yes, 0 means no. only when support remote store has effective, otherwise, must be 0

>

<EnableTimeList>		// schedule record time setup, notice that when save it, must ensure that the time can not repeat

<Workday			// support more than one options

Day="7"				//0, 1, 2, 3, 4, 5, 6, 7, represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday

>

<TimeSpan			// in a Workday can have many Time Span

StartTime=" 0: 0: 0"	// start time

EndTime=" 7:59: 0"	// end time

/>

<TimeSpan

StartTime="18: 1: 0"	// start time

EndTime="24: 0: 0"	// end time

/>

</Workday>

</EnableTimeList>

</ScheduleRecord>

<MotionDetectRecord		// motion detect alarm record

Stream="1"				// record stream, 1 means main stream, 2 means sub stream

FileFormat="AVI"			// record format, current only support AVI

MediaType="AV"			// media type, AV means video and audio, only the device with audio can support this option, VIDEO means only video

PreRecordTime="5"		// prerecord time

RecordTime="100"			// record time

LocalStore="1"			// support local record or not, 1 means yes, 0 means no. only when support local record has effective, otherwise, must be 0

RemoteStore="0"			// use remote store or not, 1 means yes, 0 means no. only when support remote store has effective, otherwise, must be 0

FtpUpload="0"			// record file upload or not, 1 means yes, 0 means no. only when set FTP correct can upload

EmailUpload="0"			// after recording set mail or not, 1 means yes, 0 means no. only when set SMTP correct can set

/>

<MotionDetectCapture		// motion detect alarm capture

PreTakeTime="5"			// pre-capture time

TotalTakeTime="10"		// capture time, only when enable capture, this function has effective

LocalStore="0"			// enable local store or not

RemoteStore="0"			// enable remote store or not

FtpUpload="0"			// upload the picture to FTP or not

EmailUpload="0"			// set E-mail or not for picture

/>

<InputAlarmRecord		// IO alarm record

Stream="2"				// record stream, 1 means main stream, 2 means sub stream

FileFormat="AVI"			// record format, current only support AVI

MediaType="AV"			// media type, AV means video and audio, only the device with audio can support this option, VIDEO means only video

PreRecordTime="5"		// prerecord time

RecordTime="100"			// record time

LocalStore="1"			// support local record or not, 1 means yes, 0 means no. only when support local record has effective, otherwise, must be 0

RemoteStore="0"			// use remote store or not, 1 means yes, 0 means no. only when support remote store has effective, otherwise, must be 0

FtpUpload="0"			// record file upload or not, 1 means yes, 0 means no. only when set FTP correct can upload

EmailUpload="0"			// after recording set mail or not, 1 means yes, 0 means no. only when set SMTP correct can set

/>

<InputAlarmCapture		// IO alarm capture

PreTakeTime="5"			// pre-capture time

TotalTakeTime="10"		// capture time, only when enable capture, this function has effective

LocalStore="0"			// enable local store or not

RemoteStore="0"			// enable remote store or not

FtpUpload="0"			// upload the picture to FTP or not

EmailUpload="0"			// set E-mail or not for picture

/>

</RecordConfig>

## 7. Alarm setup command

#define CMD_ALARM_CONFIG_BASE				800

### Read alarm setting parameter (command 800)

#define CMD_ALARM_CONFIG_BASE				800

#define CMD_GET_ALARM_CONFIG				(CMD_ALARM_CONFIG_BASE + 0)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_ALARM_CONFIG,0,"",0,0);

Return XML:

<AlarmConfig>

<InputAlarm>

<AlarmChannel

PortIndex="1"					//the number of IO input port, only one current, the number is 1

ChannelType="alarmoutput1"	//the name of input port, only one current, that is alarmoutput1

TriggerType="HIGH-LOW"		// Trigger type

Enable="1"					//enable or not

>

<EnableTimeList>	//the time can not overlap

<Workday	//workday, there can be multiple

Day="7"		//0, 1, 2, 3, 4, 5, 6, 7, represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday

>

<TimeSpan		//In the same workday, there can be multiple

StartTime=" 0: 0: 0"	//start time

EndTime="24: 0: 0"	//end time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	//alarm output

Enable="1"			//enable or not, 1 means yes, 0 means no

PortIndex="1"			//the number of output port

ChannelType="alarmoutput1"	//the name of output port

TriggerType="HIGH"			// the type of output

Duration="10"					// Duration, 0~60, 0 means alarm all the time

/>

<PTZAction

Enable="0"					// PTZAction, does not support currently, so its value is 0

ActionType="PositionLoop"		// PTZAction type, does not support currently, so its value is always PositionLoop

>

<PositionLoop					// does not support currently

Interval="1"

Duration="10"					// Duration, 0~60, 0 means alarm all the time

>

<PTZPosition					//does not support currently

PositionIndex="1"

/>

<PTZPosition					//does not support currently

PositionIndex="2"

/>

</PositionLoop>

</PTZAction>

</AlarmAction>

</AlarmChannel>

</InputAlarm>

<MotionDetectAlarm			// Motion Detect Alarm

Enable="1"					//enable or not, 1 means yes, 0 means no

BlockCount="262147"			//layout program x=(262147 & 0xffff0000)>>16,y=(262147 & 0xffff)

BlockConfig="111111111111"	//divide the full-screen into small-box, the number is x by y, then order by row priority. 1 means enable, 0 means no

Sensitivity="100"				// Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

AlarmThreshold="1"			// Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

DayNightSwitch="1"			//enable night time or not

NightSensitivity="100"			// Night Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

NightAlarmThreshold="30"		// Night Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

NightStartTime=" 0: 0: 0"		//night start time, when end time is smaller than start time, means across a day

NightEndTime="24: 0: 0"		//night end time

>

<EnableTimeList>

<Workday		//workday

Day="7"			//0,1,2,3,4,5,6,7 represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday.

>

<TimeSpan

StartTime=" 8: 0: 0"	//start time

EndTime="18: 0: 0"	//end time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm output action

Enable="1"			//enable or not, 1 means yes, 0 means no

PortIndex="1"			//output port, currently fixed at 1

ChannelType="1"		// currently fixed at 1

TriggerType="HIGH"	//output method

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

<PTZAction		//this is no longer used

Enable="0"

ActionType="PositionWalk"

>

<PositionWalk		// this is no longer used

Interval="2"

WalkCount="1"

>

<PTZPosition		// this is no longer used

PositionIndex="1"

/>

<PTZPosition		// this is no longer used

PositionIndex="2"

/>

<PTZPosition		// this is no longer used

PositionIndex="3"

/>

</PositionWalk>

</PTZAction>

</AlarmAction>

</MotionDetectAlarm>

<VideoLostAlarm	// Video Lost Alarm

Enable="1"		//enable or not, 1 means yes, 0 means no

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm output action

Enable="0"			// enable or not, 1 means yes, 0 means no

PortIndex="0"			//output port, current is 0

ChannelType="0"		//output channel, current is 0

TriggerType="HIGH"	//output method, HIGH or LOW

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoLostAlarm>

<VideoCoverAlarm		// Video Cover Alarm, this is no longer used

Enable="1"			//enable or not

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="1"				// enable or not, 1 means yes, 0 means no

PortIndex="1"				// IO output port, current is 1

ChannelType="Alarm Bell"	// IO output channel, current is Alarm Bell

TriggerType="HIGH"		// IO output method, HIGH or LOW

Duration="60"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoCoverAlarm>

<StorageFullAlarm			// Storage Alarm

Enable="1"				// enable or not, 1 means yes, 0 means no

Threshold="95"			//alarm Threshold, value is from 50 to 100

>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="0"				// enable or not, 1 means yes, 0 means no

PortIndex="0"				// output port, current is 0

ChannelType="0"			// output channel, current is 0

TriggerType="HIGH"		// output method, HIGH or LOW

Duration="10"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</StorageFullAlarm>

</AlarmConfig>

### Modify alarm setting parameter (command 820)

#define CMD_SET_ALARM_CONFIG				(CMD_ALARM_CONFIG_BASE + 20)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<AlarmConfig>

<InputAlarm>

<AlarmChannel

PortIndex="1"					// the number of IO input port, only one current, the number is 1

ChannelType="alarmoutput1"	// the name of input port, only one current, that is alarmoutput1

TriggerType="HIGH-LOW"		// Trigger Type

Enable="1"					// enable it or not

>

<EnableTimeList>	// the time can not overlap

<Workday	// workday, there can be multiple

Day="7"		//0, 1, 2, 3, 4, 5, 6, 7, represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday

>

<TimeSpan		// In the same workday, there can be multiple

StartTime=" 0: 0: 0"	// Start Time

EndTime="24: 0: 0"	// End Time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="1"			// enable or not, 1 means yes, 0 means no

PortIndex="1"			// the number of output port

ChannelType="alarmoutput1"	// the name of output port

TriggerType="HIGH"			// the type of output

Duration="10"					// Duration, 0~60, 0 means alarm all the time

/>

<PTZAction

Enable="0"					// PTZ Action, does not support currently, so its value is 0

ActionType="PositionLoop"		// PTZ Action type, does not support currently, so its value is always PositionLoop

>

<PositionLoop					// does not support currently

Interval="1"

Duration="10"					// Duration, 0~60, 0 means alarm all the time

>

<PTZPosition					// does not support currently

PositionIndex="1"

/>

<PTZPosition					// does not support currently

PositionIndex="2"

/>

</PositionLoop>

</PTZAction>

</AlarmAction>

</AlarmChannel>

</InputAlarm>

<MotionDetectAlarm			// Motion Detect Alarm

Enable="1"					// enable or not, 1 means yes, 0 means no

BlockCount="262147"			// layout program x=(262147 & 0xffff0000)>>16,y=(262147 & 0xffff)

BlockConfig="111111111111"	// divide the full-screen into small-box, the number is x by y, then order by row priority. 1 means enable, 0 means no

Sensitivity="100"				// Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

AlarmThreshold="1"			// Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

DayNightSwitch="1"			// enable night time or not

NightSensitivity="100"			// Night Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

NightAlarmThreshold="30"		// Night Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

NightStartTime=" 0: 0: 0"		// night start time, when end time is smaller than start time, means across a day

NightEndTime="24: 0: 0"		// night end time

>

<EnableTimeList>

<Workday		// workday

Day="7"			//0, 1, 2, 3, 4, 5, 6, 7 represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday.

>

<TimeSpan

StartTime=" 8: 0: 0"	// Start Time

EndTime="18: 0: 0"	// End Time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="1"			// enable or not, 1 means yes, 0 means no

PortIndex="1"			// output port, currently fixed at 1

ChannelType="1"		// currently fixed at 1

TriggerType="HIGH"	// output method

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

<PTZAction		// this is no longer used

Enable="0"

ActionType="PositionWalk"

>

<PositionWalk		// this is no longer used

Interval="2"

WalkCount="1"

>

<PTZPosition		// this is no longer used

PositionIndex="1"

/>

<PTZPosition		// this is no longer used

PositionIndex="2"

/>

<PTZPosition		// this is no longer used

PositionIndex="3"

/>

</PositionWalk>

</PTZAction>

</AlarmAction>

</MotionDetectAlarm>

<VideoLostAlarm	// Video Lost Alarm

Enable="1"		// enable or not, 1 means yes, 0 means no

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="0"			// enable or not, 1 means yes, 0 means no

PortIndex="0"			// output port, current is 0

ChannelType="0"		// output channel, current is 0

TriggerType="HIGH"	// output method, HIGH or LOW

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoLostAlarm>

<VideoCoverAlarm		// Video Cover Alarm, this is no longer used

Enable="1"			// enable or not

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="1"				// enable or not, 1 means yes, 0 means no

PortIndex="1"				// IO output port, current is 1

ChannelType="Alarm Bell"	// IO output channel, current is Alarm Bell

TriggerType="HIGH"		// IO output method, HIGH or LOW

Duration="60"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoCoverAlarm>

<StorageFullAlarm			// Storage Alarm

Enable="1"				// enable or not, 1 means yes, 0 means no

Threshold="95"			// alarm Threshold, value is from 50 to 100

>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="0"				// enable or not, 1 means yes, 0 means no

PortIndex="0"				// output port, current is 0

ChannelType="0"			// output channel, current is 0

TriggerType="HIGH"		// output method, HIGH or LOW

Duration="10"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</StorageFullAlarm>

</AlarmConfig>

### Read IO input alarm parameter (command 801)

#define CMD_GET_ALARM_INPUT_CONFIG			(CMD_ALARM_CONFIG_BASE + 1)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_ALARM_INPUT_CONFIG,0,"",0,0);

Return XML:

<InputAlarm>

<AlarmChannel

PortIndex="1"					// the number of IO input port, only one current, the number is 1

ChannelType="alarmoutput1"	// the name of input port, only one current, that is alarmoutput1

TriggerType="HIGH-LOW"		// Trigger Type

Enable="1"					// enable or not

>

<EnableTimeList>	// the time can not overlap

<Workday	// workday, there can be multiple

Day="7"		//0, 1, 2, 3, 4, 5, 6, 7, represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday

>

<TimeSpan		// In the same workday, there can be multiple

StartTime=" 0: 0: 0"	// Start Time

EndTime="24: 0: 0"	// End Time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="1"			// enable or not, 1 means yes, 0 means no

PortIndex="1"			// the number of output port

ChannelType="alarmoutput1"	// the name of output port

TriggerType="HIGH"			// the type of output

Duration="10"					// Duration, 0~60, 0 means alarm all the time

/>

<PTZAction

Enable="0"					// PTZ Action, does not support currently, so its value is 0

ActionType="PositionLoop"		// PTZ Action type, does not support currently, its value is always PositionLoop

>

<PositionLoop					// does not support currently

Interval="1"

Duration="10"					// Duration, 0~60, 0 means alarm all the time

>

<PTZPosition					// does not support currently

PositionIndex="1"

/>

<PTZPosition					// does not support currently

PositionIndex="2"

/>

</PositionLoop>

</PTZAction>

</AlarmAction>

</AlarmChannel>

</InputAlarm>

### Modify IO input alarm parameter (command 821)

#define CMD_SET_ALARM_INPUT_CONFIG			(CMD_ALARM_CONFIG_BASE + 21)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_INPUT_CONFIG ,0 ,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<InputAlarm>

<AlarmChannel

PortIndex="1"					// the number of IO input port, only one current, the number is 1

ChannelType="alarmoutput1"	// the name of input port, only one current, that is alarmoutput1

TriggerType="HIGH-LOW"		// Trigger Type

Enable="1"					// enable or not

>

<EnableTimeList>	// the time can not overlap

<Workday	// workday, there can be multiple

Day="7"		//0, 1, 2, 3, 4, 5, 6, 7, represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday

>

<TimeSpan		// In the same workday, there can be multiple

StartTime=" 0: 0: 0"	// Start Time

EndTime="24: 0: 0"	// End Time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="1"			// enable or not, 1 means yes, 0 means no

PortIndex="1"			// the number of output port

ChannelType="alarmoutput1"	// the name of output port

TriggerType="HIGH"			// the type of output

Duration="10"					// Duration, 0~60, 0 means alarm all the time

/>

<PTZAction

Enable="0"					// PTZ Action, does not support currently, so its value is always 0

ActionType="PositionLoop"		// PTZ Action type, does not support currently, so its value is always PositionLoop

>

<PositionLoop					// does not support currently

Interval="1"

Duration="10"					// Duration, 0~60, 0 means alarm all the time

>

<PTZPosition					// does not support currently

PositionIndex="1"

/>

<PTZPosition					// does not support currently

PositionIndex="2"

/>

</PositionLoop>

</PTZAction>

</AlarmAction>

</AlarmChannel>

</InputAlarm>

### Read motion detect alarm parameter (command 802)

#define CMD_GET_ALARM_MOTIONDETECT_CONFIG		(CMD_ALARM_CONFIG_BASE + 2)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_ALARM_MOTIONDETECT_CONFIG,0,"",0,0);

Return XML:

<MotionDetectAlarm			// Motion Detect Alarm

Enable="1"					// enable or not, 1 means yes, 0 means no

BlockCount="262147"			// layout program x=(262147 & 0xffff0000)>>16,y=(262147 & 0xffff)

BlockConfig="111111111111"	// divide the full-screen into small-box, the number is x by y, then order by row priority. 1 means enable, 0 means no

Sensitivity="100"				// Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

AlarmThreshold="1"			// Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

DayNightSwitch="1"			// enable night time or not

NightSensitivity="100"			// Night Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

NightAlarmThreshold="30"		// Night Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

NightStartTime=" 0: 0: 0"		// night start time, when end time is smaller than start time, means across a day

NightEndTime="24: 0: 0"		// night end time

>

<EnableTimeList>

<Workday		// workday

Day="7"			//0,1,2,3,4,5,6,7 represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday.

>

<TimeSpan

StartTime=" 8: 0: 0"	// Start Time

EndTime="18: 0: 0"	// End Time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="1"			// enable or not, 1 means yes, 0 means no

PortIndex="1"			// output port, currently fixed at 1

ChannelType="1"		// currently fixed at 1

TriggerType="HIGH"	// output method

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

<PTZAction		// this is no longer used

Enable="0"

ActionType="PositionWalk"

>

<PositionWalk		// this is no longer used

Interval="2"

WalkCount="1"

>

<PTZPosition		// this is no longer used

PositionIndex="1"

/>

<PTZPosition		// this is no longer used

PositionIndex="2"

/>

<PTZPosition		// this is no longer used

PositionIndex="3"

/>

</PositionWalk>

</PTZAction>

</AlarmAction>

</MotionDetectAlarm>

### Modify motion detect alarm parameter (command 822)

#define CMD_SET_ALARM_MOTIONDETECT_CONFIG		(CMD_ALARM_CONFIG_BASE + 22)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_MOTIONDETECT_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<MotionDetectAlarm			// Motion Detect Alarm

Enable="1"					// enable or not, 1 means yes, 0 means no

BlockCount="262147"			// layout program x= (262147 & 0xffff0000)>>16, y= (262147 & 0xffff)

BlockConfig="111111111111"	// divide the full-screen into small-box, the number is x by y, then order by row priority. 1 means enable, 0 means no

Sensitivity="100"				// Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

AlarmThreshold="1"			// Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

DayNightSwitch="1"			// enable night time or not

NightSensitivity="100"			// Night Sensitivity, this value is bigger and more prone to alarm, its value from 1 to 100

NightAlarmThreshold="30"		// Night Alarm Threshold, this value is smaller and more prone to alarm, its value from 1 to 100

NightStartTime=" 0: 0: 0"		// night start time, when end time is smaller than start time, means across a day

NightEndTime="24: 0: 0"		// night end time

>

<EnableTimeList>

<Workday		// workday

Day="7"			//0, 1, 2, 3, 4, 5, 6, 7 represent Sunday, Monday, Tuesday, Wednesday, Thursday, Friday, Saturday, everyday.

>

<TimeSpan

StartTime=" 8: 0: 0"	// Start Time

EndTime="18: 0: 0"	// End Time

/>

</Workday>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="1"			// enable or not, 1 means yes, 0 means no

PortIndex="1"			// output port, currently fixed at 1

ChannelType="1"		// currently fixed at 1

TriggerType="HIGH"	// output method

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

<PTZAction		// this is no longer used

Enable="0"

ActionType="PositionWalk"

>

<PositionWalk		// this is no longer used

Interval="2"

WalkCount="1"

>

<PTZPosition		// this is no longer used

PositionIndex="1"

/>

<PTZPosition		// this is no longer used

PositionIndex="2"

/>

<PTZPosition		// this is no longer used

PositionIndex="3"

/>

</PositionWalk>

</PTZAction>

</AlarmAction>

</MotionDetectAlarm>

### Read video lost alarm parameter (command 803)

#define CMD_GET_ALARM_VIDEOCOVER_CONFIG		(CMD_ALARM_CONFIG_BASE + 3)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_ALARM_VIDEOCOVER_CONFIG,0,"",0,0);

Return XML:

<VideoLostAlarm	// Video Lost Alarm

Enable="1"		// enable or not, 1 means yes, 0 means no

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="0"			// enable or not, 1 means yes, 0 means no

PortIndex="0"			// output port, current is 0

ChannelType="0"		// output channel, current is 0

TriggerType="HIGH"	// output method, HIGH or LOW

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoLostAlarm>

Notice:

This option no longer used

### Modify video lost alarm parameter (command 823)

#define CMD_SET_ALARM_VIDEOCOVER_CONFIG		(CMD_ALARM_CONFIG_BASE + 23)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_VIDEOCOVER_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<VideoLostAlarm	// Video Lost Alarm

Enable="1"		// enable or not, 1 means yes, 0 means no

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction	// Alarm Output Action

Enable="0"			// enable or not, 1 means yes, 0 means no

PortIndex="0"			// output port, current is 0

ChannelType="0"		// output channel, current is 0

TriggerType="HIGH"	// output method, HIGH or LOW

Duration="10"			// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoLostAlarm>

Notice:

This option no longer used

### Read video lost alarm parameter (command 804)

#define CMD_GET_ALARM_VIDEOLOST_CONFIG		(CMD_ALARM_CONFIG_BASE + 4)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_GET_ALARM_VIDEOLOST_CONFIG,0,"",0,0);

Return XML:

<VideoCoverAlarm		// Video Cover Alarm, this is no longer used

Enable="1"			// enable or not

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="1"				// enable or not, 1 means yes, 0 means no

PortIndex="1"				// IO output port, current is 1

ChannelType="Alarm Bell"	// IO output channel, current is Alarm Bell

TriggerType="HIGH"		// IO output method, HIGH or LOW

Duration="60"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoCoverAlarm>

### Modify video lost alarm parameter (command 824)

#define CMD_SET_ALARM_VIDEOLOST_CONFIG		(CMD_ALARM_CONFIG_BASE + 24)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_VIDEOLOST_CONFIG,0,pXml,strlen(pXml));

The pXml corresponding contents are as follows:

<VideoCoverAlarm		// Video Cover Alarm, this is no longer used

Enable="1"			// enable or not

>

<EnableTimeList>

</EnableTimeList>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="1"				// enable or not, 1 means yes, 0 means no

PortIndex="1"				// IO output port, current is 1

ChannelType="Alarm Bell"	// IO output channel, current is Alarm Bell

TriggerType="HIGH"		// IO output method, HIGH or LOW

Duration="60"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</VideoCoverAlarm>

### Read storage alarm patameter (command 805)

#define CMD_SET_ALARM_SF_CONFIG				(CMD_ALARM_CONFIG_BASE + 25)

Call method:

AJ_NETSDK_IPC_GetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_SF_CONFIG,0,"",0,0);

Return XML:

<StorageFullAlarm			// Storage Full Alarm

Enable="1"				// enable or not, 1 means yes, 0 means no

Threshold="95"			// alarm Threshold, value is from 50 to 100

>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="0"				// enable or not, 1 means yes, 0 means no

PortIndex="0"				// output port, current is 0

ChannelType="0"			// output channel, current is 0

TriggerType="HIGH"		// output method, HIGH or LOW

Duration="10"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</StorageFullAlarm>

### Modify storage alarm patameter (command 825)

#define CMD_SET_ALARM_SF_CONFIG				(CMD_ALARM_CONFIG_BASE + 25)

Call method:

AJ_NETSDK_IPC_SetDVRConfig(m_nLastCmdId, CMD_SET_ALARM_SF_CONFIG,0,pXml,strlen (pXml));

The pXml corresponding contents are as follows:

<StorageFullAlarm			// Storage Full Alarm

Enable="1"				// enable or not, 1 means yes, 0 means no

Threshold="95"			// alarm Threshold, value is from 50 to 100

>

<AlarmAction>

<AlarmOutputAction		// Alarm Output Action

Enable="0"				// enable or not, 1 means yes, 0 means no

PortIndex="0"				// output port, current is 0

ChannelType="0"			// output channel, current is 0

TriggerType="HIGH"		// output method, HIGH or LOW

Duration="10"				// Duration, unit is second, value is from 0 to 60, 0 means alarm all the time

/>

</AlarmAction>

</StorageFullAlarm>

## 8. System control command

#define CMD_SYSTEM_MANAGE_BASE 1000

After called command AJ_NETSDK_IPC_SystemControl, there will receive a corresponding command in the callback function OnAuxResponse. The first bit in other cmdId or say cmdId&0xff000000, if it is 0 then means command send successfully, otherwise, means failure, and cmdId&0xffffff is a corresponding command value.

### Read system information (command 1012)

#define CMD_SYSTEM_MANAGE_BASE 1000

#define CMD_GET_SYSTEM_VERSION_INFO 	(CMD_SYSTEM_MANAGE_BASE + 12)

Call method:

AJ_NETSDK_IPC_SystemControl (m_nLastCmdId, CMD_GET_SYSTEM_VERSION_INFO,0,"", pXml,0);

Return XML:

Linux 2.6.18_arm_v5t_le #1 PREEMPT Thu Nov 18 11:30:21 GMT+8 2010

### Restore to default configuration (command 1002)

#define	 CMD_CONFIG_RESTORE			(CMD_SYSTEM_MANAGE_BASE + 2)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_CONFIG_RESTORE,0,””)

If return a wrong mark 0 means success

### Modify system time (command 1005)

#define	 CMD_SET_SYSTEM_TIME			(CMD_SYSTEM_MANAGE_BASE + 5)

Call Method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_CONFIG_RESTORE,0,xmldata)

The format of xmldata:

<REQUEST_PARAM

Time="20120424 17:45:01" 			//time format is yyyyMMdd HH:mm:ss

TimeZone="4800"   // time zone, the formula of time zone a is (12+a)*60, a ranges from -12 to +12

/>

If return a wrong mark 0 means success

Note:

This control command takes effect only when time quartum is mannually modify time.

### Restart system (command 1007)

#define CMD_SYSTEM_REBOOT			(CMD_SYSTEM_MANAGE_BASE + 7)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_SYSTEM_REBOOT,0,””)

If return a wrong mark 0, then express successfully

Notice:

Recommend using AJ_NETSDK_IPC_RebootDVR

### Receive storage information (command 1014)

#define CMD_GET_STORAGE_INFO 		(CMD_SYSTEM_MANAGE_BASE + 14)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_STORAGE_INFO,0,””)

Return Xml:

<RESPONSE_PARAM>

<DeviceInfo

DevName="sd1"	//maybe sd1, sd2 or usb, express differently storage device

Mounted="0"	// mounted or not

Total="0"		//The total storage space that can be used, if the number is 0 means that this device does not exist

Used="0"		//the size of used space

Free="0"		//the size of can be used space

Percent="0" 	//the percent of used space

/>

<DeviceInfo

DevName="sd2"

Mounted="0"	// mounted or not

Total="0"		// The total storage space that can be used, if the number is 0 means that this device does not exist

Used="0"		// the size of used space

Free="0"		// the size of can be used space

Percent="0" 	// the percent of used space

/>

<DeviceInfo

DevName="usb"

Mounted="0"	// mounted or not

Total="0"		// The total storage space that can be used, if the number is 0 means that this device does not exist

Used="0"		// the size of used space

Free="0"		// the size of can be used space

Percent="0" 	// the percent of used space

/>

</RESPONSE_PARAM>

### Format front-end storage device (command 1017)

#define CMD_STORAGE_DEVICE_FORMAT	(CMD_SYSTEM_MANAGE_BASE + 17)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_STORAGE_DEVICE_FORMAT,0, devType)

devType maybe sd1,sd2,usb

If return a wrong mark 0, then express successfully

### Unmount front-end storage device (command 1018)

#define CMD_STORAGE_DEVICE_UNMOUNT  	(CMD_SYSTEM_MANAGE_BASE + 18)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_STORAGE_DEVICE_UNMOUNT,0,devType)

devType maybe sd1,sd2,usb

If return a wrong mark 0, then express successfully

### Receive device serial number information (command 1019)

#define 		CMD_GET_SERIALNUMBER		(CMD_SYSTEM_MANAGE_BASE + 19)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_SERIALNUMBER,0,””)

Return Xml:

<RESPONSE_PARAM SerialNumber="4756736557496768" />

The serial number is 475673655749676

### Read system capacity set (command 1020)

#define CMD_GET_SYSTEMCONTROLSTRING	(CMD_SYSTEM_MANAGE_BASE + 20)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_SYSTEMCONTROLSTRING,0,””)

Return Xml:

<RESPONSE_PARAM SystemConfigString=" + audio_support + schedule_record + ftpemail_storage + picture_capture + ptz_control + gpio_input + gpio_output + zh_cn + en_us + zh_tw + front_replay + media_capabiltiy" />

audio_support 		//support audio

schedule_record 		//support schedule record

ftpemail_storage 		//support ftp and email storage

picture_capture 		//support capture

ptz_control 			//support ptz

gpio_input 			//support IO input

gpio_output 			//support IO output

zh_cn 				//if the device configuration interface support Chinese

en_us 				//if the device configuration interface support English

zh_tw 				//if the device configuration interface support Traditional Chinese

front_replay 			//support front-end online replay

media_capabiltiy		//query resolution the device support

wireless_accesspoint	//support WIFI as client function

wireless_station		//support WIFI as server function

network_storage		//support network storage

ptz_action			//support PTZ action

ircut_setting			//support IRCUT function

evdo_support			//support EVDO 3G function

wcdma_support			//support WCDMA 3G function

tdscdma_support		//support TDSCDMA 3G function

### Device video file search (picture or video file) (command 1021)

#define CMD_GET_RECORD_FILE_LIST		(CMD_SYSTEM_MANAGE_BASE + 21)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_RECORD_FILE_LIST,0,searchxml)

The format of searchxml as follows:

<REQUEST_PARAM

RecordMode="MOTION"			//

StartTime="20090101 0:00:00"	//search start time

EndTime="20090102 0:00:00"		//search end time, notice that the start time to end time must less than 7 days

MediaType="PICTURE"			//

StreamIndex="1"				//1 means main stream, 2 means sub stream

MinSize="-1"					//search file more than, when it is -1, then has no limit, the unit is bit

MaxSize="-1"					//search file less than, when it is -1, then has no limit, the unit is bit

Page="1"						//search which page, when there are many pages, you can skip to any page by this button, every page show 20 records

/>

The possible value and meaning of Record Mode:

MANUAL: manual trigger record (can not support)

MOTION: motion detect alarm

SCHEDULE: schedual record

ALARM: IO alarm record

ALL:all types

The possible value and meaning of MediaType:

AUDIOVIDEO: audio and video

VIDEO: only video

AUDIO:only audio (can not support)

PICTURE: capture

Return Xml:

<RESPONSE_PARAM>

<RECORD_FILE

FilePath="/mnt/mmc0/motion/2009/04/01/103222-av-1.avi"		//file name

FileLength="14832236"									//file size

/>

<RECORD_FILE

FilePath="/mnt/mmc0/motion/2009/04/01/103107-av-1.avi"

FileLength="14463472"

/>

</RESPONSE_PARAM>

### File upload (configuration file and update file) (command 1022)

#define	 CMD_UPLOAD_FILE				(CMD_SYSTEM_MANAGE_BASE + 22)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_UPLOAD_FILE,0,uploadxml)

The format of uploadxml as follows:

<REQUEST_PARAM

FileType = "/mnt/mmc0/motion/2009/04/01/103222-av-1.avi" 		//0 means configuration files, 1 means update files

FilePath ="local file path" 	//choose local file path, notice the name of local file, if with special characters, then need XML escape, the special character maybe " , & and so on

/>

Notice:

Suggest that do not use this command directly call AJ_NETSDK_IPC_Upgrade, AJ_NETSDK_IPC_SetConfigFile and other corresponding functions.

### File download (command 1023)

#define CMD_DOWNLOAD_FILE			(CMD_SYSTEM_MANAGE_BASE + 23)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_DOWNLOAD_FILE,0,downloadxml)

The format of downloadxml as follows:

<REQUEST_PARAM

FileName = "/mnt/mmc0/motion/2009/04/01/103222-av-1.avi"		//the path of file that stored in device, return when searching file

StartPos="0"				//start download location, currently only be 0

/>

Notice:

Suggest that do not use this command, directly call the provided download API function, AJ_NETSDK_IPC_GetFileByName and other corresponding functions.

### Delete file (command 1024)

#define CMD_REMOVE_FILE				(CMD_SYSTEM_MANAGE_BASE + 24)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_REMOVE_FILE,0,removexml)

The removexml as follows:

<REQUEST_PARAM

FileName = "/mnt/mmc0/motion/2009/04/01/103222-av-1.avi"		// the path of file that stored in device, return when searching file

/>

### Search log file (command 1025)

#define CMD_GET_LOGFILE_LIST		(CMD_SYSTEM_MANAGE_BASE + 25)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_LOGFILE_LIST,0, "")

The format of return XML as follows:

<RESPONSE_PARAM>

<LOG_FILE

FilePath="/mnt/nand/20090401.log"

/>

<LOG_FILE

FilePath="/mnt/nand/20010101.log"

/>

<LOG_FILE

FilePath="/mnt/nand/20001231.log"

/>

</RESPONSE_PARAM>

### Read device type (command 1030)

#define CMD_GET_IPC_TYPE			(CMD_SYSTEM_MANAGE_BASE + 30)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_IPC_TYPE,0, "")

The format of return XML as follows:

<RESPONSE_PARAM

IPC_Type="100"	//the value maybe 100, 101, 102, 103

/>

### Enforce device generate key frame (only V2.0 firmware has) (command 1032)

#define CMD_SET_KEYFRAME (CMD_SYSTEM_MANAGE_BASE + 32)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_SET_KEYFRAME,0, Createiframxml)

The format of Createiframxml as follows:

<REQUEST_PARAM

Camera="0"	//fixed at 0

Stream="0"	//0 means main stream, 1 means sub stream

/>

Notice:

Suggest that do not call this command directly, you can call AJ_NETSDK_IPC_CreateIFrame instead.

### Read user storage data (only V2.0 firmware has) (command 1034)

#define CMD_GET_USER_DEV_DATA (CMD_SYSTEM_MANAGE_BASE + 34)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_GET_USER_DEV_DATA,0, Createiframxml)

The format of Createiframxml as follows:

<REQUEST_PARAM

Camera="0"

Stream="0"	//0 means main stream, 1 means sub stream

/>

Notice:

You can call AJ_NETSDK_IPC_GetUserData directly to achieve this function

### Set user storage data (only V2.0 firmware has) (command 1033)

#define CMD_SET_USER_DEV_DATA (CMD_SYSTEM_MANAGE_BASE + 33)

Call method:

AJ_NETSDK_IPC_SystemControl(m_nLastCmdId, CMD_SET_USER_DEV_DATA,0, Createiframxml)

The format of Createiframxml as follows:

<REQUEST_PARAM

UserDevData="010200001230123012301230123" //user storage data

/>

Suggest:

Before save data, use sprintf (str,”%02x”, c); to transform those data to a string data first. When read again, transform back.

You can call AJ_NETSDK_IPC_SetUserData directly to achieve this function.

## 9. Related micro definition table

//system config cmd

#define CMD_SYSTEM_CONFIG_BASE		            200

#define CMD_GET_SYSTEM_CONFIG		                (CMD_SYSTEM_CONFIG_BASE + 0)

#define CMD_GET_SYSTEM_PTZ_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 1)

#define CMD_GET_SYSTEM_TIME_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 2)

#define CMD_GET_SYSTEM_USER_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 3)

#define CMD_GET_SYSTEM_LOG_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 4)

#define CMD_GET_SYSTEM_MISC_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 5)

#define CMD_SET_SYSTEM_CONFIG		                (CMD_SYSTEM_CONFIG_BASE + 20)

#define CMD_SET_SYSTEM_PTZ_CONFIG	                (CMD_SYSTEM_CONFIG_BASE + 21)

#define CMD_SET_SYSTEM_TIME_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 22)

#define CMD_SET_SYSTEM_USER_CONFIG      	        (CMD_SYSTEM_CONFIG_BASE + 23)

#define CMD_SET_SYSTEM_LOG_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 24)

#define CMD_SET_SYSTEM_PTZ_COMMON	            (CMD_SYSTEM_CONFIG_BASE + 25)

#define CMD_SET_SYSTEM_PTZ_ADVANCE       	    (CMD_SYSTEM_CONFIG_BASE + 26)

#define CMD_SET_SYSTEM_MISC_CONFIG	            (CMD_SYSTEM_CONFIG_BASE + 27)

#define CMD_SYSTEM_CONFIG_LAST		            (CMD_SYSTEM_CONFIG_BASE + 27)

//network cofig cmd

#define CMD_NETWORK_CONFIG_BASE			        300

#define CMD_GET_NETWORK_CONFIG			        (CMD_NETWORK_CONFIG_BASE + 0)

//#define CMD_GET_NETWORK_COMMON_CONFIG	    (CMD_NETWORK_CONFIG_BASE + 1)

//#define CMD_GET_NETWORK_IPACQUIRE_CONFIG      (CMD_NETWORK_CONFIG_BASE + 2)

#define CMD_GET_NETWORK_LAN_CONFIG   	        (CMD_NETWORK_CONFIG_BASE + 5)

#define CMD_GET_NETWORK_WIFI_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 6)

#define CMD_GET_NETWORK_ADSL_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 10)

#define CMD_GET_NETWORK_DDNS_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 11)

#define CMD_GET_NETWORK_UPNP_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 12)

#define CMD_GET_NETWORK_G3_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 13)

#define CMD_SET_NETWORK_CONFIG			        (CMD_NETWORK_CONFIG_BASE + 20)

//#define CMD_SET_NETWORK_COMMON_CONFIG	    (CMD_NETWORK_CONFIG_BASE + 21)

//#define CMD_SET_NETWORK_IPACQUIRE_CONFIG      (CMD_NETWORK_CONFIG_BASE + 22)

#define CMD_SET_NETWORK_LAN_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 25)

#define CMD_SET_NETWORK_WIFI_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 26)

#define CMD_SET_NETWORK_ADSL_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 30)

#define CMD_SET_NETWORK_DDNS_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 31)

#define CMD_SET_NETWORK_UPNP_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 32)

#define CMD_SET_NETWORK_G3_CONFIG			    (CMD_NETWORK_CONFIG_BASE + 33)

#define CMD_NETWORK_CONFIG_LAST			        (CMD_NETWORK_CONFIG_BASE + 33)

//server config cmd

#define CMD_SERVER_CONFIG_BASE			    400

#define CMD_GET_SERVER_CONFIG			        (CMD_SERVER_CONFIG_BASE + 0)

#define CMD_GET_SERVER_FTP_CONFIG		        (CMD_SERVER_CONFIG_BASE + 1)

#define CMD_GET_SERVER_SMTP_CONFIG		    (CMD_SERVER_CONFIG_BASE + 2)

#define CMD_SET_SERVER_CONFIG			        (CMD_SERVER_CONFIG_BASE + 20)

#define CMD_SET_SERVER_FTP_CONFIG		        (CMD_SERVER_CONFIG_BASE + 21)

#define CMD_SET_SERVER_SMTP_CONFIG		    (CMD_SERVER_CONFIG_BASE + 22)

#define CMD_SERVER_CONFIG_LAST			        (CMD_SERVER_CONFIG_BASE + 22)

//media config cmd

#define CMD_MEDIA_CONFIG_BASE			            500

#define CMD_GET_MEDIA_CONFIG			            (CMD_MEDIA_CONFIG_BASE + 0)

#define CMD_GET_MEDIA_VIDEO_CONFIG		        (CMD_MEDIA_CONFIG_BASE + 1)

#define CMD_GET_MEDIA_AUDIO_CONFIG		        (CMD_MEDIA_CONFIG_BASE + 2)

#define CMD_SET_MEDIA_CONFIG			            (CMD_MEDIA_CONFIG_BASE + 20)

#define CMD_SET_MEDIA_VIDEO_CONFIG		        (CMD_MEDIA_CONFIG_BASE + 21)

#define CMD_SET_MEDIA_AUDIO_CONFIG		        (CMD_MEDIA_CONFIG_BASE + 22)

#define CMD_SET_MEDIA_VIDEO_ENCODE		        (CMD_MEDIA_CONFIG_BASE + 23)

#define CMD_SET_MEDIA_VIDEO_CAPTURE		        (CMD_MEDIA_CONFIG_BASE + 24)

#define CMD_SET_MEDIA_VIDEO_OSD			        (CMD_MEDIA_CONFIG_BASE + 25)

#define CMD_SET_MEDIA_VIDEO_JPEG		            (CMD_MEDIA_CONFIG_BASE + 26)

#define CMD_SET_MEDIA_AUDIO_CAPTURE		        (CMD_MEDIA_CONFIG_BASE + 27)

#define CMD_SET_MEDIA_AUDIO_ENCODE		      (CMD_MEDIA_CONFIG_BASE + 28)

#define CMD_SET_PRIVACY_CONFIG                   (CMD_MEDIA_CONFIG_BASE + 29)

#define CMD_MEDIA_CONFIG_LAST			          (CMD_MEDIA_CONFIG_BASE + 28)

//mediastream config cmd

#define CMD_MEDIASTREAM_CONFIG_BASE			   600

#define CMD_GET_MEDIASTREAM_CONFIG			  (CMD_MEDIASTREAM_CONFIG_BASE + 0)

#define CMD_SET_MEDIASTREAM_CONFIG			  (CMD_MEDIASTREAM_CONFIG_BASE + 20)

#define CMD_MEDIASTREAM_CONFIG_LAST			  (CMD_MEDIASTREAM_CONFIG_BASE + 24)

#define CMD_PLATFORM_CONFIG_BASE			       650

#define CMD_GET_PLATFORM_CONFIG				    (CMD_PLATFORM_CONFIG_BASE + 0)

#define CMD_SET_PLATFORM_CONFIG				    (CMD_PLATFORM_CONFIG_BASE + 20)

#define CMD_PLATFORM_CONFIG_LAST			        (CMD_PLATFORM_CONFIG_BASE + 24)

//record config cmd

#define CMD_RECORD_CONFIG_BASE			      700

#define CMD_GET_RECORD_CONFIG			          (CMD_RECORD_CONFIG_BASE + 0)

#define CMD_SET_RECORD_CONFIG			          (CMD_RECORD_CONFIG_BASE + 20)

#define CMD_RECORD_CONFIG_LAST			      (CMD_RECORD_CONFIG_BASE + 22)

//alarm config cmd

#define CMD_ALARM_CONFIG_BASE				    800

#define CMD_GET_ALARM_CONFIG				    (CMD_ALARM_CONFIG_BASE + 0)

#define CMD_GET_ALARM_INPUT_CONFIG			(CMD_ALARM_CONFIG_BASE + 1)

#define CMD_GET_ALARM_MOTIONDETECT_CONFIG	(CMD_ALARM_CONFIG_BASE + 2)

#define CMD_GET_ALARM_VIDEOCOVER_CONFIG	(CMD_ALARM_CONFIG_BASE + 3)

#define CMD_GET_ALARM_VIDEOLOST_CONFIG		(CMD_ALARM_CONFIG_BASE + 4)

#define CMD_GET_ALARM_SF_CONFIG				(CMD_ALARM_CONFIG_BASE + 5)

#define CMD_SET_ALARM_CONFIG				    (CMD_ALARM_CONFIG_BASE + 20)

#define CMD_SET_ALARM_INPUT_CONFIG			(CMD_ALARM_CONFIG_BASE + 21)

#define CMD_SET_ALARM_MOTIONDETECT_CONFIG	(CMD_ALARM_CONFIG_BASE + 22)

#define CMD_SET_ALARM_VIDEOCOVER_CONFIG	(CMD_ALARM_CONFIG_BASE + 23)

#define CMD_SET_ALARM_VIDEOLOST_CONFIG		(CMD_ALARM_CONFIG_BASE + 24)

#define CMD_SET_ALARM_SF_CONFIG				(CMD_ALARM_CONFIG_BASE + 25)

#define CMD_SET_ALARM_INPUT_CHANNEL_CONFIG	(CMD_ALARM_CONFIG_BASE + 26)

#define CMD_ALARM_CONFIG_LAST				    (CMD_ALARM_CONFIG_BASE + 27)

#define CMD_GET_CONFIG_FIELD_VALUE		  900

// config manager cmd

#define CMD_SYSTEM_MANAGE_BASE             1000

#define CMD_CONFIG_UPDATE		            (CMD_SYSTEM_MANAGE_BASE + 1)

#define CMD_CONFIG_RESTORE		            (CMD_SYSTEM_MANAGE_BASE + 2)

#define CMD_CONFIG_BACKUP		            (CMD_SYSTEM_MANAGE_BASE + 3)

#define CMD_SET_SYSTEM_TIME		            (CMD_SYSTEM_MANAGE_BASE + 5)

#define CMD_SYSTEM_REBOOT		            (CMD_SYSTEM_MANAGE_BASE + 7)

#define CMD_RECORD_REMOVE		            (CMD_SYSTEM_MANAGE_BASE + 8)

#define CMD_APPBIN_LOCAL_UPDATE 	        (CMD_SYSTEM_MANAGE_BASE + 9)

#define CMD_APPBIN_FTP_UPDATE  		    (CMD_SYSTEM_MANAGE_BASE + 10)

#define CMD_GET_SYSTEM_VERSION_INFO      (CMD_SYSTEM_MANAGE_BASE + 12)

#define CMD_GET_LOG_CONFIG      	        (CMD_SYSTEM_MANAGE_BASE + 13)

#define CMD_GET_STORAGE_INFO 		        (CMD_SYSTEM_MANAGE_BASE + 14)

#define CMD_RECORD_START			        (CMD_SYSTEM_MANAGE_BASE + 15)

#define CMD_RECORD_STOP				        (CMD_SYSTEM_MANAGE_BASE + 16)

#define CMD_STORAGE_DEVICE_FORMAT	    (CMD_SYSTEM_MANAGE_BASE + 17)

#define CMD_STORAGE_DEVICE_UNMOUNT      (CMD_SYSTEM_MANAGE_BASE + 18)

#define CMD_GET_SERIALNUMBER			    (CMD_SYSTEM_MANAGE_BASE + 19)

#define CMD_GET_SYSTEMCONTROLSTRING		(CMD_SYSTEM_MANAGE_BASE + 20)

#define CMD_GET_RECORD_FILE_LIST		    (CMD_SYSTEM_MANAGE_BASE + 21)

#define CMD_UPLOAD_FILE					    (CMD_SYSTEM_MANAGE_BASE + 22)

#define CMD_DOWNLOAD_FILE				    (CMD_SYSTEM_MANAGE_BASE + 23)

#define CMD_REMOVE_FILE					    (CMD_SYSTEM_MANAGE_BASE + 24)

#define CMD_GET_LOGFILE_LIST			        (CMD_SYSTEM_MANAGE_BASE + 25)

#define CMD_GET_NETWORK_STATUS			(CMD_SYSTEM_MANAGE_BASE + 26)

#define CMD_CONFIG_NETWORK				    (CMD_SYSTEM_MANAGE_BASE + 27)

#define CMD_SET_REPLAY_FILENAME			(CMD_SYSTEM_MANAGE_BASE + 28)

#define CMD_GET_REPLAY_FILENAME			(CMD_SYSTEM_MANAGE_BASE + 29)

#define CMD_GET_IPC_TYPE				        (CMD_SYSTEM_MANAGE_BASE + 30)

#define CMD_GET_MEDIA_CAPABILITY		    (CMD_SYSTEM_MANAGE_BASE + 31)

#define CMD_SET_KEYFRAME				    (CMD_SYSTEM_MANAGE_BASE + 32)

#define CMD_SET_USER_DEV_DATA			    (CMD_SYSTEM_MANAGE_BASE + 33)

#define CMD_GET_USER_DEV_DATA			    (CMD_SYSTEM_MANAGE_BASE + 34)

#define CMD_OPEN_RS232_PORT                (CMD_SYSTEM_MANAGE_BASE + 35)

#define CMD_CLOSE_RS232_PORT               (CMD_SYSTEM_MANAGE_BASE + 36)

#define CMD_START_RTP_PUSH				    (CMD_SYSTEM_MANAGE_BASE + 40)

#define CMD_STOP_RTP_PUSH				    (CMD_SYSTEM_MANAGE_BASE + 41)

#define CMD_ALARM_OUTPUT_CONTROL		    (CMD_SYSTEM_MANAGE_BASE + 42)

#define CMD_SNAP_JPEG_PICTURE			    (CMD_SYSTEM_MANAGE_BASE + 43)

#define CMD_NETWORK_DIAL				    (CMD_SYSTEM_MANAGE_BASE + 45)

#define CMD_IRCUT_CONTROL				    (CMD_SYSTEM_MANAGE_BASE + 46)

#define CMD_SET_AUX_DATA				    (CMD_SYSTEM_MANAGE_BASE + 47)